Radio Interface Specification (RIS) implementation of ECC/DEC/(12)03 on the harmonised conditions for UWB applications on-board aircraft

Foreword

The ECC has decided that RIS implementations of ECC deliverables should be uploaded to the ECO website in order to help administrations fill out the EFIS database.

This RIS implementation is limited to the harmonised conditions for UWB applications on-board aircraft.

RIG II Template for the harmonised conditions for UWB applications on-board aircraft

Radio Interface Notification by an administration

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Not applicable | ITU‐R RR Article number 4.4 |
|  | 2 | Application | UWB applications | For the use of devices using UWB technology on-board aircraft |
|  | 3 | Frequency bands | 6 GHz to 8.5 GHz | See ECC/DEC/(12)03 Decides 1 and Table 1 in the Annex of ECC/DEC/(12)03 |
|  | 4 | Channelling | Not specified |  |
|  | 5 | Modulation / Occupied bandwidth | Not specified | Technology neutral approach although it is recognised that equipment uses UWB (Ultra‐WideBand) technologies |
|  | 6 | Direction / Separation | Not specified |  |
|  | 7 | Transmit power / Power density | 6.0-6.650 GHz: max. mean e.i.r.p.: -41.3 dBm/MHz, max. peak e.i.r.p. within 50 MHz: 0 dBm 6.650 -6.6752 GHz: max. mean e.i.r.p.: -62.3 dBm/MHz, max. peak e.i.r.p. within 50 MHz: -21 dBm 6.6752-8.5 GHz: max. mean e.i.r.p.: -41.3 dBm/MHz, max. peak e.i.r.p. within 50 MHz: 0 dBm | 6.650 -6.6752 GHz: a notch of 21 dB should be implemented to meet a level -62.3 dBm/MHz 7.25-7.75 GHz (Fixed Satellite Service) and 7.45-7.55 GHz (Meteorological Satellite) protection: -51.3 - 20 * log10(10 [km] / x [km]) (dBm/MHz) for heights above ground above 1 000 m, where x is the aircraft height above ground in kilometres, -71.3 dBm/MHz for heights above ground of 1 000 m and below. 7.75-7.9 GHz (Meteorological Satellite) protection: -44.3 - 20* log10(10 [km] / x [km]) (dBm/MHz) for heights above ground above 1000 m, where x is the aircraft height above ground in kilometres, and -64.3 dBm/MHz for heights above ground of 1000 m and below. Alternative mitigation techniques offering equivalent protection such as the use of shielded portholes could be a solution. |
|  | 8 | Channel access and occupation rules | Not specified |  |
|  | 9 | Authorisation regime | General authorisation | No registration and/or notification is required (commonly known as “licence exempt”). |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions | Use on-board of an aircraft | The assumption used in ECC Report 175 is that not more than two UWB devices would be transmitting at any time within an aircraft within the same frequency range. |
|  | 12 | Planned changes | --- |  |
|  | 13 | Reference | ECC Decision (12)03 ECC Report 175 |  |
|  | 14 | Notification number | --- |  |
|  | 15 | Remarks | Definitions |  |
|  |  |  | Abbreviations e.i.r.p. equivalent isotropically radiated power UWB Ultra-WideBand |  |