RIS implementation of ECC DEC(11)03 on Citizens’ Band (CB) Radio

Foreword

The ECC has decided that RIS implementations of ECC deliverables should be uploaded to the ECO website in order to help administrations fill out the EFIS database.

This RIS implementation is limited to Citizens’ Band (CB) Radio.

RIG II Template for Citizens’ Band (CB) Radio Interface Notification by an administration

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| 120 Normative part | 1 | Radiocommunication Service | Not applicable | ITU-R RR Article 4.4 |
|  | 2 | Application | CB Radio | CB Radio is defined as a voice service but some administrations allow it to be used also for data transmissions. |
|  | 3 | Frequency bands | 26.960-27.410 MHz excluding 26.995 MHz, 27.045 MHz, 27.095 MHz, 27.145 MHz and 27.195 MHz | Unpaired bands |
|  | 4 | Channelling | 10 kHz channel separation |  |
|  | 5 | Modulation / Occupied bandwidth | Angle-modulated, Double Side Band (DSB) modulated, or Single Side Band (SSB) modulated equipment |  |
|  | 6 | Direction / Separation | Not specified |  |
|  | 7 | Transmit power / Power density | The maximum radiated power for Citizens’ Band radio equipment is limited to 4 Watts for angle-modulated, 4 Watts (RMS) for DSB modulated, and 12 Watts (PEP) for SSB modulated Citizens’ Band radio equipment |  |
|  | 8 | Channel access and occupation rules | Not required |  |
|  | 9 | Authorisation regime | General authorisation | CB Radios are not Electronic Communications Networks or Services according to the Directive 2002/20/EC (see “Whereas 5”) as amended by the Directive 2009/140/EC (in particular see Article 1(2) ). This legislation doesn't therefore apply. No registration and/or notification is required (“licence exempt”). |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions | No additional parameters |  |
| Informative part | 12 | Planned changes | --- |  |
|  | 13 | Reference | ECC Decision (11)03 ETSI EN 300 433-2 | Only the version(s) mentioned in the most recent publication of the list of Harmonised European Standards give(s) presumption of conformity. It is common to foresee a transition period for the earlier versions. |
|  | 14 | Notification number | --- |  |
|  | 15 | Remarks | Definitions |  |
|  |  |  | Abbreviations CB Citizens’ Band FM Frequency Modulation AM Amplitude Modulation SSB Single Side Band DSB Double Side Band RMS Root Mean Square PEP Peak Envelope Power ISM Industrial, Scientific, and Medical |  |