European Radiocommunications Committee (ERC)

within the European Conference of Postal and Telecommunications Administrations (CEPT)

FIXED SERVICE SYSTEM PARAMETERS FOR FREQUENCY SHARING

Rome, October 1996

FIXED SERVICE SYSTEM PARAMETERS FOR FREQUENCY SHARING

- Introduction

The following tables provide information on fixed service system parameters for use in initial sharing studies.    The tables were compiled from information supplied by members of CEPT Project Team SE19.

The table format follows that of ITU-R Recommendation F 758, with systems being identified by modulation type and system capacity, and systems grouped together by frequency band.    The introduction to Recommendation F 758 contains information on the use of such tables, together with basic considerations in the development of sharing criteria.    It gives advice on the basic transmitter and receiver characteristics required for interference evaluation and calculations for frequency sharing with other services.

Left blank

Table 1

Fixed service system parameters for frequency sharing

1 - 3 GHz point to multipoint systems

(a)    Typical characteristics

| PARAMETER | CENTRAL STATION | OUTSTATION |
| --- | --- | --- |
| Antenna type | Omni/Sector | Dish/Horn |
| Antenna gain (dBi) | 10/17 | 20 Analogue 27 Digital |
| e.i.r.p. (max.) (dBW) - Analogue - Digital | 12 24 | 21 34 |
| IF bandwidth (MHz) | 2 | 2 |
| Maximum permissible long term interference power (20% time) Total (dBW) dB(W/4 kHz) dB(W/MHz) | -142 -170 -147 | -142 -170 -147 |

Table 2

Characteristics of example CDMA local access radio systems

| Frequency band | 2 GHz |  | 4 GHz |  |
| --- | --- | --- | --- | --- |
| System type | Fixed point-to-multipoint (CDMA) |  | Fixed point-to-multipoint (DS CDMA) |  |
| RF transmission rate | 2 Mbit/s |  | 2 Mbit/s |  |
| Modulation | QPSK |  | QPSK |  |
|  | Central Station | Outstation | Central Station | Outstation |
| Transmit power (dBW) | -10.0 (per outstation) | -10.0 | 2 | 2 |
| Antenna polarisation | Vertical | Vertical | Vertical | Vertical |
| Antenna maximum gain | 10 | 9 | 16 | 16 |
| Feeder loss | 3.5 | 0 | 8 | 18 |
| Maximum e.i.r.p. | -3.5/outstation(1) | -1.0(2) | 10 | 0 |
| Receiver IF bandwidth | 3.2 | 3.2 | 21 | 21 |
| Receiver thermal noise | -134.0 | -134.0 | -117 | -117 |
| Receiver threshold (dBW) (BER 10-6) | -135.0(3),(4) | -135.0(3) ,(4) | -118 | -118 |
| Maximum long term (dB(W/MHz)) interference power | -150.0 | -150.0 | -140(5) | -140(5) |
| Availability target | 99.99% of time | 99.99% of time | 99.99% of time | 99.99% of time |
| Typical fade margin | < 20 dB | < 20 dB | 30 | 30 |
| Path length | 1 - 15 km | 1 - 15 km | 3 km | 3 km |

(1)	Maximum e.i.r.p.: 8.5 dBW

(2)	Automatic power control is employed, therefore typical powers may be 0 - 20 dB less

(3)	Typical signal level for a system with 15 outstations

(4)	Threshold quoted for BER 10-7 for these systems

(5)	Measured at antenna port

Table 3

Fixed service system parameters for frequency sharing

Fixed services below 3 GHz

| Frequency band (GHz) | 1.35 - 1.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.67 - 1.69 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FDM-FM | PSK- 4-PSK | FDM-FM | FDM-FM | 4-PSK | 4-PSK | 16-QAM | 4-PSK | PCM | MSK | 4-PSK | 4-PSK | 4-PSK | 4-PSK | 4-PSK |  | FDM-FM |
| Capacity | 36 ch | 704 kbit/s 2 Mbit/s | 8 ch | 2 ch | 9.6 kbit/s | 64 kbit/s | 64 kbit/s | 144 kbit/s | - | 2 Mbit/s | 2 Mbit/s | 2×2 Mbit/s | 2×2 Mbit/s | 34 Mbit/s |  |  | 2 ch |
| Channel spacing (MHz) | 0.5 | 1 | 0.2 | 0.05 | 0.025 | 0.075 | 0.0375 | 0.225 | 0.5 | 2 | 2 | 2 | 4 | 29 | 3.5 |  | 0.05 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CS | OS |  |
| Antenna gain (max.) (dBi) | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 17 | 27 | 33 |
| Feeder/multiplexer loss (min) (dB) | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| Antenna type | dish | dish | Yagi | Yagi | Yagi/dish | Yagi/dish | Yagi/dish | Yagi/dish | Yagi/dish | dish | Yagi/horn /dish | Yagi | Yagi | dish | omni/ section | dish/ horn | Yagi |
| Max. Tx output power (dBW) | 7 | 7 | 7 | 10 | 7 | 7 | 7 | 7 | 10 | 7 | 0 | 7 | 7 | 3 | 7 | 7 | 10 |
| e.i.r.p. (max.) (dBW) | 39 | 39 | 39 | 42 | 39 | 39 | 39 | 39 | 42 | 39 | 32 | 39 | 39 | 35 | 24 | 34 | 42 |
| Receiver IF bandwidth (MHz) | 0.4 | 0.7 | 0.3 | 0.04 | 0.005 | 0.032 | 0.016 | 0.072 | 0.35 | 1.2 | 1.5 | 1.2 | 3 | 20 | 3.5 | 3.5 | 0.04 |
| Receiver noise figure (dB) | 8 | 4.5 | 8 | 8 | 4 | 4 | 4 | 4 | 8 | 4 | 7 | 4 | 4 | 4 | 3.5 | 3.5 | 8 |
| Receiver thermal noise (dBW) | -140 | -141 | -141 | -149 | -163 | -155 | -158 | -151 | -141 | -139 | -135 | -139 | -145 | -127 | -135 | -135 | -149 |
| Nominal Rx input level (dBW) | -80 | -90 | -93 | -105 | -118 | -112 | -103.5 | -106 | -90 | -126 | -79 | -124 | -123 | -73 |  |  | -105 |
| Rx input level for 10-3 BER (dBW) | N/A | -120 | N/A | N/A | -143 | -137 | -128.5 | -131 | - | -86 | -119 | -84 | -83 | -113 |  |  | N/A |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference | -150 | -151 | -151 | -158 | -173 | -167 | -168 | -161 | -151 | -145 | -145 | -145 | -141 | -137 | -145 | -145 | -159 |
| Equivalent power (dB(W/4 kHz)) | -170 | - | -170 | -169 | - | - | - | - | - |  |  |  |  | - | -174 | -174 |  |
| Spectral density (dB(W/4 kHz)) | - | -149 | - | - | -150 | -152 | -150 | -150 | -146 | -146 |  | -146 | -146 | -150 |  |  | -169 |
| Refer to notes | (2),(5) | (2),(4) | (2),(5) | (2),(5) | (2),(4) | (2),(4) | (2),(4) | (2),(4) | (2),(4) |  | (2),(4) | (1),(4) | (1),(4) | (2),(4) |  |  | (2),(5) |

N/A:	Not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10% of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density.    The interference spectral density, averaged over any 4 kHz within the receiver bandwidth must not exceed this value

Table 4

Fixed service system parameters for frequency sharing

Fixed services below 3 GHz

| Frequency band (GHz) | 1.7- 2.45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FM-TVOB | FM-FDM | FM-MLT-PT |  | FM-TV | FM-FDM | FM-FDM tropo | 4-PSK tropo | 4-PSK | 4-PSK | FM-FDM | 4-PSK | 4-PSK | FM-FDM | 4-PSK |
| Capacity | 625-line PAL | 60-132 ch | 94 ch |  | 625-line PAL | 960 ch | 72-312 ch | 2 Mbit/s | 34 Mbit/s | 8 Mbit/s | 1-6 ch | 48 ch | 12.6 Mbit/s | 600 ch | 2 × 8 Mbit/s |
| Channel spacing (MHz) | Variable | 14/1 | 3.5 |  | 29 | 29 | Special | Special | 29 | 14 | 0.4 | 2.5 | 28 | 28 | 14 |
|  |  |  | CS | OS |  |  |  |  |  |  |  |  |  |  |  |
| Antenna gain (max.) (dBi) | 25 | 35.7 | 10 | 19 | 35.7 | 35.7 | 49 | 45 | 35.7 | 35.7 | 25 | 35.7 | 35.7 | 35.7 | 35.7 |
| Feeder/multiplexer loss (min) (dB) | 0 | 1 | 3 | 3 | 1 | 1 | 2 | 2 | 1 | 1 | 3 | 1 | 1 | 1 | 1 |
| Antenna type | 1.2m dish | dish | omni | horn | dish | dish | 12m dish | 9m dish | dish | dish | Yagi | dish | dish | dish | dish |
| Max. Tx output power (dBW) | 7 | 7 | 4 | 4 | 10 | 7 | 28 | 30 | 3 | 0 | 10 | -9 | -10 | -5.2 | 3 |
| e.i.r.p. (max.) (dBW) | 32 | 41.7 | 13 | 22 | 44.7 | 41.7 | 75 | 73 | 37.7 | 34.7 | 32 | 25.7 | 24.7 | 29.5 | 37.7 |
| Receiver IF bandwidth (MHz) | 30 | 2.8 | 2 | 2 | 40 | 40 | 6 | 1 | 20 | 4 | 0.15 | 1.5 | 6.5 | 20 | 8 |
| Receiver noise figure (dB) | 8 | 7 | 9 | 9 | 10 | 10 | 2 | 4 | 4 | 5 | 4 | 6 | 9 | 10 | 4 |
| Receiver thermal noise (dBW) | -121 | -133 | -132 | -132 | -118 | -118 | -132 | -140 | -127 | -133 | -148 | - | - | - | -131 |
| Nominal Rx input level (dBW) | -65 | -79 | -97 | -97 | -68 | -64 | - | - | -73 | -78 | -78 | -78 | -88.3 | -78 |  |
| Rx input level for 10-3 BER (dBW) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | -131 | -113 | -118 | N/A | - | - | N/A |  |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference | -127 | -143 | -142 | -142 | -128 | -129 | -138 | -146 | -137 | -143 | -158 |  |  |  | -137 |
| Equivalent power (dB(W/4 kHz)) | - | -172 | -169 | -169 | - | -169 | -172 | - | - | - | -174 |  |  |  | -170 |
| Spectral density (dB(W/4 kHz)) | -142 | - | - | - | -144 | - | - | -146 | -150 | -149 | - | - |  |  | -146 |
| Refer to notes | (1),(4) | (2),(5) | (2),(5) | (2),(5) | (2),(4) | (2),(5) | (1),(5) | (1),(4) | (2),(4) | (2),(4) | (2),(5) | (3),(4) | (3),(4) | (3),(5) | (1),(4) |

N/A:	Not applicable

TVOB:	Television Outside Broadcast

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10% of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density.    The interference spectral density, averaged over any 4 kHz within the receiver bandwidth must not exceed this value

Table 5

Fixed service system parameters for frequency sharing

Fixed services below 3 GHz

| Frequency band (GHz) | 1.7 - 2.45 |  |  |  |  |  |  | 2.45 - 2.69 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FM-QAM | 4-PSK | 4-PSK | 4-PSK |  | 4-OQPSK | 4-OQPSK | FM-FDM tropo | MSK | 4-PSK | FM-TVOB |
| Capacity | 1 TV + 2 Mbit/s | 8 Mbit/s | 2 × 8 Mbit/s |  |  | 8 Mbit/s | 34 Mbit/s | 72-312 ch | 2 × 2 Mbit/s | 34 Mbit/s | 625-line PAL |
| Channel spacing (MHz) | 3.5 | 7 | 14 | 3.5 |  | 14 | 14 | Special | 14 |  | Variable |
|  |  |  |  | CS | OS |  |  |  |  |  |  |
| Antenna gain (max.) (dBi) | 35.7 | 35.7 | 35.7 | 17 | 27 | 35.7 | 35.7 | 49 | 35.4 | 35.4 | 18 |
| Feeder/multiplexer loss (min) (dB) | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 2 | 4 | 4 | 1 |
| Antenna type | dish | dish | dish | omni/section | dish/horn | dish | dish | 12m dish | dish | dish | dish |
| Max. Tx output power (dBW) | 5 | 7 | -3 | 7 | 7 | 0 | 0 | 28 | 5 | -2 | 7 |
| e.i.r.p. (max.) (dBW) | 39.7 | 41.7 | 31.7 | 24 | 34 | 34.7 | 34.7 | 75 | 36.4 | 29.4 | 24 |
| Receiver IF bandwidth (MHz) | 3 | 3 | 4.6 | 3.5 | 3.5 | 5.4 | 21 | 6 | 3 |  | 30 |
| Receiver noise figure (dB) | 4 | 4 | 4 | 3.5 | 3.5 | 6 | 7 | 2 | 4 |  | 6 |
| Receiver thermal noise (dBW) | -135 | -135 | -133 | -135 | -135 | -131 | -124 | -132 | -135 |  | -123 |
| Nominal Rx input level (dBW) |  | -83 | -83 | - | - | -78 | -72 | -65 | - |  | -55 |
| Rx input level for 10-3 BER (dBW) |  | -123 | -123 | - | - | -118 | -112 | N/A | - |  | N/A |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference | -141 | -141 | -139 | -141 | -141 | -141 | -130 | -138 | -141 | -111.5 | -123 |
| Equivalent power (dB(W/4 kHz)) | -170 | - | - | -170 | -170 | -142 | -137 | -172 | -170 |  | - |
| Spectral density (dB(W/4 kHz)) | - | -146 | -146 | - | - |  |  | - | -162 |  | -129 |
| Refer to notes |  | (1) | (1) | (1) | (1) | (2),(4) | (1),(4) | (1),(5) |  |  |  |

N/A:	Not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10% of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density.    The interference spectral density, averaged over any 4 kHz within the receiver bandwidth must not exceed this value

Table 6

Fixed service system parameters for frequency sharing

Fixed services between 3 and 10 GHz

| Frequency band (GHz) | 3.4 - 3.456 | 3.4 - 3.6 | 3.6 - 4.2 |  |  |  |  |  | 3.7 - 4.2 |  | 3.8 - 4.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FM | AM | 4-PSK | 4-PSK | 4-PSK | 16-QAM | RBQPSK | 64 QAM | FM/FDM | QPSK | 64-QAM | 8-PSK | FM |
| Capacity | 1 ch | TV | 8 Mbit/s | 2x8 Mbit/s | 34 Mbit/s | 51 Mbit/s | 140 Mbit/s | 140/155 Mbit/s | 1800 ch | 34 Mbit/s | 140 Mbit/s | 2×34 Mbit/s | video |
| Channel spacing (MHz) | 1 | 6 | 7 | 14 | 28 | 28 | 90 | 28 | 29 | 29 | 29 | 29 | 29 |
| Antenna gain (max.) (dBi) | 40 | 40 | 41 | 41 | 41 | 41 | 41 | 41 | 41 | 41 | 41 | 41 | 41 |
| Feeder/multiplexer loss (min) (dB) | 4 | 4 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Antenna type | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish |
| Max. Tx output power (dBW) | 19 | 17.6 | 10 | 10 | 10 | 10 | 6 | 10 | 13 | 0 | 2 | 0 | 10 |
| e.i.r.p. (max.) (dBW) | 55 | 53.6 | 48 | 48 | 48 | 48 | 44 | 48 | 51 | 38 | 39 | 37 | 47 |
| Receiver IF bandwidth (MHz) | 0.4 | 5.7 | 5.6 | 8.9 | 17.8 | 12.6 | 56 | 22.4 | 46 | 26 | 29.8 | 28.3 | 40 |
| Receiver noise figure (dB) | 5 | 6 | 6 | 6 | 6 | 5 | 6 | 5 | 8.5 | 4 | 3 | 6 | 9 |
| Receiver thermal noise (dBW) | -143 | -132 | -136.5 | -134.5 | -131.5 | -133 | -122 | -130.5 | -119 | -128 | -126 | -123 | -119 |
| Nominal Rx input level (dBW) | -75 | -70 | -111+M | -109+M | -106+M | -104+M | -65 | -96+M | -47 | -68 | -62 | -66 | -59 |
| Rx input level for 10-3 BER (dBW) |  |  | -114 | -112 | -109 | -107 | -105 | -99 | N/A | -114.5 | -102 | -106 | N/A |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference |  |  |  |  |  |  | -132 |  | -129 | -138 | -132 | -133 | -125 |
| Equivalent power (dB(W/4 kHz)) |  |  |  |  |  |  | - |  | -170 | - | -171 | -172 | -165 |
| Spectral density (dB(W/4 kHz)) |  |  |  |  |  |  | -149 |  | - | -152 |  |  |  |
| Refer to notes |  |  | (8) | (8) | (8) | (8) | (2),(4) | (8),(9),(10) | (2),(5) | (2),(4) | (1),(4) | (2),(4) | (1),(4) |

N/A:	Not applicable
(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10% of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density.    The interference spectral density, averaged over any 4 kHz within the receiver bandwidth must not exceed this value

(6)	I/N = -13 dB: objective for FS systems employing space diversity

(7)	-7 dBW transmit power and 30 dBW e.i.r.p. without APC

(8)	M is fade margin, dependent on path length, polarisation and availability required

(9)	Channel spacing assumes adjacent 140/155 Mbit/s systems using 28 MHz cross-polar channels

(10)	140/155 Mbit/s calculation based on 64 QAM , higher order schemes may be used as appropriate

Table 7

Fixed service system parameters for frequency sharing

Fixed services between 3 and 10 GHz

| Frequency band (GHz) | 5.85 - 5.925 | 5.85 - 6.425 | 5.925 - 6.425 |  |  | 6.425 - 7.11 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FM | RBQPSK | FM/FDM | FM | 64-QAM | FM | FM | QPSK | 16-QAM | 64-QAM |
| Capacity | TV | 140 Mbit/s | 1800 ch | video | 140 Mbit/s | TV | TV | 34 Mbit/s | 140 Mbit/s | 140 Mbit/s |
| Channel spacing (MHz) | 18 | 90 | 29.65 | 29.65 | 29.65 | 20 | 40 | 20 | 40 | 40 |
| Antenna gain (max.) (dBi) | 40 | 45 | 45 | 45 | 45 | 46 | 46 | 46 | 46 | 46 |
| Feeder/multiplexer loss (min) (dB) | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Antenna type | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish |
| Max. Tx output power (dBW) | 17.6 | 6 | 13 | 12 | 2 | 10 | 13 | 0 | 0 | 0 |
| e.i.r.p. (max.) (dBW) | 53.6 | 47 | 54 | 53 | 43 | 52 | 55 | 42 | 42 | 42 |
| Receiver IF bandwidth (MHz) | 17 | 56 | 40 | 40 | 29 | 28 | 40 | 26 | 44 | 42 |
| Receiver noise figure (dB) | 6 | 6 | 8 | 11 | 4 | 8.5 | 11 | 4 | 4 | 3 |
| Receiver thermal noise (dBW) | -130.4 | -122 | -121 | -117 | -127 | -122 | -117 | -128 | -126 | -125 |
| Nominal Rx input level (dBW) | -70 | -65 | -60 | -57 | -63 | -60.5 | -57 | -68 | -65 | -65 |
| Rx input level for 10-3 BER (dBW) |  | -105 | N/A | N/A | -103 | N/A | N/A | -114.5 | -105 | -105 |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference |  | -132 | -131 | -123 | -137 | -132 | -127 | -138 | -136 | -131 |
| Equivalent power (dB(W/4 kHz)) |  | - | -171 | -163 | - | -170 | -167 | - | - | -171 |
| Spectral density (dB(W/4 kHz)) |  | -149 | - |  | -152 | - |  | -152 | -152 |  |
| Refer to notes |  | (2),(4) | (2),(5) | (1),(4) | (2),(4) | (2),(5) | (2),(4) | (2),(4) | (2),(4) | (1),(4) |

N/A:	Not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10% of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density.    The interference spectral density, averaged over any 4 kHz within the receiver bandwidth must not exceed this value

(6)	I/N = -13 dB: objective for FS systems employing space diversity

Table 8

Fixed service system parameters for frequency sharing

Fixed services between 3 and 10 GHz

| Frequency band (GHz) | 7.125 - 7.750 |  |  | 7.425 - 7.900 |  |  |  | 8 - 8.5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FM | 4-OQPSK | 4-OQPSK | 4-PSK | 4-PSK | 4-PSK | 64-QAM | 4-PSK | 8-PSK |
| Capacity | 960 ch | 8 Mbit/s | 34 Mbit/s | 8 Mbit/s | 2x8 Mbit/s | 34 Mbit/s | 140/155 Mbit/s | 34 Mbit/s | 2×34 Mbit/s |
| Channel spacing (MHz) | 20 | 7 | 28 | 7 | 14 | 28 | 28 | 32.5 | 32.5 |
| Antenna gain (max.) (dBi) | 46 | 46 | 46 | 46.4 | 46.4 | 46.4 | 46.4 | 45.5 | 45.5 |
| Feeder/multiplexer loss (min) (dB) | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Antenna type | dish | dish | dish | dish | dish | dish | dish | dish | dish |
| Max. Tx output power (dBW) | 10 | -3 | -3 | 10 | 10 | 10 | 10 | -4.5 | -4.5 |
| e.i.r.p. (max.) (dBW) | 54 | 39 | 39 | 52.4 | 52.4 | 52.4 | 52.4 | 37 | 37 |
| Receiver IF bandwidth (MHz) | 27 | 5.4 | 21 | 5.6 | 8.9 | 17.8 | 22.4 | 20.4 | 28.3 |
| Receiver noise figure (dB) | 7 | 5 | 5 | 6 | 6 | 6 | 6 | 8 | 6 |
| Receiver thermal noise (dBW) | -122.7 | -132 | -126 | -136.5 | -134.5 | -131.5 | -130.5 | -123 | -123 |
| Nominal Rx input level (dBW) | -70 | -79 | -73 | -111+M | -109+M | -106+M | -95+M | -66 | -63 |
| Rx input level for 10-3 BER (dBW) |  | -119 | -113 | -114 | -112 | -109 | -98 | -106 | -103 |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference |  | -142 | -136 |  |  |  |  | -133 | -133 |
| Equivalent power (dB(W/4 kHz)) |  | -173 | -173 |  |  |  |  | -170 | -172 |
| Spectral density (dB(W/4 kHz)) |  |  |  |  |  |  |  |  |  |
| Refer to notes |  | (2),(4) | (2),(4) | (7) | (7) | (7) | (7),(8),(9) | (2),(4) | (2),(4) |

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10% of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density.    The interference spectral density, averaged over any 4 kHz within the receiver bandwidth must not exceed this value

(6)	I/N = -13 dB: objective for FS systems employing space diversity

(7)	M is fade margin, dependent on path length, polarisation and availability required

(8)	Channel spacing assumes adjacent 140/155 Mbit/s systems using 28 MHz cross-polar channels

(9)	140/155 Mbit/s calculation based on 64 QAM , higher order schemes may be used as appropriate

Table 9

Fixed service system parameters for frequency sharing

Fixed services above 10 GHz

| Frequency band (GHz) | 10.2 - 10.68 |  | 10.5 - 10.68 |  |  | 10.7 - 11.7 |  |  |  |  |  |  |  | 12.2 - 12.44 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | 9QPRS | 9QPRS | 4-PSK | FM | FM | 64 QAM | 4-PSK | FM-FDM | FM-TV | 16-QAM | 8-PSK | 16-QAM | 16-QAM | 4-PSK | 16-QAM |
| Capacity | 2 Mbit/s | 8 Mbit/s | 34 Mbit/s | video | video | 155 Mbit/s | 140 Mbit/s | 960 ch | 625-line PAL | 140 Mbit/s | 140 Mbit/s | 140 Mbit/s | 140 Mbit/s | 13.9 Mbit/s | 50.4 Mbit/s |
| Channel spacing (MHz) | 1.25 | 5 | 14 | 14 | 28 | 40 | 67 | 40 | 40 | 40 | 60 | 60 | 40 | 20 | 20 |
| Antenna gain (max.) (dBi) | 45 | 45 | 45 | 45 | 32.2 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| Feeder/multiplexer loss (min) (dB) | 0 | 0 | 4 | 4 | 4 | 2.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 1 | 1 |
| Antenna type | dish | dish | dish | dish |  | shell (offset) | dish | dish | dish | dish | dish | dish |  | dish | dish |
| Max. Tx output power (dBW) | 0 | 0 | -7 | -7 | -20 | 1 | 10 | 10 | 10 | 7 | 9 | 9 | -4 | -5 | -5 |
| e.i.r.p. (max.) (dBW) | 45 | 45 | 34 | 34 | 8.2 | 48.5 | 56.5 | 56.5 | 56.5 | 53.5 | 55.5 | 55.5 | 42.5 | 44 | 44 |
| Receiver IF bandwidth (MHz) |  |  | 20.4 | 28 | 28 |  | 68 | 29 | 29 |  | 56.4 | 42 |  | 12.3 | 17.2 |
| Receiver noise figure (dB) | 5 | 5 | 8 | 8 | 3 | 2.5 | 7 | 7 | 8 |  | 7 | 3 |  | 7 | 5 |
| Receiver thermal noise (dBW) |  |  | -123 | -122 | -111 |  | -119 | -121 | -121 |  | -119 | -125 | -118 | - |  |
| Nominal Rx input level (dBW) | -65 | -65 | -68 | -62 |  | -80 | -62 | -65 | -65 |  | -63 | -65 |  | -59+M | -59+M |
| Rx input level for 10-3 BER (dBW) | -120 | -114 | -108 | N/A |  | -106 | -104 | N/A | N/A | -103.3 | -103 | -105 | -104 |  |  |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference | -148 | -142 | -129 | -128 |  | -140 | -129 | -131 | -131 |  | -129 | -135 |  |  |  |
| Equivalent power (dB(W/4 kHz)) |  |  | -166 | -166 |  |  | - | -170 | - |  | -171 | -175 |  |  |  |
| Spectral density (dB(W/4 kHz)) |  |  |  |  |  |  | -147 | - | -146 |  |  |  |  |  |  |
| Refer to notes |  |  | (1),(4) | (1),(4) |  |  | (2),(4) | (2),(5) | (2),(4) |  | (2),(4) | (2),(4) |  | (3),(7) | (3),(7) |

N/A: not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10 % of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value

(6)	I/N = -13 dB: objective for FS systems employing space diversity

(7)	M is fade margin, dependent on path length, polarisation and availability required

Table 10

Fixed service system parameters for frequency sharing

Fixed services above 10 GHz

| Frequency band (GHz) | 13/14 |  |  |  |  |  |  |  |  |  |  | 14.25 - 14.5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | 4-PSK | 4-PSK | 4-PSK | 4-PSK | FM | QPSK | QPSK | 4-CPOQPSK | 4-CPOQPSK | FSK | 4-PSK | 4-PSK | 16-QAM | 8-PSK |
| Capacity | 2 Mbit/s | 8 Mbit/s | 16 Mbit/s | 34 Mbit/s | 1 Video | 34 Mbit/s | 34 Mbit/s | 4 Mbit/s | 8 Mbit/s | 52 Mbit/s | 2×34 Mbit/s | 8 Mbit/s | 34 Mbit/s | 34 Mbit/s |
| Channel spacing (MHz) | 3.5 | 7 | 14 | 28 | 28 | 28 | 28 | 3.5 | 7 | 28 | 28 | 10 | 14 | 28 |
| Antenna gain (max.) (dBi) | 49 | 49 | 49 | 49 | 49 | 49 | 49 | 49 | 49 | 49 | 49 | 46 | 46 | 46 |
| Feeder/multiplexer loss (min) (dB) | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Antenna type | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish |
| Max. Tx output power (dBW) | 10 | 10 | 10 | 10 | 10 | 0 | -7.5 | -10 | -10 | -6 | -7 | -10 | -8 | -12 |
| e.i.r.p. (max.) (dBW) | 56 | 56 | 56 | 56 | 56 | 46 | 38.5 | 36 | 36 | 40 | 39 | 33 | 35 | 31 |
| Receiver IF bandwidth (MHz) | 1 | 2 | 4 | 17 | 24 |  |  | 3 | 5.4 | 26 | 42 | 5.2 | 10.3 | 20.4 |
| Receiver noise figure (dB) | 10 | 10 | 10 | 10 | 10 | 9.5 | 10.5 | 5 | 6 | 8 | 7.5 | 4 | 4 | 8 |
| Receiver thermal noise (dBW) | -134 | -131 | -128 | -122 | -120 |  |  | -134 | -131 | -122 | -120 | -133 | -130 | -123 |
| Nominal Rx input level (dBW) | -74 | -71 | -68 | -65 | -65 |  |  | -83 | -80 | -65 | -65 | -80 | -70 | -69 |
| Rx input level for 10-3 BER (dBW) | -116 | -113 | -111 | -109 | N/A | -108 | -109 | -123 | -120 | -105 | -105 | -120 | -112 | -109 |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference | -144 | -141 | -138 | -132 | -130 |  |  | -144 | -137 | -132 | -126 | -143 | -140 | -133 |
| Equivalent power (dB(W/4 kHz)) | - | - | - | - | - |  |  | -173 | -168 | -170 | -166 | -174 | -174 | -170 |
| Spectral density (dB(W/4 kHz)) | -144 | -144 | -144 | -144 | -144 |  |  |  |  |  |  |  |  |  |
| Refer to notes | (1),(4) | (1),(4) | (1),(4) | (1),(4) | (1),(4) |  |  | (2),(4) | (1),(4) | (2),(4) | (1),(4) | (2),(4) | (2),(4) | (2),(4) |

N/A: not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10 % of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value

Table 11

Fixed service system parameters for frequency sharing

Fixed services above 10 GHz

| Frequency band (GHz) | 14.4 - 15.35 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | 64-QAM | FM-FDM | 8-PSK | 4-PSK | 16-QAM | 64-QAM | 2-FSK | 4-PSK | MSK | 4-FSK | 4-FSK | QPSK |
| Capacity | 140 Mbit/s | 2 700 ch | 156 Mbit/s | 16 Mbit/s | 140 Mbit/s | 140 Mbit/s | 2 Mbit/s | 8 Mbit/s | 8 Mbit/s | 2 Mbit/s | 8 Mbit/s | 2 Mbit/s |
| Channel spacing (MHz) | 28 | 40 | 40 | 14 | 56 | 28 | 10 | 7 | 7 | 3.5 |  |  |
| Antenna gain (max.) (dBi) | 52 | 52 | 52 | 52 | 52 | 52 | 52 | 52 | 52 | 52 | 52 | 52 |
| Feeder/multiplexer loss (min) (dB) | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Antenna type | dish | dish | dish | dish | dish | dish | dish | dish |  |  |  |  |
| Max. Tx output power (dBW) | 5 | 3 | 0 | -10 | 7 | 5.5 | -9 | -10 | -7 | -1 | -13 | -13 |
| e.i.r.p. (max.) (dBW) | 55 | 53 | 50 | 40 | 57 | 55.5 | 41 | 40 | 43 | 49 | 37 | 37 |
| Receiver IF bandwidth (MHz) | 40 | 56 | 50 | 10 |  |  | 5.2 | 5.2 |  |  | 10 |  |
| Receiver noise figure (dB) | 4 | 10 | 5 | 6 |  |  | 13 | 4 |  |  | 8.5 |  |
| Receiver thermal noise (dBW) | -124 |  |  | -126 |  |  | -124 | -133 |  |  |  |  |
| Nominal Rx input level (dBW) | -66 | -48 | -44 | -70 |  |  | -69 | -80 |  |  |  |  |
| Rx input level for 10-3 BER (dBW) | -101 |  |  | -115 | -103.3 | -99 | -109 | -120 | -116 |  | -109 | -119 |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference | -134 |  |  | -138 |  |  | -134 | -143 |  |  |  |  |
| Equivalent power (dB(W/4 kHz)) | - |  |  |  |  |  | -165 | -174 |  |  |  |  |
| Spectral density (dB(W/4 kHz)) | -150 |  |  | -148 |  |  |  |  |  |  |  |  |
| Refer to notes | (1),(4) |  |  |  |  |  | (2),(4) | (2),(4) |  |  |  |  |

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10 % of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value

Table 12

Fixed service system parameters for frequency sharing

Fixed services above 10 GHz

| Frequency band (GHz) | 17.7 - 19.7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | 4-PSK | 4-QAM | 4-FSK | 4-PSK | 2-PSK | 4-PSK | 0-QPSK | 4-PSK |
| Capacity | 140 Mbit/s | 140 Mbit/s | 8 Mbit/s | 8 Mbit/s | 8 Mbit/s | 34 Mbit/s | 44.7 Mbit/s | 2×140 Mbit/s |
| Channel spacing (MHz) | 110 | 55 | 20 | 20 | 20 | 27.5 | 40 | 220 |
| Antenna gain (max.) (dBi) | 48 | 48 | 48 | 48 | 48 | 48 | 48 | 48 |
| Feeder/multiplexer loss (min) (dB) | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Antenna type | dish | dish | dish | dish | dish | dish | dish | dish |
| Max. Tx output power (dBW) | -10 | -4 | -16 | -6 | -9 | -8 | -9 | -10 |
| e.i.r.p. (max.) (dBW) | 35 | 41 | 29 | 39 | 36 | 37 | 36 | 35 |
| Receiver IF bandwidth (MHz) | 68 | 68 | 8 | 4 | 8 | 18 | 40 | 168 |
| Receiver noise figure (dB) | 7 | 8 | 13 | 7 | 7 | 7 | 5 | 9 |
| Receiver thermal noise (dBW) | -119 | -118 | -122 | -131 | -128 | -124 | -125 | -113 |
| Nominal Rx input level (dBW) | -63 | -64 | -65 | -65 | -65 | -65 | -70 | -62 |
| Rx input level for 10-3 BER (dBW) | -103 | -104 | -106 | -116 | -116 | -113 | -106 | -102 |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |
| Nominal long term interference (dBW) | -129 | -131 | -132 | -141 | -138 | -143 | -131 | -123 |
| Equivalent power (dB(W/4 kHz)) | - | - | - | - | - |  | -171 | -169 |
| Spectral density (dB(W/4 kHz)) | -147 | -149 | -141 | -147 | -147 |  |  |  |
| Refer to notes | (2),(4) | (2),(4) | (2),(4) | (2),(4) | (2),(4) | (2),(4) | (1) | (2),(4) |

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10 % of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value

Table 13

Fixed service system parameters for frequency sharing

Fixed services above 10 GHz

| Frequency band (GHz) | 21.12 - 29.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | 2-FSK | 2-FSK | 2-FSK | 4-PSK | 4-PSK | FM | 4-PSK | ASK | ASK | 2-FSK | 64-QAM | QPSK | 4-FSK | 4-FSK | FM |
| Capacity | 2 Mbit/s | 4 Mbit/s | 8 Mbit/s | 34 Mbit/s | 140 Mbit/s | 1 Video | 34 Mbit/s | 2 Mbit/s | 4x2 Mbit/s | 2 Mbit/s | 140 Mbit/s | 34 Mbit/s | 2 Mbit/s | 8 Mbit/s | video |
| Channel spacing (MHz) | 7 | 7 | 14 | 28 | 112 | 28 | 28 | 28 | 28 | 5 | 40 | 28 | 3.5 | 7 | 28 |
| Antenna gain (max.) (dBi) | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| Feeder/multiplexer loss (min) (dB) | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Antenna type | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish |
| Max. Tx output power (dBW) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -16 | -14 | -10 | -4 | -17 | -12 | -12 | -12 |
| e.i.r.p. (max.) (dBW) | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 31 | 33 | 37 | 43 | 30 | 35 | 35 | 35 |
| Receiver IF bandwidth (MHz) | 2 | 4 | 8 | 17 | 70 | 24 | 18 | 5 | 14 | 2 | 40 |  | 4.2 | 8.4 | 32 |
| Receiver noise figure (dB) | 9 | 9 | 9 | 9 | 9 | 9 | 12 | 4 | 4 | 11 | 5 | 9 | 8 | 8 | 4 |
| Receiver thermal noise (dBW) | -132 | -129 | -126 | -123 | -116 | -121 | -119 | -133 | -128 |  | -123 |  | -130 | -127 | -125 |
| Nominal Rx input level (dBW) | -105+M | -104+M | -103+M | -100+M | -94+M | -84+M | -87 | -108+M | -109+M | -115 | -73 |  | -76 | -73 | -65 |
| Rx input level for 10-3 BER (dBW) | -108 | - | -106 | -103 | -97 | N/A | -103 | -112 | -113 |  | -96 | -105 | -116 | -113 | N/A |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference | -142 | -139 | -136 | -133 | -126 | -131 | -129 | -139 | -136 |  | -131 |  | -136 | -133 | -131 |
| Equivalent power (dB(W/4 kHz)) | - | -170 | - | - | - | - | - | - | - |  | -171 |  | -166 | -166 | -170 |
| Spectral density (dB(W/4 kHz)) | -143 | -143 | -143 | -143 | -143 | -143 | -141 | -146 | -148 |  | -147 |  |  |  |  |
| Refer to notes | (1),(4), (7) | (1),(4), (7) | (1),(4), (7) | (1),(4), (7) | (1),(4), (7) | (1),(4), (7) | (1),(4) | (1),(4), (7) | (1),(4), (7) | (3),(4) |  |  | (1),(4) | (1),(4) | (1),(4) |

N/A: not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10 % of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value

(6)	I/N = -13 dB: objective for FS systems employing space diversity

(7)	M is fade margin, dependent on path length, polarisation and availability required

Table 14

Fixed service system parameters for frequency sharing

30-60 GHz fixed services

| Frequency band (GHz) | 31 - 31.3 | 37 - 39.5 |  |  |  |  |  |  |  |  |  |  | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | FM | 4 PSK | 2-FSK | 2-FSK | 4-PSK | 4-PSK | FM | FM | FSK | 4-OQPSK | 4-OQPSK | FM | FSK |
| Capacity | 2 Mbit/s | 8 Mbit/s | 2 Mbit/s | 8 Mbit/s | 34 Mbit/s | 140 Mbit/s | 1 Video | 1 Video | 8 Mbit/s | 4 Mbit/s | 8 Mbit/s | video | 8 Mbit/s |
| Channel spacing (MHz) | 35 | 7 | 7 | 14 | 28 | 140 | 28 | 56 | 14 | 3.5 | 7 | 28 | 20 |
| Antenna gain (max.) (dBi) | 35 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 39 |
| Feeder/multiplexer loss (min) (dB) | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 0 |
| Antenna type | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish |
| Max. Tx output power (dBW) | -13 | -14 | 0 | 0 | 0 | 0 | 0 | 0 | -15 | -12 | -12 | -13 | -23 |
| e.i.r.p. (max.) (dBW) | 20 | 30 | 44 | 44 | 44 | 44 | 44 | 44 | 29 | 32 | 32 | 31 | 16 |
| Receiver IF bandwidth (MHz) | 16 |  | 2 | 8 | 17 | 70 | 16 | 40 |  | 4.2 | 8.4 | 32 |  |
| Receiver noise figure (dB) | 10 | 10 | 11 | 11 | 11 | 11 | 12 | 12 | 10 | 10 | 10 | 10 | 10.5 |
| Receiver thermal noise (dBW) | -122 |  | -130 | -124 | -121 | -114 | -120 | -116 |  | -128 | -125 | -119 |  |
| Nominal Rx input level (dBW) | -62 | -65 | -108+M | -102+M | -99+M | -93+M | -98+M | -85+M |  | -79 | -76 | -59 |  |
| Rx input level for 10-3 BER (dBW) | -102 | -114 | -111 | -105 | -102 | -95 | N/A | N/A | -109 | -119 | -116 | N/A | -109 |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long term (dBW) interference | -132 | -134 | -140 | -134 | -131 | -124 | -130 | -126 |  | -134 | -131 | -125 |  |
| Equivalent power (dB(W/4 kHz)) | -168 |  | - | - | - |  | - | - |  | -164 | -164 | -164 |  |
| Spectral density (dB(W/4 kHz)) |  |  | -143 | -143 | -143 | -143 | -142 | -142 |  |  |  |  |  |
| Refer to notes | (2),(4) |  | (1),(4) | (1),(4) | (1),(4) |  | (1),(4) | (1),(4) |  | (1),(4) | (1),(4) | (1),(4) |  |

M: fade margin

N/A: not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10 % of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value

Table 15

Fixed service system parameters for frequency sharing

30-60 GHz fixed services

| Frequency band (GHz) | 55.2 - 57.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Modulation | 2-FSK | 2-FSK | 4-PSK | 4-PSK | FM | FM |
| Capacity | 2 Mbit/s | 8 Mbit/s | 34 Mbit/s | 140 Mbit/s | 1 Video | 1 Video |
| Channel spacing (MHz) | 14 | 14 | 28 | 140 | 28 | 56 |
| Antenna gain (max.) (dBi) | 42 | 42 | 42 | 42 | 42 | 42 |
| Feeder/multiplexer loss (min) (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| Antenna type | dish | dish | dish | dish | dish | dish |
| Max. Tx output power (dBW) | -10 | -10 | -10 | -10 | -10 | -10 |
| e.i.r.p. (max.) (dBW) | 29 | 29 | 29 | 29 | 29 | 29 |
| Receiver IF bandwidth (MHz) | 2 | 8 | 17 | 70 | 16 | 40 |
| Receiver noise figure (dB) | 11 | 11 | 11 | 11 | 12 | 12 |
| Receiver thermal noise (dBW) | -130 | -124 | -121 | -114 | -120 | -116 |
| Nominal Rx input level (dBW) | -108+M | -102+M | -99+M | -93+M | -98+M | -85+M |
| Rx input level for 10-3 BER (dBW) | -111 | -105 | -102 | -95 | N/A | N/A |
| Nominal short term (dBW) interference (% time) |  |  |  |  |  |  |
| Nominal long term interference (dBW) | -140 | -134 | -131 | -124 | -130 | -126 |
| Equivalent power (dB(W/4 kHz)) | - | - | - |  | - | - |
| Spectral density (dB(W/4 kHz)) | -143 | -143 | -143 | -143 | -142 | -142 |
| Refer to notes | (1),(4) | (1),(4) | (1),(4) |  | (1),(4) | (1),(4) |

M: fade margin

N/A: not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor)

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor)

(3)	Specified interference will have a relative contribution of no more than 10 % of total noise

(4)	The specified interference level is total power within the receiver bandwidth

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value