Recommendation T/R 75-04 (Odense 1986)

EXCHANGE OF DATA BETWEEN ADMINISTRATIONS AND BETWEEN ADMINISTRATIONS AND INTERNATIONAL ORGANISATIONS RELATING TO SPACE SYSTEMS

Recommendation proposed by the "Radiocommunications" Working Group T/WG 3 (R)

Revised text of Recommendation adopted by the "Telecommunications" Committee:

“The European Conference of Postal and Telecommunications Administrations,

considering

- that it is a matter of greatest interest that Administrations should be able to exchange information relating to frequency assignments to stations of space systems in a standardised form,

- that the administration of space systems encompasses four major categories of operation which require the exchange of data between Administrations or between Administrations and international organisations, namely: the advance publication of satellite networks, coordination between satellite networks, coordination between earth stations and terrestrial stations, and the notification to the IFRB of frequency assignments to stations of space systems (space stations and earth stations),

- that the presentation of information in a standardised form facilitates processing by computer,

- that in recent years the cost to CEPT Administrations of frequency administration has significantly increased with the substantial growth in number of satellite networks involving or affecting member countries of the CEPT,

recommends

that Administrations exchanging information relating to space systems should do so using the relevant information and the format indicated in the Annex to this Recommendation.”

Explanatory note on the presentation of information in the following Annex

The information is set out in four columns:

- The name of the data appears in the first column. Their exact definitions may be found in Appendices 3 and 4 of the Radio Regulations and in IFRB Circular No. 536.

- The type of data appears in the second column; this may be numeric (NUM), alphanumeric (ALF) or graphic (GRA).

- The format of the data appears in the third column. This format is as follows:

- Alphanumeric data are given either in the form X(N), where N is the number of characters, or else showing the exact position of each type of character. (For example: Geographical coordinates of the earth station: 999 x 9999 – 99 x 9999);

- Integral numerical data are given either in the form 9(N), where N is the number of characters, or in an expanded form, (For example: ± 999 represents a three figure integer which may be either positive or negative);

- Decimal numerical data are given in the format 9(N1), 9(N2), where N1 is the number of characters of the integer number and N2 is the number of characters of the decimal fraction. They may sometimes be given in an expanded form. (For example: ± 9.99 represents a positive or negative decimal number consisting of a single-figure integer and a two-figure decimal fraction);

- Where necessary the appropriate unit symbols appear after the data;

- For graphical data a reference to the corresponding Annex is given in the fourth column.

- Any supplementary remarks on the data are given in the fourth column. The expression ''refer to corresponding annex" which appears in the "remarks column'' means that for exchanging the corresponding data it is necessary to provide them by an annex that could consist of a graph, a table, or a formula.

Annex to Recommendation

- CHARACTERISTICS RELATING TO THE SATELLITE

- Orbit and network characteristics

| Name of data | Type | Format | Remarks |
| --- | --- | --- | --- |
| Name of satellite | ALF | X (20) |  |
| Type of orbit | ALF | X (1) | X = G Geostationary Orbit |
|  |  |  | X = N Non-geostationary Orbit |
| Nominal longitude | ALF | 999.99X (°) | X = E East |
|  |  |  | X = W West |
| Longitudinal tolerance | ALF | 9.99W - 9.99 E (°) |  |
| Inclination excursion | NUM | 9.99 (°) |  |
| Visible arc | ALF | 999X - 999X (°) | X = E East |
|  |  |  | X = W West The visible arc must be given West to East |
| Service arc | ALF | 999X - 999X (°) | X = E East X = W West |
| Angle of inclination of the orbit | NUM | 99.99 (°) |  |
| Satellite period | ALF | 999 ´ 99 | If X = D the period is expressed in days and hours If X = H the period is expressed in hours and minutes |
| Altitude of the apogee | NUM | 9 (7) km |  |
| Altitude of the perigee | NUM | 9 (7) km |  |
| Number of satellites | NUM | 99 |  |

- Characteristics relating to satellite antennas

- For each transmitting antenna

| Identity of the transmitting beam | ALF | X (9) |  |
| --- | --- | --- | --- |
| Isotropic gain of the antenna | NUM | ± 99.9 dB |  |
| Radiation pattern or gain contour plotted on the Earth’s surface | GRA |  | Refer to corresponding Annex |
| Polarisation of the antenna | ALF | X (4) | For symbols refer to IFRB circular No. 536 |
| Reference coordinates for pointing of the antenna | ALF | 999.9X; 99.9X (°) | The first X may be E or W The second X may be N or S |
| Pointing accuracy of the antenna | NUM | 9.99 (°) |  |

- For each receiving antenna

| Identity of the receiving beam | ALF | X (9) |  |
| --- | --- | --- | --- |
| Isotropic gain of the antenna | NUM | ± 99.9 dB |  |
| Reception pattern or gain contour plotted on the Earth’s surface | GRA |  | Refer to corresponding Annex |
| Polarisation of the antenna | ALF | X (4) | For symbols refer to IFRB circular No. 536 |
| Reference coordinates for pointing of the antenna | ALF | 999.9X; 99.9X (°) | The first X may be E or W The second may be N or S |
| Pointing accuracy of the antenna | NUM | 9.99 (°) |  |

- Characteristics of the satellite transponder (to be given for each transponder)

| Name of data | Type | Format | Remarks |  |  |
| --- | --- | --- | --- | --- | --- |
| Identity of the transponder (name or number) | ALF | X (2) |  |  |  |
| Central transmitting frequency of the transponder | ALF | 99999.99999X Hz | X = | í | K Kilo M Mega G Giga |
| Central receiving frequency of the transponder | ALF | 99999.99999X Hz | X = | í | K Kilo M Mega G Giga |
| Assigned frequency band | NUM | 9 (7), 9 (1) kHz |  |  |  |
| Transmitting antennas associated with the transponder | ALF | X (9) |  |  |  |
| Receiving antennas associated with the transponder | ALF | X (9) |  |  |  |
| Total peak envelope power of the transponder | NUM | ± 99.9 dBW |  |  |  |
| Maximum power density per Hz | NUM | ± 99.9 dB (W/Hz) |  |  |  |
| Receiving system noise temperature of the satellite | NUM | 9 (5) (K) |  |  |  |

- Other data relating to the satellite

| Class of station | ALF | X (2) | For code refer to IFRB circular No. 536 |
| --- | --- | --- | --- |
| Nature of service | ALF | X (2) | For code refer to IFRB circular No. 536 |
| Operating lifetime of the satellite | NUM | 9 (2) years |  |
| Date (foreseen or actual) of bringing into use | ALF | JJ/MM/AA |  |

- CHARACTERISTICS RELATING TO EARTH STATIONS

- Data relating to each site

| Name of earth station | ALF | X (20) |  |
| --- | --- | --- | --- |
| Country or geographical area in which earth station is located | ALF | X (3) | Symbol of the Preface to the International Frequency List |
| Geographical coordinates of earth station (degrees, minutes and seconds) | ALF | 999X9999 - 99X9999 | X = E or W for the first X X = N or S for the second X |
| Altitude | NUM | ± 9999 m |  |
| Angle of elevation of the horizon around earth station | TABLE or GRA |  | Refer to corresponding Annex |
| Standard defined transmission/reception zone (zone represented by a symbol) | ALF | X (20) |  |
| Types of earth station at this site | ALF | X (10) |  |
| Minimum angle of elevation of the earth station antenna | NUM | ± 99.9 (°) |  |

- Data relating to each type of earth station

| Name of data | Type | Format | Remarks |
| --- | --- | --- | --- |
| Type of earth station | ALF | X (10) |  |
| Frequency band | ALF | X (15) |  |
| Isotropic gain of the antenna when transmitting | NUM | ± 99.9 dB |  |
| Isotropic gain of the antenna when receiving | NUM | ± 99.9 dB |  |
| Radiation pattern of the antenna when transmitting | ALF GRA | or X (12) | Refer to corresponding Annex |
| Radiation pattern of the antenna when receiving | ALF GRA | or X (12) | Refer to corresponding Annex |
| Polarisation of the antenna when transmitting | ALF | X (4) | For code refer to IFRB circular No. 536 |
| Polarisation of the antenna when receiving | ALF | X (4) | For code refer to IFRB circular No. 536 |
| Width of main radiation beam when transmitting | NUM | 9 (3), 9 (2) (°) |  |
| Width of main radiation beam when receiving | NUM | 9 (3), 9 (2) (°) |  |
| Receiving system noise temperature of the earth station | NUM | 9 (5) (K) |  |

- Other data relating to the earth station

| Class of station | ALF | X (2) | For code refer to IFRB circular No. 536 |
| --- | --- | --- | --- |
| Nature of service | ALF | X (2) | For code refer to IFRB circular No. 536 |
| Date (foreseen or actual) of bringing into use | ALF | JJ/MM/AA |  |

- CHARACTERISTICS RELATING TO THE CARRIERS

- For each carrier

| Name or number of the transponder(s) used for the type of carrier | ALF | N X (2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Maximum value of peak envelope power supplied to the input of the antenna |  |  |  |  |  |
| - for the earth station | NUM | ± 99.9 dBW |  |  |  |
| - for the satellite | NUM | ± 99.9 dBW |  |  |  |
| Maximum power density per Hz | NUM | ± 99.9 dB (W/Hz) |  |  |  |
| Class of emission | ALF | X (5) |  |  |  |
| Indication, if possible, of the frequency (-ies) used for the type of carrier | ALF | N 99999.99999X Hz | X = | í | K Kilo M Mega G Giga |
| Necessary (occupied) bandwidth | NUM | 9 (7), 9 (1) kHz |  |  |  |
| Bandwidth allotted on the satellite | NUM | 9 (7), 9 (1) kHz |  |  |  |

- For FDM/FM or SCPC/FM carriers

| Name of data | Type | Format | Remarks |
| --- | --- | --- | --- |
| Telephone channel capacity of each carrier of this type | NUM | 9 (4) |  |
| Highest frequency of the baseband | NUM | 9 (4) kHz |  |
| Lowest frequency of the baseband | NUM | 9 (4) kHz |  |
| R.m.s. frequency deviation of the 0 dBm0 test-tone | NUM | 9 (4) kHz |  |
| R.m.s. frequency deviation of a multi-channel signal | NUM | 9 (4) kHz |  |
| Characteristics of the energy dispersal where relevant | ¾ |  | Refer to corresponding Annex |
| Peak-to peak frequency deviation | NUM | 9 (4) kHz |  |
| Sweep frequency of the energy dispersal waveform | NUM | 9 (4) kHz |  |
| Pre-emphasis characteristics | ¾ |  | Refer to corresponding Annex |

- For TV/FM carriers

| Type of multiplexing of the video signal and the sound signal(s) or other signal(s) | ¾ |  | Refer to corresponding Annex |
| --- | --- | --- | --- |
| Standard of the TV signal |  |  |  |
| ¾ TV system | ALF | X (1) | For code refer to IFRB circular No. 536 |
| ¾ Colour system | ALF | X (1) | For code refer to IFRB circular No. 536 |
| Pre-emphasis characteristics | ¾ |  | Refer to corresponding Annex |
| Frequency deviation for the reference frequency of the pre-emphasis characteristic | NUM | 9 (4) kHz |  |

- For digital carriers

| Bit rate | ALF | 999X bit/s | X = K Kilo X = M Mega |
| --- | --- | --- | --- |
| Type of modulation and number of phases | ALF | X (5) |  |
| Type of demodulation (coherent or differential) | ALF | X (1) | C = Coherent D = Differential |

- OTHER CHARACTERISTICS ASSOCIATED WITH THE SATELLITE NETWORK

| Minimum equivalent noise temperature of the satellite link | NUM | 9 (5) (K) |  |
| --- | --- | --- | --- |
| Associated transmission gain | NUM | ± 99.9 (dB) |  |
| Annexed special section of the IFRB weekly circular in which advance publication of the satellite network appeared | ALF | X (16) |  |
| Annexed special section of the IFRB weekly circular in which the request for coordination with other satellite networks appeared | ALF | X (16) |  |