Radio Interface Specification (RIS) implementation of ECC/DEC/(13)01 on the harmonised use, free circulation and exemption from individual licensing of Earth Stations On Mobile Platforms (GSO ESOMPs) within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz

Foreword

The ECC has decided that RIS implementations of ECC deliverables should be uploaded to the ECO website in order to help administrations fill out the EFIS database.

This RIS implementation is limited to the harmonised use, free circulation and exemption from individual licensing of Earth Stations On Mobile Platforms (GSO ESOMPs) within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz.

RIG II Template for Earth Stations On Mobile Platforms (GSO ESOMPs) within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz

Radio Interface Notification by an administration

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Fixed Satellite Service (FSS) |  |
|  | 2 | Application | Earth Stations On Mobile Platforms (GSO ESOMPs) | Applies to ESOMPs operating in a geostationary satellite network. |
|  | 3 | Frequency bands | To be specified by the administration, within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz | See ECC/DEC/(13)01 Decides 2a, 2b and 2c. |
|  | 4 | Channelling | Not applicable | Defined by the satellite network operator. |
|  | 5 | Modulation / Occupied bandwidth | Not applicable | Defined by the satellite network operator. |
|  | 6 | Direction / Separation | 27.5-30.0 GHz (Earth-to-space) 17.3-20.2 GHz (space-to-Earth) |  |
|  | 7 | Transmit power / Power density | Maximum e.i.r.p. to be specified by the administration, with a value within the range 60-55 dBW | The maximum e.i.r.p. of ESOMP equipment shall be limited to a value within the range from 60 to 55 dBW as specified in Annex 1 of ECC/DEC/(13)01. |
|  | 8 | Channel access and occupation rules | Not applicable | Defined by the satellite network operator. |
|  | 9 | Authorisation regime | Exemption from individual licensing | No registration and/or notification is required (commonly known as “license exempt”). |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions |  | Additional technical and operational requirements are described in ECC Decision (13)01 for GSO ESOMPs in general, GSO ESOMPs installed onboard an aircraft and GSO ESOMPs installed on-board vessels for the protection of other authorised users in the operating frequency band. Note that national requirements related to the protection of aircraft from GSO ESOMPs operating on the earth surface in the vicinity of an airfield should be provided to the ECO (see Decides 2e of ECC/DEC/(13)01). |
| Informative part | 12 | Planned changes | --- |  |
|  | 13 | Reference | ECC Decision (13)01 ETSI EN 303 978 |  |
|  | 14 | Notification number | --- |  |
|  | 15 | Remarks | Definitions |  |
|  |  |  | Abbreviations e.i.r.p. equivalent isotropically radiated power |  |