- Recommendation T/R 12-01 E (Helsinki 1991, revised Rottach-Egern, February 2010)

- PREFERRED CHANNEL ARRANGEMENTS FOR FIXED SERVICE SYSTEMS

- OPERATING IN THE FREQUENCY BAND 37.0 - 39.5 GHz

Recommendation adopted by the Working Group ''Spectrum Engineering'' (SE)

- “The European Conference of Postal and Telecommunications Administrations,

considering

- that CEPT has a long-term objective to harmonise the use of frequencies throughout Europe in order to make the most effective use of the spectrum available;

- that within the European Common Allocation and the ITU Radio Regulations (RR), the band 37.0 - 37.5 GHz is allocated, on primary basis, to the fixed service and the band 37.5 - 39.5 GHz is allocated, on equal primary basis, to the mobile, fixed and fixed-satellite services (space-to-Earth); moreover, the band 37.0-38.5 GHz is also allocated on equal primary basis to the space research service;

- that ERC/DEC/(00)02 designates the band 37.5 - 39.5 GHz for the use of point-to-point fixed links and uncoordinated FSS earth stations shall not claim protection from FS stations;

- that recommendations ITU-R F.1669 and ITU-R SF.1573 defines the protection criteria and the maximum power flux density (PFD) for the protection of FS systems from the FSS space stations;

- that Recommendation ITU-R F.749 contains radio-frequency arrangements for systems of fixed service operating in the 38 GHz band.

recommends

- that CEPT administrations should consider the radio frequency channel arrangements in the band 37.0 - 39.5 GHz given in Annex A;

- that in order to improve spectrum utilisation, administrations may wish to consider using parts of the centre gap and guard bands as given in Annex A.”

- Note:

Please check the ECO web site (http//:www.ero.dk) for the up to date position on the implementation of this and other ECC and ERC Recommendations.

- Annex A

Radio-frequency channel arrangements in the band 37.0 - 39.5 GHz

The radio frequency channel arrangement for carrier spacings of 112 MHz, 56 MHz, 28 MHz, 14 MHz, 7 MHz and 3.5 MHz shall be derived as follows:

Let	fr	be the reference frequency of 38 248 MHz;

fn	be the centre frequency (MHz) of a radio-frequency channel in the lower half of the band;

f´n	be the centre frequency (MHz) of a radio-frequency channel in the upper half of the band;

then the frequencies of individual channels are expressed by the following relationships:

- For systems with a carrier spacing of 112 MHz:

- Lower half of band:	fn = (fr - 1246 + 112n) 	MHz

- Upper half of band:	f´n = (fr + 14 + 112n) 	MHz	where:	n = 1, 2, 3, …. 10

- For systems with a carrier spacing of 56 MHz:

- Lower half of band:	fn = (fr - 1218 + 56n) 	MHz

- Upper half of band:	f´n = (fr + 42 + 56n) 		MHz	where:	n = 1, 2, 3, …. 20

- For systems with a carrier spacing of 28 MHz:

- Lower half of band:	fn = (fr - 1204 + 28n) 	MHz

- Upper half of band:	f´n = (fr + 56 + 28n) 		MHz	where:	n = 1, 2, 3, …. 40

- In addition, according to Recommends 2, where practical, Administrations may consider the use of channels with index n = 0 and 41

- For systems with a carrier spacing of 14 MHz:

- Lower half of band:	fn = (fr - 1197 + 14n) 	MHz

- Upper half of band:	f´n = (fr + 63 + 14n) 		MHz	where:	n = 1, 2, 3, …. 80

- In addition, according to Recommends 2, where practical, Administrations may consider the use of channels with index n = -2,-1, 0 and 81 and 82, 83

- For systems with a carrier spacing of 7 MHz:

- Lower half of band:	fn = (fr - 1193.5 + 7n) 	MHz

- Upper half of band:	f´n = (fr + 66.5 + 7n) 	MHz	where:	n = 1, 2, 3, …. 160

- In addition, according to Recommends 2, where practical, Administrations may consider the use of channels with index n = -5, -4,-3, -2, -1, 0, and 161, 162, 163, 164, 165, 166

- For systems with a carrier spacing of 3.5 MHz:

- Lower half of band:	fn = (fr - 1191.75 + 3.5n) 	MHz

- Upper half of band:	f´n = (fr + 68.25 + 3.5n) 	MHz	where:	n = 1, 2, 3, …. 320

- In addition, according to Recommends 2, where practical, Administrations may consider the use of channels with index n = -11, -10, to 0 and 321 to 332

- A fully homogeneous band plan, based on a 3.5 MHz raster, with channel edges aligned as shown in Figure 1.

Figure 1:	Occupied spectrum 37.0 - 39.5 GHz band