CEPT/ERC/RECOMMENDATION 12-06 E (Rome 1996, revised Rottach Egern, February 2010)

PREFERRED CHANNEL ARRANGEMENTS FOR FIXED SERVICE SYSTEMS OPERATING IN THE FREQUENCY BAND 10.7 - 11.7 GHz

Recommendation adopted by the Working Group “Spectrum Engineering” (SE)

“The European Conference of Postal and Telecommunications Administrations,

considering

- that CEPT has a long-term objective to harmonise the use of frequencies throughout Europe in order to make the most effective use of the spectrum available,

- that the band 10.7 - 11.7 GHz is co-primarily allocated to the fixed service and to the fixed-satellite service (space-to-Earth); however, ERC decision (00)08 specify that uncoordinated Earth stations in the FSS should operate on a non-protected basis,

- that ERC decision (00)08 also limits the use of the band 10.7 - 11.7 GHz to high capacity (i.e. 140 Mbit/s or higher) point-to-point links,

- that ITU-R Recommendation F.387 also gives channel arrangements for the fixed service in this band,

- that, when very high capacity links are required, it may be achieved by using wider channel bandwidth,

- that ITU Radio Regulation Footnote 5.340 states that all emissions in the band 10.68 - 10.7 GHz are prohibited except for those provided for by Radio Regulation Footnote 5.483,

recommends

1.	that in the 10.7 - 11.7 GHz band, CEPT administrations may consider the preferred radio frequency channel arrangement for digital point-to-point fixed wireless systems with a duplex frequency of 530 MHz as presented in Annex A,

2.	that in the 10.7 - 11.7 GHz band, CEPT administrations may consider the preferred radio frequency channel arrangement for digital point-to-point fixed wireless systems with a duplex frequency of 490 MHz as presented in Annex B,

3.	that CEPT administrations may consider merging any of two adjacent 40 MHz channels recommended in Annex A.1 or Annex B.1 to create one 80 MHz channel, with centre frequency lying in the central point of the distance between the merged channels. The same spectral efficiency should be maintained. To assist international co-ordination, administrations may refer to the channel identifiers described in Annex C,

4.	that CEPT administrations may consider merging any of two adjacent 28 MHz channels recommended in Annex A.2 or Annex B.2 to create one 56 MHz channel, with centre frequency lying in the central point of the distance between the merged channels. The same spectral efficiency should be maintained. To assist international co-ordination, administrations may refer to the channel identifiers described in Annex C.”

Note:

Please check the ECO web site (http//:www.ero.dk) for the up to date position on the implementation of this and other ECC and ERC Recommendations.

Annex A

RECOMMENDATION FOR CHANNEL ARRANGEMENT WITH DUPLEX FREQUENCY 530 MHz

let	f0 be the frequency of the centre of the band of frequencies occupied (MHz); and

fn be the centre frequency of a radio frequency channel in the lower half of the band (MHz); and

f’n be the centre frequency of a radio frequency channel in the upper half of the band (MHz);

and f0 = 11200 MHz

- The radio frequency channel arrangement for digital point-to-point fixed wireless systems based on 40 MHz channel separation will contain 11 go/return channels and the individual channels should be derived as follows:

Lower half of the band	  fn = (f0 - 505 + 40n) 	MHz

Upper half of the band	f’n = (f0 + 25 + 40n) 	MHz	where n = 1, 2, 3, … 9, 10, or 11

The channel arrangement is shown in Figure 1. (all frequencies in MHz)

Guard band	Centre Gap	Guard Band

|  | 11 x 40 MHz channels |  | 11 x 40 MHz channels |  |
| --- | --- | --- | --- | --- |

10700	10715	11155	11245	11685	11700 MHz

Figure 1: Channel Arrangement for 40 MHz channel separation with duplex frequency 530 MHz

- The radio frequency channel arrangement for digital point-to-point fixed wireless systems based on 28 MHz channel separation will contain 16 go/return channels and the individual channels should be derived as follows:

Lower half of the band	 fn = (f0 - 505 + 28n) 	MHz

Upper half of the band	f’n = (f0 + 25 + 28n) 	MHz	where n = 1, 2, 3, ……. 15, or 16

The channel arrangement is shown in Figure 2. (all frequencies in MHz)

Guard band	Centre Gap	Guard Band

|  | 16 x 28 MHz channels |  | 16 x 28 MHz channels |  |
| --- | --- | --- | --- | --- |

10700	10709	11157	11239	11687	11700 MHz

Figure 2: Channel Arrangement for 28 MHz channel separation with duplex frequency 530 MHz

| Parameter | Annex A.1 | Annex A.2 |
| --- | --- | --- |
| XS / MHz | 40 | 28 |
| n | 11 | 16 |
| f1 / MHz | 10735 | 10723 |
| f11 / MHz | 11135 | 11003 |
| f12/ MHz |  | 11031 |
| f16/ MHz |  | 11143 |
| f’1 / MHz | 11265 | 11253 |
| f’11 / MHz | 11665 | 11533 |
| f’12/ MHz |  | 11561 |
| f’16/ MHz |  | 11673 |
| Z1S / MHz | 35 | 23 |
| Z2S / MHz | 35 | 27 |
| YS / MHz | 130 | 110 |
| DS / MHz | 530 | 530 |

Table 1: Calculated parameters according to ITU-R Rec. F.746

XS 	Separation between centre frequencies of adjacent channels

YS 	Separation between centre frequencies of the closest go and return channels

Z1S 	Separation between the lower band edge and the centre frequency of the first channel

Z2S 	Separation between centre frequencies of the final channel and the upper band edge

DS 	Duplex spacing (fn’ - fn)

Note: On a national level, CEPT administrations not implementing ERC DEC(00)08, may wish to use 14 MHz, 7 MHz channel arrangement by subdividing the 28 MHz channel arrangement.

Annex B

RECOMMENDATION FOR CHANNEL ARRANGEMENT WITH DUPLEX FREQUENCY 490 MHz

let	f0 be the frequency of the centre of the band of frequencies occupied (MHz); and

fn be the centre frequency of a radio frequency channel in the lower half of the band (MHz); and

f’n be the centre frequency of a radio frequency channel in the upper half of the band (MHz);

and f0 = 11200 MHz

- The radio frequency channel arrangement for digital point-to-point fixed wireless systems based on 40 MHz channel separation will contain 12 go/return channels and the individual channels should be derived as follows:

Lower half of the band	  fn = (f0 - 505 + 40n) 	MHz

Upper half of the band	 f’n = (f0 - 15 + 40n)  	MHz	where n = 1, 2, 3, … 10, 11 or 12

The channel arrangement is shown in Figure 3. (all frequencies in MHz)

Guard band	Centre Gap	Guard Band

|  | 12 x 40 MHz channels |  | 12 x 40 MHz channels |  |
| --- | --- | --- | --- | --- |

10700	10715	11195	11205	11685	11700 MHz

Figure 3: Channel Arrangement for 40 MHz channel separation with duplex frequency 490 MHz

- The radio frequency channel arrangement for digital point-to-point fixed wireless systems based on 28 MHz channel separation will contain 17 go/return channels and the individual channels should be derived as follows:

Lower half of the band	 fn = (f0 - 505 + 28n) 	MHz

Upper half of the band	f’n = (f0 - 15 + 28n)  	MHz	where n = 1, 2, 3, ……. 16, or 17

The channel arrangement is shown in Figure 4. (all frequencies in MHz)

Guard band	Centre Gap	Guard Band

|  | 17 x 28 MHz channels |  | 17 x 28 MHz channels |  |
| --- | --- | --- | --- | --- |

10700	10709	11185	11199	11675	11700 MHz

Figure 4: Channel Arrangement for 28 MHz channel separation with duplex frequency 490 MHz

| Parameter | Annex B.1 | Annex B.2 |
| --- | --- | --- |
| XS / MHz | 40 | 28 |
| n | 12 | 17 |
| f1 / MHz | 10735 | 10723 |
| f11 / MHz | 11135 | 11003 |
| f12/ MHz | 11175 | 11031 |
| f16/ MHz |  | 11143 |
| f17/ MHz |  | 11171 |
| f’1 / MHz | 11225 | 11213 |
| f’11 / MHz | 11625 | 11493 |
| f’12/ MHz | 11665 | 11521 |
| f’16/ MHz |  | 11633 |
| f’17/ MHz |  | 11661 |
| Z1S / MHz | 35 | 23 |
| Z2S / MHz | 35 | 39 |
| YS / MHz | 50 | 42 |
| DS / MHz | 490 | 490 |

Table 2: Calculated parameters according to ITU-R Rec. F.746

XS 	Separation between centre frequencies of adjacent channels

YS 	Separation between centre frequencies of the closest go and return channels

Z1S 	Separation between the lower band edge and the centre frequency of the first channel

Z2S 	Separation between centre frequencies of the final channel and the upper band edge

DS 	Duplex spacing (fn’ - fn)

Note: On a national level, CEPT administrations not implementing ERC DEC(00)08, may wish  to use 14 MHz, 7 MHz channel arrangement by subdividing the 28 MHz channel arrangement.

Annex C

### CHANNEL IDENTIFIERS FOR DERIVATIVE 80 MHz AND 56 MHz CHANNELS

The derivative 80 MHz channels (ref. recommends in Annex A.1 and Annex B.1) can be identified by using the following numbering and illustrated in Figures 5 (a) and 5 (b), respectively:

| recommends in Annex A.1 |  |  |
| --- | --- | --- |
| lower half of the band: | fn = f0 - 485 + 40n MHz |  |
| upper half of the band: | fn’ = f0 + 45 + 40n MHz | where n = 1, 2, 3, ….9, 10 |
| recommends in Annex B.1 |  |  |
| lower half of the band: | fn = f0 - 485 + 40n MHz |  |
| upper half of the band: | fn’ = f0 + 5 + 40n MHz | where n = 1, 2, 3, ….10, 11 |

The derivative 56 MHz channels (ref. recommends in Annex A.2 and Annex B.2) can be identified by using the following numbering and illustrated in Figures 6 (a) and 6 (b), respectively:

| recommends in Annex A.2 |  |  |
| --- | --- | --- |
| lower half of the band: | fn = f0 - 491 + 28n MHz |  |
| upper half of the band: | fn’ = f0 + 39 + 28n MHz | where n = 1, 2, 3, ….14, 15 |
| recommends in Annex B.2 |  |  |
| lower half of the band: | fn = f0 - 491 + 28n MHz |  |
| upper half of the band: | fn’ = f0 - 1 + 28n MHz | where n = 1, 2, 3, ….15, 16 |

In all cases	f0 = 11200 MHz

Note: The numbering is just for identification of the channelling. It should be noted, that adjacent channel numbers can not be used on the same physical link due to channel overlap. See diagrams below for channel arrangement examples.

(a) duplex frequency 530 MHz

(b) duplex frequency 490 MHz

Figure 5: illustration of the channel identifiers for derivative 80 MHz channels 
from the channel arrangements recommended in Annex A.1 (a) and Annex B.1 (b)

(a) duplex frequency 530 MHz

(b) duplex frequency 490 MHz

Figure 6: illustration of the channel identifiers for derivative 56 MHz channels 
from the channel arrangements recommended in Annex A.2 (a) and Annex B.2 (b)