CEPT/ERC/RECOMMENDATION 12-05 E (Rome 1996)

Harmonised radio frequency channel arrangements for digital terrestrial fixed systems operating in the band 10.0 - 10.68 GHz

Text of the Recommendation adopted by Working Group “Spectrum Engineering” (WG SE):

“The European Conference of Postal and Telecommunications Administrations,

considering

- that CEPT has a long term objective to harmonise the use of frequencies throughout Europe,

- that CEPT should develop radio frequency channel arrangements in consultation with organisations developing standards for radio systems, in order to make the most effective use of the spectrum available,

- that any radio frequency channel arrangement should incorporate a provision for the future introduction of improved equipment standards,

noting

- that CEPT/ERC Report 25 allocates the paired bands 10.15 - 10.3 and 10.5 - 10.65 GHz for fixed systems including point to multipoint, and the removal of the Mobile service from the band 10.5 - 10.68 GHz,

- that Article S5 of the Radio Regulations allocates the band 10 - 10.45 GHz on an equal primary basis to the Fixed and Radiolocation services, and the band 10.6 - 10.68 GHz to the Fixed, Earth Exploration Satellite (passive), Radio Astronomy, and Space Research services,

- that CEPT/ERC has defined the band 10.0 - 10.68 GHz as one of the harmonised tuning ranges for ENG/OB equipment,

- that in some countries sharing studies indicate sharing between Fixed and Radar systems is feasible,

- that in some countries it is not currently practicable to implement the paired band 10.15 - 10.3 and 10.5 - 10.65 GHz due to military requirements,

- that in some countries coordination is required between the Fixed and Radioastronomy services in the 10.6 - 10.68 GHz band (see CEPT/ERC Report 36),

recommends

- that CEPT Administrations which have the paired bands 10.15 - 10.3 and 10.5 - 10.65 GHz available for the Fixed service should follow the recommended radio frequency channel arrangements given in Annex A,

- that administrations which have different paired bands available may allow the current channel arrangements to remain unchanged until the year 2015, and may determine, on a national basis, the degree of protection given. International coordination between existing and new systems shall be in accordance with article S11 of the Radio Regulations,

- that administrations may choose to allocate certain parts of the bands to one way links, for example ENG/OB links within the tuning range 10.0 - 10.68 GHz, in agreement to the recommended radio frequency channel arrangements given in Annex A,

- that further sharing studies between Fixed and Radar systems in these bands need to be completed.”

## Annex A

## DERIVATION OF RADIO FREQUENCY CHANNELS

The radio frequency channel arrangements for the 10 to 10.68 GHz band are based on channel slots of 0.5 MHz and are derived as follows:

fp=f0−1701+0.5p (MHz)

where p ranges from 0 to 1359

Let f0 be the reference frequency of the pattern = 11701 MHz

and fp be the lower edge frequency (MHz) of each slot

Within the band 10.15 - 10.3 GHz (p=300 to 599) paired with 10.5 - 10.65 GHz (p=1000 to 1299) equipment will utilise a duplex spacing of 350 MHz.

For point-to-multipoint systems discrete channel centre frequencies can be obtained from within the 0.5 MHz slots.

For other systems the channel centre frequencies could be derived by multiplication of the 0.5 MHz slots as indicated below:

Let	f0 be the reference frequency of the pattern = 11701 MHz
fn be the centre frequency (MHz) of a radio frequency channel in the lower half of the band 
fn' be the centre frequency (MHz) of a radio frequency channel in the upper half of the band

The frequencies of individual channels are expressed by the following relationships:

- For systems with a carrier spacing of 28 MHz
Lower half of band: fn=(f0−1561+28n) MHz
Upper half of band: fn'=(f0−1211+28n) MHz
where	n = 1,2 ... 5

- For systems with a carrier spacing of 14 MHz
Lower half of band: fn=(f0−1554+14n) MHz
Upper half of band: fn'=(f0−1204+14n) MHz
where	n = 1,2 ... 10

- For systems with a carrier spacing of 7 MHz
Lower half of band: fn=(f0−1550.5+7n) MHz
Upper half of band: fn'=(f0−1200.5+7n) MHz
where	n = 1,2 ... 20

- For systems with a carrier spacing of 3.5 MHz
Lower half of band: fn=(f0−1552.25+3.5n) MHz
Upper half of band: fn'=(f0−1202.25+3.5n) MHz
where	n = 1,2 ... 42

Table 1.  Calculated parameters according to ITU-R Recommendation F 746

| XS MHz | n | f1 MHz | fn MHz | f1' MHz | fn' MHz | Z1S MHz | Z2S MHz | YS MHz | DS MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 1,2 ... 5 | 10168 | 10280 | 10518 | 10630 | 18 | 20 | 238 | 350 |
| 14 | 1,2 ... 10 | 10161 | 10287 | 10511 | 10637 | 11 | 13 | 224 | 350 |
| 7 | 1,2 ... 20 | 10157.5 | 10290.5 | 10507.5 | 10640.5 | 7.5 | 9.5 | 217 | 350 |
| 3.5 | 1,2 ... 42 | 10152.25 | 10295.75 | 10502.25 | 10645.75 | 2.25 | 4.25 | 206.5 | 350 |

XS	Separation between centre frequencies of adjacent channels

YS	Separation between centre frequencies of the closest go and return channels

Z1S	Separation between the lower band edge and the centre frequency of the first channel

Z2S	Separation between the centre frequency of the final channel and the upper band edge

DS	Duplex Spacing (fn'−fn)

Table 2.  Occupied spectrum: 10.15 - 10.65 GHz band

|  |  |  |  |  |  | Centre Gap 200 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 300 × 0.5 MHz channel slots |  |  |  |  |  |  |  |  |  | 300 × 0.5 MHz channel slots |  |  |  |  |  |  |  |
| é |  |  |  |  |  |  | é |  |  | é |  |  |  |  |  |  | é |  |  |
|  | 10150 MHz |  |  |  | 10300 MHz |  |  |  |  |  | 10500 MHz |  |  |  | 10650 MHz |  |  |  |  |

| Guard band 4 MHz |  |  |  |  |  | Centre Gap 210 MHz |  |  |  |  |  |  |  |  |  | Guard band 6 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 × 28 MHz channels |  |  |  |  |  |  |  |  | 5 × 28 MHz channels |  |  |  |  |  |  |  |
| é |  |  |  |  |  |  | é |  |  | é |  |  |  |  |  |  | é |  |  |
|  | 10150 MHz |  |  |  | 10294 MHz |  |  |  |  |  | 10504 MHz |  |  |  | 10650 MHz |  |  |  |  |

| Guard band 4 MHz |  |  |  |  |  | Centre Gap 210 MHz |  |  |  |  |  |  |  |  |  | Guard band 6 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 × 14 MHz channels |  |  |  |  |  |  |  |  | 10 × 14 MHz channels |  |  |  |  |  |  |  |
| é |  |  |  |  |  |  | é |  |  | é |  |  |  |  |  |  | é |  |  |
|  | 10150 MHz |  |  |  | 10294 MHz |  |  |  |  |  | 10504 MHz |  |  |  | 10650 MHz |  |  |  |  |

| Guard band 4 MHz |  |  |  |  |  | Centre Gap 210 MHz |  |  |  |  |  |  |  |  |  | Guard band 6 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 20 × 7 MHz channels |  |  |  |  |  |  |  |  | 20 × 7 MHz channels |  |  |  |  |  |  |  |
| é |  |  |  |  |  |  | é |  |  | é |  |  |  |  |  |  | é |  |  |
|  | 10150 MHz |  |  |  | 10294 MHz |  |  |  |  |  | 10504 MHz |  |  |  | 10650 MHz |  |  |  |  |

| Guard band 0.5 MHz |  |  |  |  |  |  | Centre Gap 203 MHz |  |  |  |  |  |  |  |  |  | Guard band 2.5 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 42 × 3.5 MHz channels |  |  |  |  |  |  |  |  |  | 42 × 3.5 MHz channels |  |  |  |  |  |  |  |  |
| é |  |  |  |  |  |  |  | é |  |  | é |  |  |  |  |  |  | é |  |  |  |
|  | 10150 MHz |  |  |  |  | 10297.5 MHz |  |  |  |  |  | 10500.5 MHz |  |  |  | 10650 MHz |  |  |  |  |  |