# Coexistence study between WBB (low/medium power) and FSS receiving earth stations

The present study determines the coexistence conditions between WBB LMP system and FSS receiving stations in the 3.8-4.2 GHz frequency band while identifying the key parameters on the co-channel protection distances.

The study considered the example of protection of two real FSS earth stations in France. The first one is the Rambouillet Teleport site (Eutelsat) located at coordinates longitude 1.7825° and latitude 48.54944°.

Recommendation ITU-R P.452-16 was used to model propagation losses. This propagation model provides a prediction method for the evaluation of propagation losses from about 0.1 GHz to 50 GHz, accounting for both clear-air and hydrometeor scattering interference mechanisms.

The models within Recommendation ITU-R P.452 are designed to calculate propagation losses not exceeded for time percentages over the range 0.001 ≤ p ≤ 50%. This assumption does not imply the maximum loss will be at p = 50%. In this section, the used percentage of time is set to 20%.

Predicting with more accurately the path loss, requires knowing the exact position of all obstacles including building in the path between the transmitter and the receiver. This can be achieved by using a national terrain and buildings database: in France they are both provided by the ”Institut National de l’Information Géographique et Forestière” (IGN), with a precision of 5 m (1m is also possible but it increases significantly the computation time and provides a level of accuracy that is not necessarily needed for the purposes of this report). However, the terrain profile is taken from SRTM data of 1s.

Once the path profile between the transmitter and the receiver is determined, the P.452 is applied for a time percentage of 20%.

## characteristics of the Fixed satelitte system

In the 3.8-4.2 GHz band, licensed earth stations communicate only with geostationary satellites (GSO).

### Receiver characteristics: FSS ES

FSS parameters are based on characteristics provided by ITU-R WP 4A as well as on characteristics of existing FSS ES where indicated, as shown in the table below.

Table 1: FSS earth station parameters

| Parameter | Typical value |
| --- | --- |
| Antenna size (m) | 2.4-12 m |
| ES Carrier Bandwidth (MHz) | 40 MHz |
| Antenna reference pattern | ITU-R S.465 |
| Receiving system noise temperature | 120 K for small antennas (1.2-3 m) 70 K for large antennas (4.5 metres and above) |
| ES antenna elevation pointing | 10 degrees |
| ES Antenna Centre Height above ground | 10 m |

#### FSS protection criteria

Table 2: Protection criteria for FSS (in-band)

| Frequency Ranges |  | Percentage of time for which the I/N value could be exceeded (%) | I/N Criteria (dB) |
| --- | --- | --- | --- |
| 3 800-4 200 MHz | Long term Short term | 20% 0.005% (NOTE) | −10.5 −1.3 (NOTE) |

NOTE: Studies using these short-term protection criteria could be assessed on the basis that these values were put forward by WP 4A to facilitate and complete the work for WRC-23 agenda items and these values may evolve in the future based on inputs to the ITU-R. WP 4A has not completed its work in developing short-term protection criteria, however WP 5D is invited to consider these short-term protection criteria to the extent practicable.

## Characteristics of WBB LMP providing Local Area Connectivity

Table 3: Parameters of the WBB LMP providing local area network connectivity in 3.8-4.2 GHz from UK/NOR approach

| Parameter | Low Power BS | Medium Power BS |
| --- | --- | --- |
| Bandwidth | 10 MHz to 100 MHz | 10 MHz to 100 MHz |
| Antenna height | Outdoor: Limited to a maximum of 10 m above ground Indoor: Any height within building | No limit |
| Deployment scenario | Outdoor/indoor or Indoor-only | Rural areas only |
| BS Tx EIRP limit (for AAS & non-AAS) | 24 dBm / carrier for carriers ≤ 20 MHz; or 18 dBm / 5 MHz for carriers > 20 MHz | 42 dBm / carrier for carriers ≤ 20 MHz; or 36 dBm / 5 MHz for carriers > 20 MHz |
| Maximum terminal power | Mobile/nomadic: TRP 28 dBm (Note 2) Fixed: EIRP 28 dBm (Note 2) | Mobile/nomadic: TRP 28 dBm (Note 2) Fixed: TRP 28 dBm and EIRP 35 dBm/5 MHz (Note 1) |
| Note 1: Higher EIRP limit for fixed terminals in the medium power BS case is to account for the use case of Fixed Wireless Access (FWA) Note 2: The authorisation of 28 dBm includes a 2 dB tolerance consistent with the European harmonisation. |  |  |

Table 4: Parameters of the WBB providing local area network connectivity in 3.8-4.2 GHz

| Parameter | Low Power BS | Medium Power BS |
| --- | --- | --- |
| TDD / FDD | TDD | TDD |
| BS Sectorization | 1 | 1 |
| Use case information single BS cell range (AAS & non-AAS) | 0.05–0.4 km for outdoor BS typical antenna height above ground 10 m 0.01–0.1 km for Indoor BS any height within building | 0.05–3 km (check WP5D guidance) antenna height above ground 5 m ~ 30 m for indoor/outdoor BS |
| BS TDD activity factor | 25-75% | 25-75% |
| Network loading factor | 50% (for the network) 50% and/or 100% (for a single base station) | 50% (for the network) 50% and/or 100% (for a single base station) |
| BS Frequency reuse | 1 | 1 |
| Terminal antenna gain | -4 dBi | -4 dBi |
| Antenna gain for AAS/non-AAS | See Table 7 for AAS and Table 8 for non-AAS |  |
| MIMO (number of RF chains) | 4T/4R for Medium Range and Local Area BS |  |
| MIMO gain | 6 dB for 4T/4R |  |
| BS Noise Figure | 13 – 5 (for MIMO processing gain) dB Note: not include MIMO Rx gain, if for MIMO processing gain is used | 10 – 5 (for MIMO processing gain) dB Note: not include MIMO Rx gain, if for MIMO processing gain is used |
| UE Noise Figure | 9 dB | 9 dB |
| Interference criteria | I/N threshold -6 dB and/or 5% throughput loss | I I/N threshold -6 dB and/or 5% throughput loss |
| Estimated Indoor/outdoor UE percentage | For outdoor BS 70%/30% For indoor BS 100%/0% | For outdoor BS rural 50%/50% For indoor BS 100%/0% For incremental approach: outdoor BS suburban 70%/30% |
| Building entry loss (dB) | A value of 12 dB is used in the UK coordination approach for indoor WBB LMP BSs, and should be applied in initial studies of indoor WBB LMP. | A value of 12 dB is used in the UK coordination approach for indoor WBB LMP BSs7, and should be applied in initial studies of indoor WBB LMP. |
| UE height | For outdoor BS: 1.5 m For indoor BS: all UE are indoor at the same floor as indoor BS at 1.5 m above floor | For outdoor BS: 1.5 m For indoor BS: all UE are indoor at the same floor as indoor BS at 1.5 m above floor |

Table 7 and Table 8 define antenna patterns for simulation of AAS and non-AAS systems respectively. It should be noted that the coordination of new WBB LMP licence applications in UK/NOR national frameworks do not account for the pointing direction and pattern of the antenna, applying licenced EIRP in the coordination where WBB LMP is the interferer and peak Rx gain where WBB LMP is the victim for coordination.  To account for this difference in approach, both an omnidirectional antenna pattern and those defined in Tables 5 and 6 could be studied for all cases. Omnidirectional antennas can be used for simulation, if antenna pointing is not known.

Table 5: Parameters for the incremental studies (first step) for non-AAS and AAS Medium Power BS

| Parameter | Medium Power BS |
| --- | --- |
| Maximum antenna height | 30 m |
| Deployment scenario | Rural, suburban, urban |
| BS Tx EIRP limit (for AAS & non-AAS) | 51 dBm/100 MHz |

Table 6: Tx Power for AAS Medium Power BS

| TRP | AAS configurations | Antenna gain |
| --- | --- | --- |
| 32.55 dBm | 4x4 elements | 18.45 dBi |
| 29.55 dBm | 4x8 elements | 21.45 dBi |

Table 7: WBB LMP Base station AAS antenna characteristics

| AAS antenna pattern | Recommendation ITU-R M.2101 (section 5) Extended AAS Model 3GPP TR 38.803 (Section 5.2.3.2.4) |
| --- | --- |
| Element gain (dBi) | 6.4 |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 2) | 8 x 8 elements 4 x 4 elements (4 x 8 elements could be used as well) |
| Horizontal/Vertical radiating element/sub-array spacing, dh /dv | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array, Msub (Note 1) | 3 |
| Vertical radiating element spacing in sub-array, dv,sub (Note 1) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt, θsubtilt (degrees) (Note 1) | 3 |
| Base station horizontal coverage range (degrees) | ±60° |
| Base station vertical coverage range (degrees) (Note 3) | 0 to -30 |
| Mechanical downtilt (degrees) | 0 and 10 |
| Note 1: Only needed when subarray antenna model is used Note 2: For the small/micro cell case, 8 × 8 means there are 8 vertical and 8 horizontal radiating elements. For the extended AAS model case, 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. Note 3: The vertical coverage range is given in global coordinate system, i.e. 0° being at the horizon. |  |

Table 8: Directional WBB LMP base station non-AAS antenna characteristics

| Non-AAS Antenna Pattern | ITU-R F.1336 |
| --- | --- |
| Sectorization | 1 sector for single BS; tri-sector for network layout simulation |
| Non-AAS BS downtilt (degrees) | 0 and 10 |
| Frequency reuse | 1 |
| Non-AAS BS antenna pattern | Recommendation ITU-R F.1336 (recommends 3.1) ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336. Vertical beamwidths of actual antennas may also be used when available. |
| Antenna polarisation | Linear ±45° |
| Non-AAS BS Tx and Rx antenna gain per RF chain (including system loss) | 10 dBi for Medium Range (MR) BS 6 dBi for Local Area (LA) BS 0 dBi (omni) for indoor BS |

Note: The combination of power and antenna gain should be such, that max defined EIRP per sector/BS is not exceeded.

## Simulation results

### Non AAS BS system

In the present study, only the interference coming from the BS (WBB LMP downlink) is evaluated. The simulation results are presented in terms of I/N which is calculated as follows:

I/N (dB) = e.i.r.p(dBm/5MHz)  – PL (dB) + Gr(dBi) – N (dBm/5MHz)

The propagation model used in the studies is ITU-R P.452-16, with time percentage p=20%, using a national terrain and building database. The relief and the building are respectively taken from the SRTM(1s) and IGN(5m) data.

The protection criterion considered is that of the long term (I/N = -10 dB) which leads to a threshold interference level of:

Ilong term (dBm/5MHz) = -10 + 10log10(1.38 10-23 TFSS°K×B5MHz)= -123 dBm/5MHz

For a receiver noise temperature of 70°K.

𝐺𝑟: Gain towards the horizon for a given azimuth of the FSS receiving from a geostationary satellite constellation. When the FSS station sweeps geostationary orbit 𝐺𝑟 = max{Gain(θ)} where θ=(FSS→Geo ^ FSS→5G site).

#### Teleport site (Eutelsat) Earth STATION IN Rambouillet

Non-AAS WBB LMP BS

Interference from low power vertical 5G BS (e.i.r.p 18 dBm/5MHz)

Figure 1: I/N from BS of antenna height of 10m with 0° downtilt (left) and of 20m with 10° downtilt (right)

Figure 2: The areas where the I/N exceeds -10 dB for antenna height of 10m with 0° downtilt (left) and of 20m with 10° downtilt (right)

AAS WBB LMP BS

In this study the case where the FSS ES and the WBS BS are pointing towards each other is considered.

Figure 3:

For the AAS case the base station is taken in the same azimuth as the earth station. The propagation model is P.452 with a time percentage between 0 and 50%. The AAS antenna is pointing in a given direction within its coverage area (combination of its mechanical and electrical pointing). At each iteration, a user equipment is assumed to be randomly deployed within the BS service area

Figure 4: time percentage of I/N (dB)

The second case assumes a clutter modelling ITU-R P.2108 (section 3.2). The clutter was assumed systematically at the BS end, assuming that the local-area network would be deployed below the clutter height in order to mitigate interference into existing services. The percentage of locations is set to 50% and 30 for respectively urban and sub-urban environment.

Figure 5:

## analysis of study results

From the simulation results, it can be noted that the maximum e.i.r.p and the maximum antenna height (or alternatively maximum EIRP for various range of antenna heights) of the base station have an important impact on the co-channel protection distances.

Thus, it may be necessary to define some limitation to antenna height of medium power station, since it impacts the interfering distance. Other metrics for technical conditions could be used instead, such as defining a maximum pfd at a certain distance of the 5G vertical stations.

Additionally, limiting the bandwidth for a given 5G vertical local coverage would ensure the possibility of

a later deployment of an earth stations, in close vicinity and receiving in other part of the 3.8-4.2 GHz band

further 5G vertical local coverage in the same area and in another block

Of course, a spectrum mask to be met by 5G vertical stations is also essential to ensure the protection of earth stations in adjacent blocks.

In addition, the restriction to “only rural” for medium power could be questioned since, in some countries, earth stations are also encouraged to be installed in rural areas, including to facilitate their protection from MNO networks operating in the frequency band 3.4-3.8 GHz. On the other hand, possible additional restrictions should be investigated for the antenna height given the relevance of this parameter in terms of interfering distances.

Table 9: Results for non-AAS system

| Site: Rombouilet | BS Tx Power | Antenna height | Down tilt (non-AAS) | Protection distance (km) (L.T) (Note) |
| --- | --- | --- | --- | --- |
|  | L.P | 5 | 0 | 5.3 |
|  |  | 10 | 0 | 14.12 |
|  |  | 20 | 10 | 15.48 |
|  | M.P | 10 | 0 | 17.5 |
|  |  | 20 | 10 | 21.68 |
|  |  | 30 | 10 | 26.6 |
| Note: Results obtained with real terrain, building, clutter data and while considering a percentage of time of 20% for P.452. |  |  |  |  |

Table 10: Results for (4x8) AAS system

| Site: Rombouilet | AAS System | Antenna height | Environment | Clutter | Coordination distance (km) (L.T***) (Note) |
| --- | --- | --- | --- | --- | --- |
|  | MP: 51 dBm/100 MHz | 20 | Urban | No clutter | 52 |
|  |  |  |  | Clutter (P.2108, p = 50%) | 27 |
|  |  |  | Sub-urban | Clutter (P.2108, p= 30%) | 30 |
| Note: AAS the pointing is varying in direction of ES |  |  |  |  |  |

By considering a percentage of location of 50% and 30% for respectively urban and sub-urban environments.

Document 5A/395, available at https://www.itu.int/md/meetingdoc.asp?lang=en&parent=R19-WP5A-C-0395

In UK medium power BS are permitted for rural only

The BS cell range depends on the antenna height and indoor/outdoor deployment

Picocell Noise Figure as per ITU-R M.2292

Microcell Noise Figure as per ITU-R M.2292

ITU-R M.2292

The background for the UK coordination approach BEL value of 12 dB is described in Ofcom’s consultation Enabling opportunities for innovation: Shared access to spectrum supporting mobile, see paragraph 5.54, and is based on the BEL CDFs of traditional and thermally efficient buildings defined in ITU-R P.2109.