# Introduction

Italy is one of the CEPT countries that has the largest number of Fixed Service (FS) point-to-point (P-P) links in the 3.8-4.2 GHz band (almost 400 FS links). The 3.8-4.2 GHz band is used for transport links of broadcasting infrastructure mainly by the exclusive concessionaire of the public radio and television service in Italy. These fixed radio links are used for the transport of audio/video content necessary for the performance of the Public Broadcasting Service. The extensive use of the 3.8-4.2 GHz band by FS links increased in 2018 when, in order to free up the 3.6-3.8 GHz band for 5G, it was necessary to migrate the links to this band. On that occasion, operators, who were required to change their equipment in order to be able to operate in the adjacent band, were assured that the new bandwidth would remain unchanged in the long term. For this reason, it is very important for Italy to study the coexistence conditions between the wireless broadband low and medium power (WBB LMP BS) systems and the FS links that guarantee the protection and the future evolution and development of the fixed service.

This document firstly provides the coexistence scenario and a description of the FS links distribution at national level. Then the key parameters that are considered in the simulations and the adopted methodology to assess interference into FS P-P links from WBB LMP BS are presented. In this contribution results on separation distances and some considerations on exclusion areas are shown for all the scenarios (i.e. dense urban, urban suburban and rural).

# INterference Scenario

Figure 1 shows the coexistence scenario (worst case situation) that will be considered in the study between WBB LMP BS transmitters and the FS receiver in the band 3.8-4.2 GHz which are:

Co-frequency, interferer: outdoor Low power BS, victim: FS receiver;

Co-frequency, interferer: outdoor Medium power BS, victim: FS receiver.

Figure 1: Coexistence scenario between WBB LMP BS transmitters and the FS receiver

## Fixed Service Parameters

Figure 2 shows the FS links antenna heights distribution at national level: most of the FS links have a height of 15 meters even if some of them can reach up to 100 meters antenna height.

The minimum FS link length is about 9.6 km and the maximum value is 238 km with a typical link length value of about 68 km.

Figure 2: Real deployment of FS antenna heights in Italy

Figure 3-Figure 6 represent the distribution of the FS antenna heights for each possible scenario of the FS receivers. As shown in Figure 6, most of the FS receivers are placed in rural areas (327 FS receivers) and some even in dense urban areas (27 FS receivers). Only a little part of the FS receivers is in urban (24 FS receivers) and in suburban areas (13 FS receivers).

Figure 3: Distribution of antenna heights of the FS receivers in dense urban areas in Italy

Figure 4: Distribution of antenna heights of the FS receivers in urban areas in Italy

Figure 5: Distribution of antenna heights of the FS receivers in suburban areas in Italy

Figure 6: Distribution of antenna height of the FS receivers in rural areas in Italy

The studies will consider real FS parameters (Table 1).

Table 1: FS receiver: scenario parameters

| Parameter | Value |  |
| --- | --- | --- |
| Bandwidth | 28 MHz/29 MHz | It depends on real deployment |
| Noise figure | 3.5 dB |  |
| hFS_rx | Real deployment | See previous figures for each scenario values |
| GFS_max | Real deployment | 33 dBi to 47 dBi |
| FS antenna elevation | Real deployment | -3.3° to 3.1° |
| Feeder Loss | Real deployment | 1 dB to 8.75 dB |

The ITU -R Recommendation F.699-8 will be used to calculate the antenna pattern of the FS receiver. In Figure 7 the antenna pattern is plotted for different values of the antenna diameters which correspond to different antenna gains.

Figure 7: FS antenna pattern (Recommendation ITU-R F.699-8)

## WBB LMP system Parameters for 3800-4200 MHz

Parameters for the WBB LMP 3800-4200 MHz are based on the UK/NOR approach and are listed in ANNEX 1:.

Table 2 provides the parameters on WBB LMP BS that will be considered in the coexistence studies.

Table 2: WBB LMP: scenario parameters

| Parameter | Value | Note |
| --- | --- | --- |
| Bandwidth | 100 MHz |  |
| hBS,LP | 10 m | Base station height low power BS scenario |
| hBS,MP | 30 m (Note 1) | Base station height medium power BS scenario |
| BS Tx EIRP limit (Low power BS) | 18 dBm/5 MHz | 11 dBm/MHz |
| BS Tx EIRP limit (Medium power BS) | 36 dBm /5 MHz | 29 dBm/MHz |
| Mechanical downtilt (degrees) | 0 |  |
| Note 1: National median value of MNO’s BS antenna height |  |  |

## Propagation model

The propagation model that will be used in the simulations follows the Recommendation ITU-R P.452-16 at 20% of time (i.e. the interference is characterised as the interference power that is exceeded by 20% of the time at the victim receiver input).

Clutter is applied on the WBB side for the low power BS in dense urban, urban and suburban scenarios (as the maximum antenna height is limited to 10 m). ITU-R P.2108-1, with 50% of locations (dense urban and urban) and 30% of locations (suburban), is used to calculate the average clutter loss.

No clutter is applied for the medium power BS in rural scenario, as the antenna height of the WBB system is considered above the average clutter level.

## Simulation scenarios

Table 3 summarises the simulation scenarios that are considered in the studies:

Table 3: Simulation scenarios

| Environment | Analysis | Clutter Loss (dB) (Note 1) | hWBB [m] | BS EIRPWBB | NFS_Rx |
| --- | --- | --- | --- | --- | --- |
| Dense Urban | Low Power | 31 | 10 | 18 dBm/5 MHz | 27 |
| Urban | Low Power | 31 | 10 | 18 dBm/5MHz | 24 |
| Suburban | Low Power | 29 | 10 | 18 dBm/5MHz | 13 |
| Rural | Medium Power | 0 | 30 | 36 dBm/5 MHz | 327 |
| Note 1: Evaluated with Recommendation ITU-R P.2108-1 (09/2021), “Prediction of clutter loss” |  |  |  |  |  |

# Minimum Coupling Loss approach

The adopted methodology to assess interference into FS P-P receivers from WBB LMP systems will be based on the Minimum Coupling Loss approach (MCL) taking into account I/N protection criteria:

I/N (∆f, d, ϑ1, ϑ2) = Pt - Att(∆f) + Gt(ϑ1) + Gr(ϑ2) – 10*LOG10(BWBB/BFS) – PL(d) – N

In the formula:

Pt is the transmitted power (dBm) of the WBB LMP interferer;

∆f is the difference (in MHz) between the carrier frequencies of the WBB LMP interferer and the FS receiver victim systems;

Att(∆f) is the Net Filter Discrimination (It is equal to zero in co-frequency analysis);

Gt(ϑ1) is the gain (dBi) of the WBB LMP interferer antenna at angle ϑ1;

Gr(ϑ2) is the gain (dBi) of the FS victim antenna at angle ϑ2;

ϑ1 and ϑ2 are the angles between the interfering system (WBB LMP BS) site and victim system (FS receiver) site as in Figure 8;

BWBB is the WBB LMP bandwidth;

BFS is the FS receiver bandwidth;

PL(d) is path loss attenuation (dB) due to the propagation along distance d (km);

d is the distance between the interfering system (WBB LMP BS) and victim system (FS receiver);

N is the noise level (dBm) of the victim FS receiver.

Figure 8: ϑ1 and ϑ2 angles

The sharing studies will evaluate the exclusion area around the FS receiver (i.e. geographical area within which WBB LMP BSs are not allowed to transmit) and the separation distance between the WBB LMP BS and the FS receiver (i.e. the distance required between interferer and victim to ensure that there is no interference) so that the I/N protection criterion = -10 dB (FS protection) is respected. This is the long term interference protection criteria suggested by Recommendation ITU-R F.758 and should be used with a time percentage of pt = 20%.

# Results

Results considering both flat terrain and the altimetry of real FS deployments are provided for Low Power BS in dense urban, urban and suburban scenarios and for Medium Power BS in rural scenarios (see Table 3).

## Flat terrain results

In case of flat terrain, the results in terms of separation distances and exclusion areas have been derived considering that the only effect that can affect the propagation is the Earth curvature (i.e. no altimetry of real deployments is considered).

Figure 9 represents the distribution of separation distances between Low Power BS and FS receivers as a function of the FS antenna heights in dense urban, urban and suburban scenarios. The maximum antenna gains of the considered FS receivers range from 37 dBi to 42 dBi and the feeder loss values range between 1 dB and 5 dB.

The separation distances vary from 6 km to almost 46.3 km depending on the FS receiver antenna height (HRX), the maximum FS antenna gain and the FS elevation angle.

Figure 9: Separation distances vs FS antenna heights

(dense urban, urban and suburban scenarios in flat terrain)

Figure 10 shows the distribution of separation distances between Medium Power BS and FS receivers as a function of the FS antenna heights in rural environment.

The maximum antenna gains of the considered FS receivers range from 33 dBi to 47 dBi and the feeder loss values ranges between 1 dB and 8.75 dB.

The separation distances vary from 34.5 km to almost 101 km depending on the FS receiver antenna height, maximum FS antenna gain and FS elevation angle.

Figure 10 shows the trendline of the separation distances that has been obtained: the separation distance increases, increasing the height of the FS receivers. In case of Medium Power WBB in rural areas, the separation distances are higher than in the Low Power WBB since the effect of clutter is not considered in any way, the transmitted power is higher than for the Low Power WBB as well the BS antenna height.

Figure 10: Separation distances vs FS antenna heights (rural scenario in flat terrain)

## Real deployment considerations

When considering real altimetry where the FS links are placed, two different cases can arise:

- the geographical conformation of the territory can help the coexistence and, compared to considering flat terrain, the separation distance decreases;

- the geographical conformation of the territory can help the propagation and, compared to considering flat terrain, the separation distance increases.

The second case occurs in those situations where the height of the transmitter and/or receiver is such that there is no longer an intersection of the earth with the first Fresnel ellipsoid.

The following figures represent examples of the effect of altimetry in the two different mentioned situations. Figure 11 shows the effect of the obstruction of the Fresnel zone due to the Earth curvature in flat terrain for a link having a fixed distance d = 29 km.

Figure 11: Effect of the obstruction of the Fresnel zone due to the Earth curvature in flat terrain

Figure 12 shows a link with the same distance (i.e. 29 km) but considering the actual terrain elevation between the transmitter and the receiver. In this case the propagation of the signal is immediately hindered by a mountain that completely intersects the first Fresnel zone and therefore coexistence is favoured.

Figure 12: Effect of the altimetry that intersects the Fresnel zone

Figure 13 shows instead a situation in which the altimetry helps the propagation. The considered link has the same distance as in the previous cases (29 km), but in this case the conformation of the territory raises one end of the link so that there is no intersection with the first Fresnel zone.

Figure 13: Effect of the altimetry that helps the propagation

Therefore the case of flat terrain is not the most precautionary situation from the point of view of protection of the incumbent and the altimetry of the real terrain should be taken into account in the coexistence assessments on a case-by-case basis.

## Comparison between flat terrain and real deployment results

### Results for Low Power WBB

This paragraph contains some examples of separation distances between WBB LP BS and FS receivers and of exclusion areas around the FS receiver obtained considering both flat terrain and the real altimetry.

Figure 14 and Figure 15 represent the exclusion areas and the separation distances for the same FS receiver (FS receiver A) obtained considering both flat terrain (Figure 14) and the real terrain (Figure 15). As it can be seen from the figures, in this case the altimetry helps the coexistence and the separation distance decreases from 22.3 km for flat terrain to 2.5 km considering the terrain. The same decrease occurs with respect to the exclusion area that is equal to 37.2 km2 in case of flat terrain and decreases to 7.6 km2 when the actual terrain elevation is taken into account.

Figure 14: Separation distance and exclusion area around the FS receiver A in flat terrain

Figure 15: Separation distance and exclusion area around the FS receiver A considering the actual terrain elevation

Figure 16 and Figure 17 show the exclusion areas and the separation distances for the same FS receiver (FS receiver B) considering flat terrain and the real altimetry. In this case the separation distance, considering flat terrain, is equal to 17.8 km and it increases to 29 km when the altimetry is taken into account. In this case the real terrain favours the propagation: the increase in the separation distance when the terrain is considered is due to the phenomenon explained in the previous paragraph in Figure 13 (i.e. the altimetry ensures that there is no obstruction of the first Fresnel zone).

In this situation the separation distance does not provide a real indication of the actual effect of the interference and it is more important to consider the exclusion areas: considering flat terrain the exclusion area is 29.5 km2 and it decreases to 16.6 km2 in the real scenario, but in this second case the exclusion area is not continuous resulting in a greater separation distance.

Figure 16: Separation distance and exclusion area around the FS receiver B in flat terrain

Figure 17: Separation distance and exclusion area around the FS receiver B considering the actual terrain elevation

When altimetry is taken into account in coexistent assessments the exclusion area may not be continuous, therefore the separation distance alone could be used as a coexistence condition only if the exclusion zone remains continuous.

### Results for Medium Power WBB

Figure 18 shows an example of exclusion area around a FS receiver installed in rural area: the green area represents the case of flat terrain and purple one the exclusion area that has been obtained considering the real altimetry.

Figure 18: Example of exclusion areas around a FS receiver in rural area in case of flat terrain (green area) and in case the altimetry is considered (purple area)

As can be seen from Figure 18, the exclusion area obtained considering the flat terrain (an extension of 3942 km2) is much larger than areas that have been obtained in case of Low Power WBB. This is due to the fact that the effect of clutter is not considered in the coexistence analysis, the transmitted power for Medium Power BS is higher than for the Low Power WBB as well the BS antenna height.

Furthermore, unlike in the case of the Low Power WBB, the exclusion area increases when the real conformation of the territory is considered (in the case of the example the exclusion area is 6766.8 km2): this is due to the fact that the FS receiver antennas in rural areas are located at high altitudes and, when there are no obstructions due to the orography of the terrain, the propagation is improved because the effective height of the FS receiver is equal to the sum of the antenna height and the altitude of the terrain (i.e. the FS receiver is at a greater height than in the case of flat terrain case. See lower part of the Figure 18). However, if obstructions due to the orography of the terrain prevail around the FS receiver, the interference area is still limited (as can be clearly seen in the upper part of the figure).

Figure 19 shows the distribution of the altitudes at which the FS receivers are placed at national level. As can be seen, in Italy many FS receivers are located at around 900 m altitude in the rural case and 90% of the FS sites are located at more than 400 m altitude. This implies that the effect shown in Figure 13 is even more evident.

The result presented in Figure 18 represents only an example of an exclusion area for the rural case that increases when altimetry is taken into account, but simulations have shown that this effect occurs for many national FS links in rural areas.

It follows from the above considerations that in all real cases not only the separation distance but also the exclusion area should be assessed. The evaluation of both separation distances and exclusion areas should be performed taking into account the altimetry of the terrain. For these reasons a case-by-case coexistence assessment is needed.

Figure 19: Distribution of the altitudes of the FS receivers in rural area

### Cross-border considerations

Figure 20 shows an example of results of co-frequency coexistence analysis between LMP WBB and FS considering real FS receivers deployed near the borders.

The coloured areas show the exclusion area that exceeds the national border:

red for the medium power WBB case;

blue for the low power WBB case.

Since the area near the border is predominantly rural, the effect of clutter was not considered.

As shown in Figure 20, the exclusion zones resulting from the coexistence analyses carried out for WBB BSs installed beyond national borders are smaller in the low power case (blue areas) than in the medium power case (red areas), but the impact remain significant in both cases.

It should also be noted that exclusion zones, although not continuous, can reach significant distances beyond the borders (again longer in the medium power than in the low power WBB case).

Appropriate mitigation techniques can facilitate coexistence between WBB and FS systems both at national level and with the neighbouring countries.

The results of the studies show that coexistence between FS and both low and medium power WBB systems cannot be managed at national level only but may require coordination and related bilateral or even multilateral agreements among neighbouring countries.

Figure 20: Example of exclusion areas beyond the border for MP WBB (red areas) and LP WBB

(blue areas)

# Conclusions

This contribution provides some information on the real deployment of FS links in Italy in terms of heights distribution and clutter distribution. Italy is one of the countries with the highest number of fixed links (almost 400 FS links) and some of them are located in dense urban and urban areas.

The presence of FS receivers in dense urban and urban areas could limit the possible use of WBB LMP systems in these scenarios that could be of interest for these applications.

In order to guarantee coexistence on a non-interference basis, it is necessary to study which are the coexistence conditions that guarantee the protection of the FS systems in a real deployment of the FS links.

Co-frequency analyses have been performed. Results in terms of separation distances between the WBB LMP BS interferer and the real FS victim receivers have been shown considering flat terrain for dense urban, urban suburban and rural scenarios.

The following hypotheses have been considered for the dense urban, urban and suburban coexistence analysis:

Low Power WBB deployment.

Co-frequency coexistence analysis.

LP WBB parameters in Table 2.

Clutter loss = 31 dB for dense urban and urban.

Clutter loss = 29 dB for suburban.

FS parameters in real deployment in Italy:

FS antenna heights range from 7.3 m to 100 m (most of the FS receivers in the considered scenarios have an antenna height of 15 meters).

The maximum antenna gains range from 37 dBi to 42 dBi.

The FS elevation angles range from -2.2° to 3.1°.

The feeder loss values range between 1 dB and 5 dB.

The separation distances in the direction of the FS main lobe that have been obtained in the coexistence analyses in case of LP WBB in dense urban, urban and suburban scenarios considering flat terrain vary from 6 km to almost 46.3 km depending on the FS receiver antenna height, the maximum FS antenna gain, the FS elevation angle and the feeder loss.

The following hypotheses have been considered for the rural coexistence analysis:

- Only Medium Power WBB deployment.

Co-frequency coexistence analysis.

MP WBB parameters in Table 2.

No clutter loss has been considered.

FS parameters in real deployment in Italy:

- FS antenna heights range from 2.1 m to 84 m (most of the FS receivers in rural scenarios have an antenna height of 15 meters).

The maximum antenna gains range from 33 dBi to 47 dBi.

The FS elevation angles range from -3.3° to 2.4°.

The feeder loss values range between 1 dB and 8.75 dB.

The separation distances in the direction of the FS main lobe that have been obtained in the coexistence analyses in case of MP WBB in rural scenario considering flat terrain vary from 34.5 km to 101 km depending on the FS receiver antenna height, the maximum FS antenna gain, the FS elevation angle and the feeder loss.

Based on the hypothesis considered, co-frequency coexistence between FS and MP WBB systems in rural areas represents a worst case coexistence scenario. In Italy, given the presence of many FS receivers in rural areas (327 links), the co-frequency coexistence between FS and MP WBB systems could be difficult if mitigation techniques will not be taken into account.

Sharing studies considering real data of the FS links as well as real terrain elevation have been performed.

Comparing the results of the analysis in terms of separation distances and exclusion areas, considering both the flat terrain and the real terrain elevation, some conclusions can be drawn:

The altimetry of the real terrain should be taken into account in the coexistence assessments on a case by case basis.

This is due to the fact that the case of flat terrain is not the most precautionary situation from the point of view of protection of the incumbent, since the altimetry does not always hinder propagation, but sometimes favours it, depending on whether or not the first Fresnel zone of the link is intersected by the terrain.

In the second case propagation is favoured and the separation distance increases, but the exclusion area may increase or decrease.

The exclusion area increases in particular when clutter loss is not taken into account (rural scenario with WBB BS considered above the average clutter height) and FS receivers are at high altitudes.

When altimetry is taken into account:

the exclusion area may not be continuous;

the separation distance alone could be used as a coexistence condition only if the exclusion zone remains continuous;

the calculation of the exclusion area and the population within it provides more relevant information on the reduction of the impact of the interference in real cases.

It follows from the above considerations that in all real cases not only the theoretical maximum separation distance but also the exclusion area should be assessed, taking into account the altimetry of the terrain, therefore a case-by-case coexistence assessment is needed (see Figure 17).Since it is not possible a priori to define the technical conditions that guarantee the protection of the incumbent FS and its future development (i.e. a case-by-case coexistence assessment is needed) particular attention shall be paid to the borders.

With regard to cross-border, the coexistence analyses carried out for possible WBB BSs installed beyond national borders have shown that the exclusion zones to protect FS links in rural area close to the national borders are smaller in the low power case (blue areas) than in the medium power case (red areas), but the impact remains significant in both cases.

Appropriate mitigation techniques can facilitate coexistence between WBB and FS systems, both at national level and with the neighbouring countries.

According to the analyses coexistence between FS and both low and medium power WBB systems cannot be managed at national level only but requires coordination and related bilateral or even multilateral agreements among neighbouring countries.

WBB LMP PARAMETERS

Table 4: Parameters of the WBB LMP providing local area network connectivity in 3.8-4.2 GHz from UK/NOR approach

| Parameter | Low Power BS | Medium Power BS |
| --- | --- | --- |
| Bandwidth | 10 MHz to 100 MHz | 10 MHz to 100 MHz |
| Antenna height | Outdoor: Limited to a maximum of 10 m above ground Indoor: Any height within building | No limit |
| Deployment scenario | Outdoor/indoor or Indoor-only | Rural areas only |
| BS Tx EIRP limit (for AAS & non-AAS) | 24 dBm / carrier for carriers ≤ 20 MHz; or 18 dBm / 5 MHz for carriers > 20 MHz | 42 dBm / carrier for carriers ≤ 20 MHz; or 36 dBm / 5 MHz for carriers > 20 MHz |
| Maximum terminal power | Mobile/nomadic: TRP 28 dBm (Note 2) Fixed: EIRP 28 dBm (Note 2) | Mobile/nomadic: TRP 28 dBm (Note 2) Fixed: TRP 28 dBm and EIRP 35 dBm/5 MHz (Note 1) |
| Note 1: Higher EIRP limit for fixed terminals in the medium power BS case is to account for the use case of Fixed Wireless Access (FWA) Note 2: The authorisation of 28 dBm includes a 2 dB tolerance consistent with the European harmonisation. |  |  |

Summary of coexistence studies between WBB and FS (Italy)

This Annex contains an update of the ECC Report in sections 3.1.2, 5.1.1, 6.2.2 and 6.2.3 and a proposal of tables that summaries the main results obtained from coexistence studies between LMP WBB and FS considering flat terrain.