Electronic Communications Committee (ECC)

within the European Conference of Postal and Telecommunications Administrations (CEPT)

NUMBERING AND ADDRESSING

IN

MACHINE-TO-MACHINE (M2M) COMMUNICATIONS

Luxembourg, November 2010

# EXECUTIVE SUMMARY

In recent years a rapid development of a variety of services that make use of Machine-to-Machine (M2M) communication has taken place. M2M is a communication technology where data can be transferred in an automated way with little or no human interaction between devices and applications.

In this report, M2M communications are understood as a fully or largely automated communication (data transfer) between two or more information and communications (ICT) entities, that may be part of a predetermined group. A goal of this report is to help National Regulatory Authorities (NRA) in their considerations on efficient numbering and addressing solutions for Machine to Machine (M2M) applications and to avoid possible exhaustion of existing numbering ranges. Although a clear need for harmonization of numbers for use of these applications is not foreseen, the information in this report might be useful as policies on M2M numbering in most countries are still in an early stage of development and information on best practices does not yet exist.

It is expected that the development of M2M applications will have an impact on national numbering plans because machines need to be uniquely addressed in order to communicate with them, or rather to enable them to communicate with each other. The natural intention by operators and M2M SPs is to use E.164 numbers from the existing numbering plan for M2M services because of the relatively simple implementation in already existing network infrastructure.

The conclusion based on analysis from this report is as follows:

- The expected annual growth rate of required M2M numbers between years 2010 and 2020 is approximately 20%, but this is just a rough estimate and the actual figure may differ from this estimation significantly.

- In the long run IPv6 addressing will become very important as an alternative numbering resource for at least part of the M2M applications. However, there is much uncertainty what period is involved so that numbering policy for M2M should be flexible enough to be also a solution for the longer term.

- A significant number of CEPT countries do not have sufficient capacity in their existing numbering plan to accommodate numbering requirements for M2M applications.

- A complete harmonised approach on possible M2M numbering solutions is not needed in Europe.

One of the main goals of numbering policy for M2M applications is that the provision of numbers for this kind of applications will not compromise specific end-user interests which are covered by the existing national numbering plans, such as user-friendliness and tariff transparency.

The report recommends as follows:

- Taken into account the potential extensive growth rates of M2M applications or other similar telecommunication services, the NRAs should, in cooperation with market players, establish a numbering policy for these kinds of services as part of the national numbering plan. The policy shall be flexible enough to accommodate also a longer term solution.

- As a long term solution IPv6 addresses, or numbers/addresses other than E.164 numbers should preferably be used for device based communication applications. These numbering/addressing schemes or switching from E.164 numbering plan to a new plan should not prohibit market development or competition.

- Four numbering options were discovered for M2M applications: A) Existing mobile number ranges, B) A new number range, C) An international numbering solution and D) Network internal numbers. In a short and a medium term the four numbering options (A-D) can be envisaged; based on the current knowledge of the market none of these have a clear preference. In cases where there is a need to distinguish M2M services and the traditional mobile services option A has to be excluded. Options B and D are needed if the growth of M2M is larger than expected or if there is no adequate space in existing number ranges in use. Option C depends on ITU.

- There are possible situations where a new number range (option B) should be opened. For example, the number range in question may require different regulatory treatment, e.g. relating access to emergency services, or the services to be provided have certain characteristics (e.g. M2M applications in fixed networks) where existing mobile number ranges may not be adequate.

- For the services where number portability is an obligation or where subscribers can take advantage on number portability, option D should not be used.

Additional considerations regarding possible new number range(s) for M2M communications or other device-based applications:

- The new numbering range(s) accommodate future mass volume applications, such as M2M applications, and do not have prohibitive conditions for such applications.

- The NRA should ensure that the new numbering range(s) are not used as an alternative to existing numbering ranges to escape regulatory requirements.

- As some existing regulatory requirements may not be relevant or useful for certain applications such as M2M applications, exceptions regarding existing regulatory requirements could be applied to new numbering range(s) accommodating these applications. For example, for M2M numbering resources the issue of user friendliness (e.g. short numbers) is not relevant. Number portability requirements should be applied to network external numbers but not to network internal numbers, as this is not applicable in the latter case.

- The number length of network external numbers should be as long as possible (max 15 digits according to E.164). The length of network internal numbers should not be predetermined.

Table of Contents

EXECUTIVE SUMMARY	2

1	INTRODUCTION	5

2	Scope	5

3	General Description of M2M Communications	6

3.1	Parties of M2M Communications	6

3.2	Access from the M2M Devices to Network	7

3.2.1	Fixed Access	7

3.2.2	Mobile Access	8

3.2.3	Conclusion on Access	8

4	Assessment of Numbering Capacity Required for M2M Communications	8

4.1	Methodology	8

4.2	Assumptions and Scenarios	8

4.2.1	Starting Points	8

4.2.2	Growth Assumptions Based on Population	9

4.2.3	Growth Assumptions Based on Households	10

4.2.4	Other Growth Assumptions	10

4.3	Conclusions on Required M2M Numbering Capacity	10

5	M2M Numbering and Addressing	11

5.1	Introduction to M2M Numbering	11

5.2	Numbering Resources for M2M Applications	11

5.3	Adequacy of National Mobile Number Ranges for M2M Services	13

5.4	Strategy for M2M Numbering and Addressing	14

6	IMPLEMENTATION ASPECTS OF A NEW NUMBER RANGE	14

6.1	Network External and Network Internal Numbers	14

6.2	Installed Base of M2M Applications	14

6.3	Assignment Mechanism	15

6.4	Use of Long Numbers	15

7	Regulatory requirements	15

8	Conclusions and Recommendations	16

Numbering and Addressing

in

Machine-to-Machine (M2M) Communications

# INTRODUCTION

In recent years a rapid development of a variety of services that make use of Machine-to-Machine (M2M) communication has taken place. M2M is a communication technology where data can be transferred in an automated way with little or no human interaction between devices and applications. The advantages of M2M are great for both business and consumer purposes. For business use, M2M technology may lead to more effective and efficient operations (e.g. fleet control). For consumers, as an example, applications used for home security and smart metering can be facilitated by M2M.

It is expected that the development of M2M applications will have an impact on national numbering plans because machines need to be uniquely identified and addressed in order to communicate with them, or rather to enable them to communicate with each other. The natural intention by operators and M2M SPs is to use E.164 numbers from the existing numbering plan for M2M services because of the relatively simple implementation in already existing network infrastructure.

The potential amount of M2M devices could be large and some applications may need several numbers (e.g. E.164 numbers) or addresses (e.g. IPv6 addresses), and therefore there is a potential need for a large amount of identifiers. It is recognized that some national numbering plans may not be prepared for such big demand for numbers. Thus a numbering strategy is required for both short and long term to meet requirements for M2M services.

# Scope

The purpose of this report is to help National Regulatory Authorities (NRA) in their considerations on efficient numbering and addressing solutions for M2M applications and to avoid possible exhaustion of existing numbering ranges. The information in this report might be useful as policies on M2M numbering in most countries are still in an early stage of development and information on best practices does not exist, yet.

The essence of the communications that is subject of this report is that there is no need for the numbering to provide any aspect of “human user friendliness” (e.g. that numbers are kept as short as possible) whereas in traditional approach to numbering “human user friendliness” is a major objective of the design of a numbering plan. Furthermore, in most cases M2M communications are understood as a fully or largely automated communication (information transfer) between two or more information and communications (ICT) entities that may be part of a predetermined group.

Machine related communications also includes Human to Machine (H2M) and Machine to Human (M2H) communications where the “Human” is a single person or a group of predetermined people or any person who happens to be associated with the terminal that is identified by the number (e.g. anyone in a given car that has a built-in terminal for data or speech communication).

The report discusses the basic M2M characteristics, different access methods from M2M devices to networks and numbering and addressing alternatives for M2M applications. Finally, a set of recommendations for the NRAs regarding M2M communications are given.

# General Description of M2M Communications

The M2M applications can be either for fixed or moving locations. However, there is a tendency that most M2M SPs deploy their business models on mobile infrastructure even for fixed locations. Some examples of mobile location applications are car pool monitoring, person monitoring, logistical supply chain control or automatic remote paying systems. Fixed location applications are for example remote surveillance and monitoring of buildings and homes, etc.

## Parties of M2M Communications

M2M communications are a fully automated means of communication between two or more entities. Human interaction is not foreseen in this communication process. Parties in M2M communications are typically

- a M2M SP;

- an operator; and

- an end-user (the subscriber to a M2M service) as a M2M SP’s customer.

Although M2M applications vary very much from each other, today E.164 numbers for a M2M application are typically assigned to the M2M SP by the operator. Often the end-users do not see and even do not need to see these numbers. The end-user typically buys a certain M2M application (e.g. home supervision and alarm system) including the necessary equipment and telecommunication installations and connecting lines from a M2M SP. The M2M SP normally supervises the connections from the M2M equipment in customer’s premises to his service centre, and thus the M2M application is not dependent on the end-user’s choices on his private communications means.

Figure 1 illustrates M2M communications and communication parties with examples A and B. The example A could be automatic meter reading service, where an energy meter includes a communication terminal () able to receive a communication from the energy company () requesting a meter reading to be sent back. The example B could be a car theft alarm, where a GPS system installed in the vehicle () transfers data to the vehicle’s GSM connection () to send the vehicle’s identification and position to the vehicle monitoring service provider (). These examples are clarified in tables below figure 1.

Figure 1: Examples of M2M communications

Example A (figure 1): M2M communications using fixed line connections

| Item | Explanation |
| --- | --- |
|  | Subscriber (end-user) to a M2M service with a M2M service terminal, for example: - supervision of vending machines - collection of data from office/industry automation equipment - smart metering solutions, e.g. electricity metering |
|  | M2M SP |
|  | Subscriber’s M2M communication line, typically controlled by the M2M SP, provided by an operator. Subscriber’s public communication line, e.g. a fixed telephone subscriber line is separate from the M2M communication line. |
|  | Operator’s switch or concentrator. |
|  | M2M SP’s communication line to the operator. This may also be used for the M2M SP’s other communications means. |

Example B (figure 1): M2M communications using mobile connections

| Item | Explanation |
| --- | --- |
|  | Subscriber to a M2M service with a M2M service terminal, for example: - collection of information from a bicycle messenger’s service terminal - vehicle alarm systems - machine park rental supervision |
|  | M2M SP |
|  | Wireless communication from the subscriber’s M2M service terminal typically controlled by the M2M SP, provided by an operator. |
|  | Operator’s base station. |
|  | M2M SP’s communication line to the operator. This may also be used for the M2M SP’s other communications means. |

## Access from the M2M Devices to Network

The choice of M2M communications depends much on the application itself, but also the access method from the end-user’s device to network. The following approach applies:

|  | Fixed location | Mobile (moving) location |
| --- | --- | --- |
| Access method | - Fixed access: PSTN / IP - Mobile access: GSM / 3G (IP) | - Mobile access: GSM / 3G (IP) |

Typically, the M2M SP wants to provide the full service package to the end-user, including the telecommunication connection, which in turn has been negotiated by the M2M SP with an operator. In this way the M2M SP is able to control his service portfolio in whole.

### Fixed Access

Fixed access from the end-user to the M2M SP requires, quite naturally, a physical connection including “the last mile” of the subscriber line. Typically the existing subscriber line is used for the subscriber’s other telecommunication purposes, such as PSTN line and/or broadband. This connection may be affected if the end-user resigns his fixed connections, and this may also affect the possible M2M communication connections. Thus the M2M SP would, in practice, be required to build his specialised M2M subscriber line in addition to the subscriber’s private line. This is one of the reasons why M2M SPs prefer mobile access from their customers’ devices.

A PSTN connection is reliable and cost-efficient. However, the PSTN connection to a M2M device limits, in most cases, the device to be wired to the PSTN, which may result to vulnerable implementations, as the wires may be easily cut. Although wireless implementations also exist, at least the PSTN is connected to the premises by wire.

IP connection is also reliable and cost-efficient. Here, too, we talk about wired solutions in most cases. Although wireless implementations also exist, at least the broadband is connected to the premises by wire.

Both of the fixed access methods would allow large volumes to be transferred with relatively low cost.

### Mobile Access

Mobile access is provided by wireless connection from the device to the base station providing flexibility, security and increased safety in installation. The clear advantage for mobile access from the M2M SP’s point of view is that it is available for both fixed and moving customers. This allows the M2M SP to implement only one platform (i.e. the mobile) to serve all customers.

### Conclusion on Access

Mobile access gives flexibility, security and allows the M2M SP to implement only one service platform. Mobile technology is mature and more flexible than fixed networks. Typically the mobile access is implemented with a SIM card module in the subscriber’s M2M device allowing an already secured connection between the terminal and a base station. Mobile technology is rather inexpensive and allows easy installations.

As a conclusion the mobile access from M2M devices is the most viable access method.

# Assessment of Numbering Capacity Required for M2M Communications

## Methodology

An assessment of E.164 numbering capacity required for M2M communications depends on various factors, such as type of services and potential customer base. A strong growth is expected for M2M communication in the future. There is only little reliable information on the existing market situation for M2M and its growth rates. Thus it is very difficult to make valid assumptions. Furthermore, those few sources that exist measure market in rather different ways.

As different M2M applications cannot be easily compared with each other in terms of assessing the future demand of numbering capacity, one alternative is to take a number of inhabitants as a starting point. In European countries the number of inhabitants is rather stable throughout recent (and forthcoming) years. In the following chapters different growth estimations and scenarios for M2M communications are given, based on this starting point. It must be understood that the estimates given do not necessarily take account of all future machine related applications. However, the growing use of SIM card technology implies that in the near future additional consumer products will be connected to mobile networks. So there is a substantial possibility that real market demand for numbers for mass applications could become far higher than the given estimates.

First, an assumption on the amount of M2M numbers today (year 2010) was assessed. In order to make figures comparable, this assumption was made as numbers per capita. Secondly, information from different sources was collected to estimate the amount of M2M numbers in year 2020, different growth rates were assessed. Finally, the information was used to estimate the annual average growth rate for M2M numbers from year 2010 to 2020.

Based on information collected from European (CEPT) countries the above mentioned information was used to assess how existing (mobile) number ranges would meet the requirements for M2M numbering capacity in year 2020.

It has to be underlined that the assumptions are not based on scientific facts, but rather estimates on markets by different players.

## Assumptions and Scenarios

### Starting Points

The following starting points and assumptions deal with assessing the required M2M numbering capacity:

| 1. | These assumptions deal with mobile number ranges as concluded in chapter 5.1 - Operators and M2M SPs see mobile numbers as very important for addressing M2M applications in short and medium term. |
| --- | --- |
| 2. | Time window for M2M numbering assessment is from year 2010 to 2020 - Various sources indicate that IPv6 may take over in addressing M2M applications. This would be likely to happen earliest in year 2020. - In emerging business the market players would like to see regulatory stability for the business for some time. 10 years have been mentioned. |
| 3. | M2M numbers in use per capita in year 2010 is 0,1-0,2; figure 0,2 is used as a starting point - The figure is based on information from a few operators. The figure is considered as rather optimistic but it was decided as a starting point by the PT FNI. |
| 4. | A hypothetical country of 10 million inhabitants is chosen as an example - Actually, the number of inhabitants does not play a role here, it is used as a relative indicator to be able to compare countries in a reliable way. |
| 5. | Annual growth rate of M2M applications is 10% - 30% - It is very difficult to get estimations for M2M growth rate. Annual rates of more than 30% annually have been mentioned. - It may be possible that the growth is more intense in the beginning of the assessment window (2010-2020) and will slow down towards the end. This has been taken into account in figure 2. |

### Growth Assumptions Based on Population

The figure 2 illustrates the estimated growth of M2M numbers. The starting point is a hypothetical country with 10 million inhabitants where there are either 0,1 or 0,2 M2M numbers in use per capita in year 2010. Growth rates between 10% and 30% have been assessed with possibility of decreasing the growth with time (the growth rate colours are just for information purposes). The following assumptions are considered:

| Alternatives (see figure 2) | M2M numbers per capita 2010 | M2M numbers 2010 | Growth rate 2011 – 2015 | Growth rate 2016 – 2018 | Growth rate 2019 – 2020 |
| --- | --- | --- | --- | --- | --- |
| Alternative 1 | 0,1 | 1.000.000 | 10 % | 10 % | 10 % |
| Alternative 2 | 0,2 | 2.000.000 | 10 % | 10 % | 10 % |
| Alternative 3 | 0,1 | 1.000.000 | 30 % | 20 % | 10 % |
| Alternative 4 | 0,1 | 1.000.000 | 30 % | 30 % | 30 % |
| Alternative 5 | 0,2 | 2.000.000 | 30 % | 20 % | 10 % |
| Alternative 6 | 0,2 | 2.000.000 | 30 % | 30 % | 30 % |

Figure 2: Estimated growth of M2M numbers

There are very little or no studies to support different scenarios. The table 1 below gives number information (estimation of M2M numbers needed) from years 2015 and 2020 from the graph in figure 2.

Table 1. M2M numbers needed (in millions) according to figure 2

| Year | Alternative 1 | Alternative 2 | Alternative 3 | Alternative 4 | Alternative 5 | Alternative 6 |
| --- | --- | --- | --- | --- | --- | --- |
| 2015 | 1,6 | 3,2 | 3,7 | 3,7 | 7,4 | 7,4 |
| 2020 | 2,6 | 5,1 | 7,8 | 13,8 | 15,5 | 27,6 |

As stated in chapter 4.2 a starting point for numbers per capita in year 2010 is 0,2. The alternative 5 starts with 0,2 numbers in year 2010 per capita and the annual growth rate would be 30% in years 2011-2015, 20% in years 2016-2018 and 10% in years 2019-2020 giving a demand of 15,5 million (new) numbers for M2M applications only in year 2020.

### Growth Assumptions Based on Households

This estimation is based on a hypothetical country of 10 million inhabitants in year 2020. Some studies indicate that the average number of persons per household in developed countries varies from 2,2 (Switzerland 2008) and 2,4 (European Union 2007) to 2,6 (USA 2007). The following assumptions are made:

- 4 million households

- 75% of the households have 3 smart metering systems using M2M applications (electricity, water, heating)

- 25% of the households have a security system using M2M

- 1 car in a household; 50 % of cars include 2 M2M applications (eCall, theft alarm)

Numbering capacity required for the above mentioned example:

- Smart metering systems: 0,75 * 4 * 106 * 3 = 9 million

- Security systems: 0,25 * 4 * 106 = 1 million

- Car systems: 0,5 * 4 * 106 * 2 = 4 million

- Grand total: 14 million

### Other Growth Assumptions

Various M2M estimates are listed here mainly for background information:

- A telecom analyst firm has estimated an increase of 32,9 % machine-connections to mobile networks in Europe every year – giving 52 million units in 2011. The firm estimates that much of this growth will come from the car industry.

- Conclusions from some M2M presentations suggest that within next 10 years, i.e. up to year 2020, 1-2 M2M related numbers are needed per inhabitant.

- A M2M SP estimated in May 2009 that the Netherlands (16,5 million inhabitants) would require 25 million M2M numbers within 10 years.

- A telecommunications operator has estimated in May 2009 that France (63 million inhabitants) would have 5 million M2M numbers in use in 2010 and would require 30 million numbers in 2015 and more than 70 million numbers in 2020.

## Conclusions on Required M2M Numbering Capacity

This chapter draws conclusions on required M2M numbering capacity based on estimates given above. Different approaches have been ‘harmonized’ by calculating a factor with which the number of inhabitants needs to be multiplied in order to get an estimate on required M2M numbering capacity in year 2020. It has to be noted that all the figures here are very rough estimates.

- A population based approach gives factors 0,3 – 2,8 depending on the alternative in question. As the alternative 5 was assessed as one of the most realistic ones the factor is likely to be 1,6.

- A household based approach gives a factor value 1,4.

- Other growth assumptions give factor values between 1,1 and 2,0.

Although this is not a statistical approach, the above mentioned assumptions give the factor a typical value of 1,4. This factor means that in year 2020 a country needs M2M numbers equal to the number of inhabitants multiplied by 1,4. For example, in a country of 10 million people 14 million M2M numbers are needed in year 2020. When the (starting) level in amount of M2M numbers in 2010 is 0,2 per capita this gives an average annual growth of 20% in 2010-2020.

It must be understood that the above mentioned figures do not incorporate all possible future machine related applications. However, the growing use of SIM card technology implies that in the near future additional consumer products will be connected to mobile networks. So there is a substantial possibility that real market demand for numbers for mass applications could become far higher than the given estimates.

# M2M Numbering and Addressing

## Introduction to M2M Numbering

Numbering and addressing alternatives for M2M applications deal quite much with the chosen access method. Fixed and mobile access for M2M applications has been discussed in chapter 3.2. Today fixed access can be divided into PSTN and IP-based networks.

For PSTN access the E.164 numbering resources are obvious. For IP-based fixed networks the IP-addresses are used either in IPv4 or IPv6 format. The former is close to exhaustion while the latter is not yet mature technology in all aspects.

Although IP-connection with IP-addresses would be possible with 3G/UMTS mobile networks in the future, today’s mobile networks are not capable to support IP-based mobile access, i.e. it is not possible to set up communications using IP-addresses. Therefore, for mobile access E.164 numbers are the natural choice for the time being.

It can be concluded that the E.164 numbering resources (i.e. numbers in the national numbering plan) are the most viable solution for addressing M2M application at least in the short and medium run. It is expected that most M2M applications will be based on mobile networks, and therefore within E.164 numbers the present mobile number ranges seem to be most suitable for M2M solutions as described earlier. It is possible that IP-based solutions with IPv6 addressing will become more important in the long run.

## Numbering Resources for M2M Applications

It was concluded above that E.164 mobile numbering resources would be the most viable methods for addressing M2M applications. The basic options are as follows (option A is based on mobile infrastructure, while options B, C and D are independent of underlying network technology):

| - Option A: | Existing mobile number ranges, including possible expansion of them (E.164 numbers) |
| --- | --- |
| - Option B: | A new number range for M2M or similar applications (E.164 numbers) (for example longer numbers than normally, however max 15 digits according to E.164) |
| - Option C: | An international numbering solution (E.164 numbers) |
| - Option D: | Network internal numbers |

For the purpose of this report the following definitions shall apply:

| - Network external numbers | E.164 numbers with maximum length of 15 digits or national-only numbers in the National Numbering Plan. E.164 numbers may pass the international network-network interface whereas national-only numbers may only pass national network-network interfaces. National-only numbers do not belong to the international E.164 numbering plan nor do they conform to the structure of international E.164 numbers as defined in ITU-T Recommendation E.164. |
| --- | --- |
| - Network internal numbers (intra-network numbers) | Network internal numbers are meant to be used within one network only, i.e. they do not pass network-network interfaces. Network internal numbers are not part of the international E.164 numbering plan, or they do not need to be E.164 numbers or national-only numbers, but they may belong to the national numbering plan. However, in order to avoid conflicts between the use of network internal and network external numbers, it may be necessary to reserve a specific number range within the national numbering plan to allow the use of network internal numbers. The number structure and length of these numbers may not be limited. A consequence of this is, however, that it will not be possible at a later stage to use these numbers between networks. From their nature these numbers are not portable, which might limit competition between operators, and potentially also between M2M SPs. |

There are possible situations where a new number range could be opened if the number range in question may require different regulatory treatment, e.g. tariff transparency, access to emergency services, etc.

Another aspect is that the services to be provided have certain characteristics (e.g. M2M applications in fixed networks) where existing mobile number ranges may not be adequate, for example a home alarm system connected to the fixed network cannot use a mobile number in order to comply with the existing numbering plan.

The following basic possibilities exist:

- Communication remains within the same network (on-net)

- Communication may pass network-network interface, either national or international (off-net)

In on-net communications the communication process remains within one operator’s or service provider’s network, i.e. the M2M end-user and SP are in the same network. In off-net communications the network-network interface between two or more networks may be passed.

Numbering resources used between two or more networks (network external numbers) can be used for both on-net and off-net applications. Although network internal numbers do not pass network-network interfaces, they may be used for off-net applications (thus facilitating traffic between networks) as the numbers do not have to be diallable by the broad public. Such use of network internal numbers is.based on bilateral agreement between operators of (mobile) networks and M2M SPs for the connection of M2M data networks to public (mobile) networks.

Table 2: Characteristics of network external and network internal numbers

| Network external numbers – Option A-C | Network internal numbers – Option D |
| --- | --- |
| - Comply with ITU-T Rec. E.164 - Telecommunications regulations apply - Numbers are E.164 numbers with maximum length of 15 digits - Number portability may be applicable - Allows less new capacity than on-net solution - Flexibility in choosing the operator by the M2M SP, although the physical SIM card may remain another threshold - International traffic is possible under normal circumstances (needs no special routing agreements) - Interconnection is possible | - Not strictly regulated in many countries; decisions and management up to the operator - Numbers are typically not E.164 numbers - Allows long numbers with much capacity – even longer than 15 digits numbers are possible - No need for determining number length; older networks may gradually change to longer numbers - Allows use of hexadecimal digits - Same numbers can be used in every network allowing multiplied capacity - Number portability is – in practice – not possible - M2M SP is – in practice – locked with one operator → possible competition issues - Difficult or impossible to evolve to ‘network external’ mode if required for some reason - International traffic only possible with special routing agreements - Interconnection is not an issue |

Table 3 lists some characteristics between optional numbering ranges including on-net and off-net alternatives.

Table 3: Characteristics of number ranges

| Option A Existing mobile number ranges | Option B New number range | Option C International number range | Option D Network internal numbers |
| --- | --- | --- | --- |
| - Number ranges may be quite full already - Possible challenges in number analysis if different number length is used for M2M from traditional mobile users - Little or no possibilities to treat M2M numbers differently from regulatory point of view - May not allow separate back-office solutions for M2M applications | - Full capacity of numbers of the new range is available - Large blocks available for each operator - A fresh start for number analysis - A fresh start for possibly different regulatory requirements - May allow easier back-office solutions, such as charging and billing | - Number range needs to be assigned by the ITU and the applicant needs to be qualified - International number, i.e. dialling starts with the international prefix even though the communication would stay in own country (not an issue for M2M) - Challenges in number analysis and effective routing - May need to be treated in the same way as other international traffic | - See the table 2 above (network internal numbers) - In many countries this is not regulated, i.e. an operator may start to use network internal numbers without needing any permissions |

According to the ITU requirements option C can only be implemented for transnational services (at least two countries), and as such cannot be considered as the solution to solve the numbering capacity issue for M2M services deployed within a national environment.

The most challenging characteristics of existing mobile number ranges for M2M applications is that they may not have capacity enough to accommodate potentially very large amount of numbers required by all new M2M applications. An international number range may be viable, but has disadvantages such as it is not fully in national control and requires international dialling. Network internal numbers would give a huge capacity of resources for M2M applications, but have challenges as listed in table 2.

In case of number scarcity the practical alternatives that remain for M2M numbering in the short and medium term are to open up a new number range and/or to use network internal numbering.

## Adequacy of National Mobile Number Ranges for M2M Services

As network operators have a strong preference for the use of mobile number ranges for M2M applications, it is necessary to understand how the mobile number ranges today are used. The CEPT Working Group Numbering, Naming and Addressing carried out in November 2009 – January 2010 a questionnaire on usage of national mobile number ranges amongst CEPT countries.

Responses from 29 countries were received. As countries have different ways to collect statistics on number usage not all information was available for all countries. Assessing the capacity for mobile numbers is also especially difficult in countries with variable number length. Annex 1 lists the detailed replies and analysis.

The available mobile number capacity in a country depends, quite naturally, on the population. Thus it does not make much sense to compare absolute figures of mobile number capacity in a country. The figures given here are averages amongst 29 countries. Mobile number capacity forms 18% out of the total availability in a numbering plan, and the mobile number capacity is 18 numbers per capita. Half of the mobile number capacity has been assigned to operators, thus giving another half to be free. 24% out of the mobile number assignment is in operation. National mobile number penetration is 143%.

Taken the basic assumptions into account as discussed earlier in this chapter, the 29 countries were assessed based on population, free mobile number capacity and annual growth rate of M2M numbers. The assessment revealed that 7 countries do not have adequate existing mobile number capacity to accommodate the estimated growth of M2M services and the amount of numbers they require. The number of countries where mobile number ranges will not be adequate may easily become higher because there is a substantial possibility that real market demand becomes far higher than the given estimates.

## Strategy for M2M Numbering and Addressing

From chapter 4 of this report it can be concluded that the annual growth rate in M2M applications between years 2010 and 2020 is about 20%. This chapter revealed that 8 European countries do not have adequate mobile number capacity available to accommodate estimated growth in M2M applications.

With the fast development of M2M applications, countries are challenged to accommodate the demand for numbers by using numbers from the national numbering plans. It is likely that in the long run IP-based solutions, such as IPv6, may become viable as alternative numbering resource for at least part of the M2M applications. It is, however, uncertain how long a period a numbering policy for M2M applications should cover, before IP-based solutions take over.

One of the main goals of numbering policy for M2M applications is that the provision of numbers for these applications will not compromise specific end-user interests which are covered by the existing national numbering plans, such as user-friendliness and tariff transparency. A new number range for M2M communications or other device-based applications with possibly different regulatory requirements would give large numbering capacity and would lower the risk that end-user interests are negatively affected.

The NRAs should introduce a numbering policy for M2M communications. Such policy could include reserving a separate numbering range for M2M communications or similar device-based applications in the national numbering plan if the existing number ranges do not have sufficient unallocated capacity to support the potential demand for M2M applications. In addition, there are possible situations where a new number range could be opened if the number range in question may require different regulatory treatment, e.g. tariff transparency, access to emergency services, etc. A new number range would also allow flexibility to accommodate possible future applications similar to M2M communications into this new number range.

When considering which form of numbering range to use, countries should take into consideration which sort of range (existing or new, on-net or off-net) would be most applicable and what possible consequences this might have on competition in the whole M2M business chain.

A harmonised approach regarding M2M numbering, i.e. the same M2M numbering range with the same rules in every country would be very difficult because of the difference in national numbering plans and approaches the cost of harmonising numbering policies exceeds benefits perceived. On the other hand, in the report the basic principles and approaches of M2M numbering are harmonised.

# IMPLEMENTATION ASPECTS OF A NEW NUMBER RANGE

In this chapter some aspects of the creation of a new numbering range are elaborated.

## Network External and Network Internal Numbers

As set out in chapter 5, two types of numbers, network external numbers and network internal numbers, are candidates for M2M numbering resources. The appropriate type of numbers depends on the type of application and market conditions for the application concerned. From the viewpoint of an efficient use of numbers, network internal numbers should be preferred but this should not compromise the market position of the users of the M2M applications. Therefore the use of separate number ranges for network external numbers and network internal numbers is a valid option.

As there is no need to pre-determine the number structure and length of network internal numbers, an option is to create a numbering range for these numbers without conditions related to a number length. This may assist networks that have some difficulties with routing long numbers, in taking a gradual path, starting with using shorter numbers but stepping over to longer numbers when this is technically feasible for these networks.

As number portability can not be applied to network internal numbers, the users of these numbers should be aware that it may be more difficult to change their services from one operator to another.

## Installed Base of M2M Applications

Many countries, which already have assigned numbers from existing number ranges for M2M applications, are considering opening a new number range to accommodate future numbering requirements in M2M business. Typically these existing M2M applications have mobile numbers because of commercial reasons. It will be a national matter whether existing M2M numbers should be moved to a new number range or not. However, the following factors are important:

- the level to which numbers used for current M2M applications forms an additional risk that in a later stage number scarcity in the number range concerned is to be expected;

- applying an adequate period for phasing out of such number use may reduce to a large extent the negative effects for operators and M2M SPs;

- the time the market needs for commercially setting up the new numbering range, so that this number range has no disadvantages in comparison to the (mobile) numbers currently used;

- possible renumbering of existing devices may have commercial barriers.

## Assignment Mechanism

There are no strong reasons to apply specific assignment principles to the new numbering range which are different from those most countries apply to geographic numbers and mobile numbers (secondary number assignment via operators would still apply). The interests of users of M2M applications such as large companies regarding the numbers used for these applications are not stronger than in the case of geographic or mobile numbers. Given the availability of number portability where this is appropriate (for network external numbers), assignment of numbers from a new number range to operator would be the most appropriate option as this may be easier for operator to implement (block routing).

## Use of Long Numbers

The number length of network external numbers should be as long as possible, however maximum of 15 digits according to ITU-T Rec. E.164. The length of network internal numbers should not be predetermined.

The use of long numbers in networks with older technologies may imply that the impact on such networks may be larger than on more modern (mobile) networks, especially for network external routing. This may affect the way in which long numbers are introduced in the new numbering range.

# Regulatory requirements

As a starting point, existing regulatory requirements should be in force for M2M communications. There are impacts to regulatory requirements based on the choice between the use of network external or network internal numbers.

It is possible that the use of network internal numbers will require a regulatory decision, as in principle it will violate or even not make it possible to apply certain existing regulatory requirements. On the other hand, the use of such numbers may be regarded as operator internal business, as it does not influence or affect any other operator. The latter is however not a strong argument because network internal numbers can also be used to route traffic from or to other networks based on bilateral agreements between operators, and some regulatory issues may also be relevant for on-net traffic based on national regulations.

Table 4: Impact of the choice between network external and

network internal numbers to regulatory requirements

| Regulatory measure | Network external numbers | Network internal numbers | Notes |
| --- | --- | --- | --- |
| Interconnection / Interoperability | Valid | May not be applicable | If interoperability issues are valid, then termination rate issues are valid, too, as is the way to charge interconnection. |
| Termination rates | Valid | N/A |  |
| Tariff transparency | N/A | N/A | End-users may use M2M and similar applications where network connections are started without or with only limited human interaction. Therefore, if tariffs are call or time dependent, there would be a clear need for tariff transparency. However, it is expected that for most M2M applications the tariff structure for end-users will be flat fee. This also holds for wholesale tariffs. In this case no additional measures need to be taken to ensure adequate tariff transparency for a new numbering range. This policy may change when there are signs that also other tariff structures will be applied. |
| Number portability | Valid | N/A | Number portability in M2M environment is less relevant than in traditional voice telephony world as it is easier to change numbers. |
| Emergency calls | Not an issue with most applications, relevant with e-Call | Not an issue with most applications, might be relevant with certain applications | Although the situation is not totally clear, it is not likely that numbers for M2M applications should be subject to conditions for emergency calls. |
| Legal interception | May be valid | May be valid |  |
| Directory aspects | N/A | N/A | There is no need for the wide public to know M2M numbers. |
| Data retention | May be valid | May be valid | Might be applicable according to Directive 2006/24/EC |
| Calling Line Identification | Valid | N/A | CLI may be used to identify the M2M application. |

The financial aspects of M2M applications, typically low revenues per number and high volume of numbers per application, should be taken into account if administrative fees exceed costs of managing the numbering plan when NRAs define the administrative numbering fees.

# Conclusions and Recommendations

With the fast development of M2M applications, countries are challenged to accommodate the demand for numbers by using numbers from the national numbering plans. The numbering options B-D are independent of underlying network technology, while option A is based on mobile networks.

The conclusion based on analysis from this report is as follows:

- The expected annual growth rate of required M2M numbers between years 2010 and 2020 is approximately 20%, but this is just a rough estimate and the actual figure may differ from this estimation significantly.

- In the long run IPv6 addressing will become very important as alternative numbering resource for at least the major part of the M2M applications. However, there is much uncertainty what period is involved so that numbering policy for M2M should be flexible enough to be also a solution for the longer term.

- A significant number of CEPT countries do not have sufficient capacity in their existing numbering plan to accommodate numbering requirements for M2M applications.

- A fully harmonised approach on possible M2M numbering solutions is not needed in Europe.

One of the main goals of numbering policy for M2M applications is that the provision of numbers for this kind of applications will not compromise specific end-user interests which are covered by the existing national numbering plans, such as user-friendliness and tariff transparency.

The report recommends as follows:

- Taken into account the potential extensive growth rates of M2M applications or other similar telecommunication services, the NRAs should, in cooperation with market players, establish a numbering policy for these kinds of services as part of the national numbering plan. The policy shall be flexible enough to accommodate also a longer term solution.

- As a long term solution IPv6 addresses, or numbers/addresses other than E.164 numbers should preferably be used for device based communication applications. These numbering/addressing schemes or switching from E.164 numbering plan to a new plan should not prohibit market development or competition.

- In a short and a medium term the 4 numbering options (A-D) can be envisaged; based on the current knowledge of the market none of these have a clear preference. In cases where there is a need to distinguish M2M communications and the traditional mobile services option A has to be excluded. Options B and D are needed if the growth of M2M is larger than expected or if there is no adequate space in existing number ranges in use. Option C depends on ITU.

- There are possible situations where a new number range (option B) should be opened. For example, the number range in question may require different regulatory treatment, e.g. relating access to emergency services, or the services to be provided have certain characteristics (e.g. M2M applications in fixed networks) where existing mobile number ranges may not be adequate.

- For the services where number portability is an obligation or where subscribers can take advantage on number portability, option D should not be used.

Additional considerations regarding possible new number range(s) for M2M applications or other device based communication applications:

- The new numbering range(s) accommodate future mass volume applications, such as M2M applications, and do not have prohibitive conditions for such applications.

- The NRA should ensure that the new numbering range(s) are not used as an alternative to existing numbering ranges to escape regulatory requirements.

- As some existing regulatory requirements may not be relevant or useful for certain applications such as M2M applications, exceptions regarding existing regulatory requirements could be applied to new numbering range(s) accommodating these applications. For example, for M2M numbering resources the issue of user friendliness (e.g. short numbers) is not relevant. Number portability requirements should be applied to network external numbers but not to network internal numbers, as this is not applicable in the latter case.

- The number length of network external numbers should be as long as possible (max 15 digits according to ITU-T Rec. E.164). The length of network internal numbers should not be predetermined.

Annex 1: Mobile Number Usage in Europe

The Annex 1 lists the results of the questionnaire on mobile number usage in Europe carried out by the Working Group Numbering, Naming and Addressing in November 2009 – January 2010.

The population amount is taken from the World Factbook as status of July 2007 in order to have the figures comparable with each other. The questions asked were

- Mobile number capacity out of the whole numbering plan

- Amount of mobile numbers assigned to the operators

- Amount of mobile numbers in operation

In Analysis columns the following has been calculated:

- Mobile numbers in operation per capacity (%)

- National mobile number penetration: numbers in operation divided by population (%)

- Mobile number capacity per capita: capacity divided by population

- Free mobile number capacity: capacity minus allocation

- Free mobile number capacity as percentage out of capacity (%)

Requirement of M2M numbers in year 2020 has been calculated based on the following assumptions:

- The starting point is 0,2 M2M numbers per capita in year 2010

- The annual growth rate of required M2M numbers is 20%

The assessed amount of M2M numbers in year 2020 has been compared with the free mobile number capacity (in year 2010).

Countries that would have problems in this assessment to accommodate M2M applications within their existing mobile number ranges are:

- Denmark

- France

- Lithuania

- The Netherlands

- Poland

- Spain

- Sweden

Additional notes:

- Norway: Norway has opened a new number range for M2M applications and thus they do not have problems with numbering space.

- Sweden: Since 2005 there are two separate NDCs (one for fixed networks and one for mobile networks) for Telematic services (M2M communications) in the Swedish National Numbering Plan.

- Luxembourg and Romania should pay close attention to their M2M growth as the estimated required amount of M2M numbers exceed 50% of the free capacity.

Annex 2: Description of M2M Applications

This annex describes M2M applications based on various factors given in table 5 (the list of applications is not exhaustive). The purpose of this annex is to give additional information to the reader on possible M2M applications and their usage.

Table 5: A model for assessing M2M applications

| Name of the Application | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | <fixed / nomadic / moving> | <e.g. H2M aspects> | <is cross border traffic valid> |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | <point-to-point / point-to-multipoint / broadcasting> | <is the service open to all, or e.g. subscription based> |  |
| Description of the Application | Networks involved? | Type of traffic? |  |
|  | <network internal / cross network> | <volume, bursting / continuous> |  |
|  | Network types involved? | Terminal control? |  |
|  | <fixed / mobile / IP based> | <who controls the M2M terminals, e.g. M2M SP> |  |

- Monitoring and Smart Metering

| Monitoring and Smart Metering | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Fixed | No user interaction – usually metering applications are sealed to prohibit “fixing”. | Typically within one country |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Two way communication – the provider sends a request and the application replies with the metering value | A predetermined group of users |  |
| Automatic Meter Reading (AMR) can be used to monitor electricity, water, windmills and gas meters. Utility providers within the electricity sector have rolled out SIM-based AMR-systems to consumers in Sweden and a major rollout is scheduled to take place in Norway within few years. The system offers a major improvement of efficiency and accuracy. | Networks involved? | Type of traffic? |  |
|  | On-net, i.e. mainly network internal | Small volumes a few times in a year |  |
|  | Network types involved? | Terminal control? |  |
|  | Mainly mobile | Terminals are controlled by the utility provider |  |

- Automatic Toll Systems and eCall

- Both GSM and GPRS/UMTS will probably be the preferred communication technology from an economic point of view

- application providers prefer network external numbers as this may facilitate competition between mobile telecom operators

- application falls within scope of mobile networks, mobile networks might be able to easily implement long numbers, so a range with long numbers may be the preferred solution here

| Automatic Toll Systems | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | A moving vehicle through a toll gate | No | National |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-point | A predetermined group of users | This application collects vehicle information by sending a radio frequency signal to vehicles driving through a toll gate. The vehicle’s tag responds to this signal with relevant vehicle information, e.g. by using RFID. This application does not require public numbering/addressing schemes. |
| An application to collect road tolls in an automated way. The system is based on a vehicle installed tag/short range identifier (e.g RFID) which communicates with the toll gate to identify the vehicle and register various parameters for the interaction. | Networks involved? | Type of traffic? |  |
|  | Network internal | Burst |  |
|  | Network types involved? | Terminal control? |  |
|  | Not relevant | By the toll system provider |  |

| eCall | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Moving | Automatic, no human interaction | National |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-Point | Open application, i.e. everybody with a relevant equipment may use eCall |  |
| eCall is a project of the European Commission intended to bring rapid assistance to motorists involved in a collision anywhere in the European Union. The projects aims to employ a hardware black box installed in vehicles that will wirelessly send airbag deployment and impact sensor information, as well as GPS coordinates to local emergency agencies to the general emergency number 112. | Networks involved? | Type of traffic? |  |
|  | Cross network | Burst |  |
|  | Network types involved? | Terminal control? |  |
|  | Mobile | ? |  |

- Positioning and Tracking

- M2M communication can be used in facilitating better control over moving objects such as cars, trailers, ships, containers, expensive medical equipment etc. GPS bracelets can be put on animals and humans.

- In the transport sector trucks are monitored by GPS and GSM-communication in the German system "Toll Collect". Positioning would make it easier to trace valuable items, like boats and cars, if stolen. Insurance companies might offer discounts for insurance policies for cars if such positioning systems are installed.

| Positioning and Tracking | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Moving | No | Both national and international traffic is likely |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-Point | A predetermined group of users |  |
| Collection of location information of moving targets. Tracking of the path of moving targets. | Networks involved? | Type of traffic? |  |
|  | Cross network | Burst |  |
|  | Network types involved? | Terminal control? |  |
|  | Mobile | By the service provider |  |

- Road Navigation

| Road Navigation | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | A moving target is receiving GPS satellite information. | Not valid | Not valid |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Broadcasting (signal from satellites to vehicle = one way) | Open | Public telecommunications networks are normally not involved in this application, therefore no public numbering resources are needed. Public broadcasting networks with RDS information may be involved to provide additional traffic information to users. |
| The actual road navigation system is nowadays typically based on Global Positioning System providing the user with map information and directions to a destination. | Networks involved? | Type of traffic? |  |
|  | Not valid | Continuous reception of satellite information |  |
|  | Network types involved? | Terminal control? |  |
|  | Satellite | Not valid |  |

- Machine Park Control

- Seem to have less implications for numbering as only fixed networks are used and network internal or service provider-specific numbering solutions exist

| Machine Park Control | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Fixed | No | Both |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-Point | A predetermined group of users |  |
| Automated status control of various machines of different kinds. | Networks involved? | Type of traffic? |  |
|  | Cross network | Burst |  |
|  | Network types involved? | Terminal control? |  |
|  | All | Status control service provider |  |

- ICT-entities in Vehicles

- Advanced ICT-entities and telematics in cars (“fleet managing”) represent an upcoming area of M2M communication. It implies communication of different types of information about car and traffic status. Car manufacturers are offering integrated solutions such as post-crash systems, eCall services, navigation services, traffic-information services and diagnostics about brake status, weight etc.

| ICT-entities in Vehicles | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Moving | Could be possible, but usually not | International |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-Point | A predetermined group of users |  |
| Advanced ICT-entities and telematics in cars (“fleet managing”) represent an upcoming area of M2M communication. It implies communication of different types of information about car and traffic status. Car manufacturers are offering integrated solutions such as post-crash systems, eCall services, navigation services, traffic-information services and diagnostics about brake status, weight etc. | Networks involved? | Type of traffic? |  |
|  | Cross network | Bursting |  |
|  | Network types involved? | Terminal control? |  |
|  | Mobile or IP based | Fleet management service provider |  |

- Security and Surveillance

- M2M communication is relevant for alarm systems, access controllers, mobility controllers, property surveillance systems, weather/climate surveillance systems, vermin traps etc. For example M2M communication over UMTS can be used to facilitate sending of live video from cameras in taxi cabs, busses and other mobile entities.

- The security features can be based on pure machine to machine communication, but there could also be elements of human interaction, e. g. a person sends an SMS to an SIM-equipped camera which returns MMS pictures or live video. Communication can also originate from an ICT-entity when an alarm is tripped, sending footage to an ordinary handheld terminal.

| Security Systems | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Fixed | Possibility for a manual alarm | Typically national |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-point | A predetermined group of users |  |
| Typical application would be a burglary alarm system in a building including door and window sensors as well as movement detectors. An alarm would be passed to the security center via public or private telecommunications networks. | Networks involved? | Type of traffic? |  |
|  | Cross network | Burst |  |
|  | Network types involved? | Terminal control? |  |
|  | Typically mobile, but other networks are also possible | By security system service provider |  |

| Surveillance Systems | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Fixed | Possibility for a manual alarm | Typically national |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-point | A predetermined group of users |  |
| This application may be connected together with the Security Systems (see above), but it may also have its own supervision system. Examples include heating control or over flood control. | Networks involved? | Type of traffic? |  |
|  | Cross network | Burst |  |
|  | Network types involved? | Terminal control? |  |
|  | Typically mobile but other networks are also possible | By surveillance system service provider |  |

- Point of sales

- M2M can be used to communicate with different payment terminals such as vending machines, credit card terminals, parking meters, gambling/lottery machines etc.

| Point of Sales | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Fixed | Not directly | Typically national, ATMs may require international communication |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-Point | A predetermined group of users |  |
| M2M can be used to communicate with different payment terminals such as vending machines, credit card terminals, parking meters, gambling/lottery machines etc. | Networks involved? | Type of traffic? |  |
|  | Cross network | Burst |  |
|  | Network types involved? | Terminal control? |  |
|  | Fixed or IP based | By point of sales provider |  |

- Remote medicine

- M2M can be used for life support systems equipped with a link to a high level entity via the Internet. Life support systems are medical equipments that substitute or augment an essential function of the human body, such as a heart pacemaker.

- Interactive telemedicine services (e.g. tele-surgery) provide real-time M2M-based interactions between automated system for surveillance of critical life parameters of the patient, during an remote operation.

| Life-support systems | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Mobile | Not directly | Typically national |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-point | A predetermined group of users | Life support systems are medical equipments that substitute or augment an essential function of the human body, such as a heart pacemaker. Application dealt with by the FP7 IST project LOLA (Achieving Low Latency in wireless communications) |
| The patient wears a local life supporting system, e.g., pacemaker. If the system detects abnormal readings that it is not able to process on its own, it will contact a central main server, which might offer more processing power or a bigger database, or will set an alarm and ask for assistance. | Networks involved? | Type of traffic? |  |
|  | Cross network | Burst |  |
|  | Network types involved? | Terminal control? |  |
|  | Mobile | By the medical service provider |  |

| Tele-surgery | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Fixed | Yes (surgeons, medical team) | National and international |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Point-to-point and multipoint-to-point (several surgeons at different locations participating in the same surgery) | A predetermined group of users (surgeons) | Application dealt with by the FP7 IST project LOLA (Achieving Low Latency in wireless communications) |
| M2M can be used for tele-surgery, also known as remote surgery, which is performed by a surgeon at a site away from the patient. Surgical tasks are directly performed by a robotic system controlled by the surgeon at the remote site. Sensor monitoring vital parameters of the patient are reported also to the control center in which the doctor is located and alarm or specific actions can be automatically activated if some abnormal situation occurs or the decision of the medical team is requested | Networks involved? | Type of traffic? |  |
|  | Cross network | Continuous reception of a video flow (high data rate, possibly in High Definition) and all vital constants of the patient at the surgeon side and continuous reception of commands for the robotic system at the patient side. |  |
|  | Network types involved? | Terminal control? |  |
|  | Typically fixed. | By the medical service provider |  |

- M2M games

- M2M communication can also be used for virtual games which take place contemporarily in different locations and in presence of mobility. The human body or sport vehicles are monitored by an M2M application (which may be distributed in different user equipments of located in a unique central server). The player’s changing physical parameters (heartbeat, speed, etc.) and position on the planet interact with the environment and with other players playing in the same or distant location. The games can work on, e.g., GPS-enabled phones that receive sensory data from sensors installed in the human body or sport vehicle and exchange this information and the exact position of the player with the game server or other players, e.g., via cellular connections. The range of location and mobile sensory games that can be supported is wide.

| M2M games | Point of origin? | User interaction? | National/international? |
| --- | --- | --- | --- |
|  | Mobile | Yes | National and international |
|  | Direction of communications? | Open application or a predetermined group of users? | Other remarks? |
|  | Depends on the game. Typically two-way communication | A predetermined group of users (players) | Application dealt with by the FP7 IST project LOLA (Achieving Low Latency in wireless communications) One of the many possible M2M games is the “virtual” bicycle race using real bicycles. The opponents are on different locations, possibly many kilometres away (hundreds or even thousands). At the beginning the corresponding “length” of the race is agreed (i.e. 10 km or 20 min). The measurements are taken by sensors (GPS, temperature, humidity, speed, etc.). Measurements are exchanged between the bikes. They are used to calculate the equivalent position of the participants and to show them the corresponding state of the race (e.g. “you are leading by 10 m”). A similar scenario can be used for racing (jogging) to compare the results using the motion detector. |
| M2M communication can also be used for virtual games which take place contemporarily in different locations and in presence of mobility.. The human body or sport vehicles are monitored by an M2M application. The player’s changing physical parameters and position interact with the environment and with other players playing in the same or distant location. The range of location and mobile sensory games that can be supported is wide. | Networks involved? | Type of traffic? |  |
|  | Cross network | Low data rate traffic. Burst or continuous transmission/reception, depending on the game settings. Frequency of transmissions typically increases as the end of the game approaches. |  |
|  | Network types involved? | Terminal control? |  |
|  | Mobile | Not valid. Note: there might be services provided by operators to support this type of games. |  |

- Other applications

- M2M communication can be used in communication with ICT-entities integrated in heaters and coolers, door locking systems, street lighting systems.

- New applications occur continuously.

- Consumer electronics devices.

These recommendations are given for the purpose of this report only, and they should not be mixed with ECC Recommendations (capital ‘R’) as regulative tools.

The definitions of H2M and M2H imply a margin of interpretation, for example machine initiated call set-up can be followed by voice communication.

PT FNI project team ”Future Numbering Issues” (meeting in Brussels in February 2010) under the CEPT Working Group Numbering, Naming and Addressing.

Federal Administration, Swiss Statistics – http://www.bfs.admin.ch/bfs/portal/en/index/themen/01/04/blank/key/haushaltsgroesse.html

Eurostat: Average number of persons per household – http://nui.epp.eurostat.ec.europa.eu/nui/show.do?dataset=lfst_hhantych&lang=en

U.S. Bureau of the Census, May 1996 – http://www.census.gov/population/projections/nation/hh-fam/table1n.txt

Estimation by PT FNI project team ”Future Numbering Issues” (meeting in Brussels in February 2010) under the CEPT Working Group Numbering, Naming and Addressing

Berg Insight: Fleet Management and Wireless M2M – 4th Edition (December 2008)

http://www.berginsight.com/ShowReport.aspx?m_m=3&id=79

Stratix, May 2009

France Telecom Group, May 2009

This was confirmed in the public consultation of this report by the respondents.

Term ”mobile number ranges” includes all existing network external number ranges which are allowed to use in mobile networks.

The expansion is restricted to the spare capacity of the total reserved number ranges for mobile services.

ITU-T Rec. E.101 Definitions of terms used for identifiers (names, numbers, addresses and other identifiers) for public telecommunication services and networks in the E-series Recommendations (11-2009)

Also E.164 numbers or national-only numbers can be used within one network only, i.e. as network internal numbers.

These recommendations are given for the purpose of this report only, and they should not be mixed with ECC Recommendations (capital ‘R’) as regulative tools.