RIG II Templates for the harmonised conditions for the use of the band 169.4 - 169.8125 MHz

Radio Interface Notification by an administration

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Not applicable | ITU‐R RR Article number 4.4 |
|  | 2 | Application | Aids for hearing impaired | Assistive Listening Devices (ALD) for hearing impaired people |
|  | 3 | Frequency bands | 169.400 - 169.475 MHz 169.4875 - 169.5875 MHz |  |
|  | 4 | Channelling | Not specified |  |
|  | 5 | Modulation / Occupied bandwidth | Max. 50 kHz |  |
|  | 6 | Direction / Separation | Not specified |  |
|  | 7 | Transmit power / Power density | 500 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Not specified |  |
|  | 9 | Authorisation regime | General authorisation | No registration and/or notification is required (commonly known as “licence exempt”). |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions | --- |  |
| Informative part | 12 | Planned changes | --- |  |
|  | 13 | Reference | EC Decision 2013/752/EU ECC Decision (05)02 EN 300 422-2 |  |
|  | 14 | Notification number | --- |  |
|  | 15 | Remarks | Definitions |  |
|  |  |  | Abbreviations e.r.p. …..Effective radiated power |  |

RIG II Templates for the harmonised conditions for the use of the band 169.4 - 169.8125 MHz

Radio Interface Notification by an administration

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Not applicable | ITU‐R RR Article number 4.4 |
|  | 2 | Application | Non-specific SRDs |  |
|  | 3 | Frequency bands | 169.400 - 169.475 MHz |  |
|  | 4 | Channelling | Not specified |  |
|  | 5 | Modulation / Occupied bandwidth | Max. 50 kHz |  |
|  | 6 | Direction / Separation | Not specified |  |
|  | 7 | Transmit power / Power density | 500 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle limit: 1.0% | Duty cycle controls (hardware or software) related to the duty cycle requirements shall not be accessible to the user. The duty cycle shall not be capable of being disabled or altered and shall be implemented as an automatic feature in the equipment. |
|  | 9 | Authorisation regime | General authorisation | No registration and/or notification is required (commonly known as “licence exempt”). |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions | --- |  |
| Informative part | 12 | Planned changes | --- |  |
|  | 13 | Reference | EC Decision 2013/752/EU ECC Decision (05)02 EN 300 220-2 |  |
|  | 14 | Notification number | --- |  |
|  | 15 | Remarks | Definitions |  |
|  |  |  | Abbreviations e.r.p. …..Effective radiated power |  |

RIG II Templates for the harmonised conditions for the use of the band 169.4 - 169.8125 MHz

Radio Interface Notification by an administration

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Not applicable | ITU‐R RR Article number 4.4 |
|  | 2 | Application | Non-specific SRDs |  |
|  | 3 | Frequency bands | 169.400 - 169.4875 MHz 169.4875 - 169.5875 MHz 169.5875 - 169.8125 MHz | Equipment that concentrates or multiplexes individual equipment is excluded in the frequency range 169.4875 - 169.5875 MHz |
|  | 4 | Channelling | Not specified |  |
|  | 5 | Modulation / Occupied bandwidth | Not specified |  |
|  | 6 | Direction / Separation | Not specified |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle limit: 0.1% In the frequency band 169.4875 - 169.5875 MHz, the duty cycle limit is 0.001%; a duty cycle limit of 0.1% can be used between 00:00h to 06:00h local time | Duty cycle controls (hardware or software) related to the duty cycle requirements shall not be accessible to the user. The duty cycle shall not be capable of being disabled or altered and shall be implemented as an automatic feature in the equipment. |
|  | 9 | Authorisation regime | General authorisation | No registration and/or notification is required (commonly known as “licence exempt”). |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions | --- |  |
| Informative part | 12 | Planned changes | --- |  |
|  | 13 | Reference | EC Decision 2013/752/EU ECC Decision (05)02 EN 300 220-2 |  |
|  | 14 | Notification number | --- |  |
|  | 15 | Remarks | Definitions |  |
|  |  |  | Abbreviations e.r.p. …..Effective radiated power |  |

RIG II Templates for the harmonised conditions for the use of the band 169.4 - 169.8125 MHz

Radio Interface Notification by an administration

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Not applicable | ITU‐R RR Article number 4.4 |
|  | 2 | Application | Meter Reading |  |
|  | 3 | Frequency bands | 169.400 - 169.475 MHz |  |
|  | 4 | Channelling | Not specified |  |
|  | 5 | Modulation / Occupied bandwidth | Max. 50 kHz |  |
|  | 6 | Direction / Separation | Not specified |  |
|  | 7 | Transmit power / Power density | 500 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle limit: 10% | Duty cycle controls (hardware or software) related to the duty cycle requirements shall not be accessible to the user. The duty cycle shall not be capable of being disabled or altered and shall be implemented as an automatic feature in the equipment. |
|  | 9 | Authorisation regime | General authorisation | No registration and/or notification is required (commonly known as “licence exempt”). |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions | --- |  |
| Informative part | 12 | Planned changes | --- |  |
|  | 13 | Reference | EC Decision 2013/752/EU ECC Decision (05)02 EN 300 220-2 |  |
|  | 14 | Notification number | --- |  |
|  | 15 | Remarks | Definitions |  |
|  |  |  | Abbreviations e.r.p. …..Effective radiated power |  |