Radio Interface Specification (RIS) implementation of ECC/DEC/(06)01 
on harmonise the use of the bands 1920-1980 MHz and 2110-2170 MHz for mobile/fixed communications networks (MFCN) including terrestrial IMT systems

Foreword

The ECC has decided that RIS implementations of ECC deliverables should be uploaded to the ECO website in order to help administrations fill out the EFIS database.

This RIS implementation is limited to harmonise the use of the bands 1920-1980 MHz and 
2110-2170 MHz for mobile/fixed communications networks (MFCN) including terrestrial IMT systems.

RIG II Template on harmonise the use of the bands 1920-1980 MHz and 2110-2170 MHz

for mobile/fixed communications networks (MFCN) including terrestrial IMT systems

Radio Interface Notification by an administration

| Nomative part | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
|  | 1 | Radiocommunication Service | Mobile Service, Fixed Service |  |
|  | 2 | Application | Mobile/fixed communications networks (MFCN) | Including terrestrial IMT systems |
|  | 3 | Frequency band | 1920-1980 MHz and 2110-2170 MHz | FDD harmonised frequency arrangement |
|  | 4 | Channelling | Channel spacing is not specified Frequency blocks edges are defined as follows: - The block edge nearest to 1920 MHz should start at 1920.3 MHz or above (Where necessary, this frequency can be lowered to 1920.0 MHz for consistency with conditions of some existing authorisations ). - The block edge nearest to 1980 MHz should end at 1979.7 MHz or below (Where necessary, this frequency can be raised to 1980.0 MHz for consistency with conditions of some existing authorisations ). - The block edge nearest to 2110 MHz should start at 2110.3 MHz or above (Where necessary, this frequency can be lowered to 2110.0 MHz for consistency with conditions of some existing authorisations ). - The block edge nearest to 2170 MHz should end at 2169.7 MHz or below (Where necessary, this frequency can be raised to 2170.0 MHz for consistency with conditions of some existing authorisations ). | For licensing purposes the bands 1920-1980 MHz and 2110-2170 MHz are divided into twelve paired blocks and the minimum block size should be in the range 4.8 MHz to 5 MHz. |
|  | 5 | Modulation / Occupied bandwidth | Not specified | Technology neutral approach |
|  | 6 | Direction / Separation | Uplink: 1920-1980 MHz Downlink: 2110-2170 MHz Duplex separation: 190 MHz |  |
|  | No | Parameter | Description | Comments |
|  | 7 | Transmit power / Power density | In block e.i.r.p. levels for Base stations are regulated on a national basis (administrations may choose to set an in block e.i.r.p. limit for BS if needed on a national or local basis). Out of block e.i.r.p. BEM requirements for MFCN base stations are defined in Annex 2 Table 1 of ECC/DEC(06)01 and were based on the assumption of an in-block e.i.r.p. limit of 61dBm/5MHz. | An administration should ensure that operators to which it has granted authorisations in this band are free to enter into bilateral or multilateral agreements to develop less stringent technical parameters and, if agreed among all affected parties, these less stringent technical parameters may be used, if the level of protection for other networks (not party to the agreement) is not affected. |
|  | No | Parameter | Description | Comments |
|  | 8 | Channel access and occupation rules | Not specified |  |
|  | 9 | Authorisation regime | Individual authorisation including Block Edge Mask (BEM) technical conditions | A BEM is a spectrum mask that is defined, as a function of frequency, relative to the edge of a block of spectrum that is licensed to an operator. It consists of in-block and out-of-block components which specify the permitted emission levels over frequencies inside and outside the licensed block of spectrum respectively. |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions |  |  |
|  | 12 | Planned changes |  |  |
|  | 13 | Reference | ECC/DEC/(06)01 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Abbreviations BS Base Station e.i.r.p. Equivalent isotropically radiated power FDD Frequency Division Duplex IMT International Mobile Telecommunications |  |