RIS implementation of ECC DEC(09)02

Foreword

The ECC has decided that RIS implementations of ECC deliverables should be uploaded to the ECO website in order to help administrations fill out the EFIS database.

The RIS implementation is limited to harmonisation of the bands 1610-1626.5 MHz and 2483.5-2500 MHz for use by systems in the Mobile-Satellite Service.

| Radio Interface Specification | ECC DEC (09)02 |  | 26 June 2009 |
| --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Land mobile-satellite service |  |
|  | 2 | Application | "Satellite-Personal Communications earth stations" applications |  |
|  | 3 | Frequency band | 1 610 - 1 626.5 MHz Transmit (earth to space) 1621.35 - 1 626.5 MHz Receive (space to earth) 2 483.5 - 2 500 MHz Receive (space to earth) |  |
|  | 4 | Channelling | to be defined by the satellite operator |  |
|  | 5 | Modulation / Occupied bandwidth | to be defined by the satellite operator |  |
|  | 6 | Direction / Separation | to be defined by the satellite operator |  |
|  | 7 | Transmit power / Power density | -3 dB (W/4 kHz), (mean limit) or -15 dB (W/4 kHz), (peak limit) | Applicable in 1610 – 1626.5 MHz Applicable in parts of the band where there are airborne electronic aids to air navigation, in accordance with the provisions of RR No. 5.364 and 5.366 |
|  | 8 | Channel access and occupation rules | Duty cycle of 1 % max |  |
|  | 9 | Authorisation regime | land mobile satellite terminals operating under the control of an authorised satellite system are license exempt | See DEC(07)05 |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions | Not required |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | Harmonised Standard: EN 301 441 ECC DEC(09)02 ECC DEC(07)05 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |