Radio Interface Specification (RIS) implementation of ECC/DEC/(15)04 on the harmonised use, free circulation and exemption from individual licensing of Land and Maritime Earth Stations On Mobile Platforms (NGSO ESOMPs) operating with non-geostationary (NGSO) FSS satellite systems in the frequency ranges 17.3-20.2 GHz, 27.5-29.1 GHz and 29.5-30.0 GHz

Foreword

The ECC has decided that RIS implementations of ECC deliverables should be uploaded to the ECO website in order to help administrations fill out the EFIS database.

This RIS implementation is limited the harmonised use, free circulation and exemption from individual licensing of Land and Maritime Earth Stations On Mobile Platforms (NGSO ESOMPs) operating with NGSO FSS satellite systems in the frequency ranges17.3-20.2 GHz, 27.5-29.1 GHz and 29.5-30.0 GHz.

RIG II Template for Earth Stations On Mobile Platforms (NGSO ESOMPs) operating with NGSO FSS satellite systems in the frequency ranges

17.3-20.2 GHz, 27.5-29.1 GHz and 29.5-30.0 GHz

Radio Interface Notification by an administration

|  | No | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Fixed Satellite Service (FSS) |  |
|  | 2 | Application | Earth Stations On Mobile Platforms (NGSO ESOMPs) | Applies to land and maritime ESOMPs operating in a non-geostationary (NGSO) satellite network. |
|  | 3 | Frequency bands | To be specified by the administration, within the frequency bands 17.3-20.2 GHz, 27.5-29.1 GHz and 29.5-30.0 GHz | See ECC/DEC/(15)04 Decides 2a, 2b and 2c. |
|  | 4 | Channelling | Not applicable | Defined by the satellite network operator. |
|  | 5 | Modulation / Occupied bandwidth | Not applicable | Defined by the satellite network operator. |
|  | 6 | Direction / Separation | 17.3-19.7 GHz (space-to-Earth), 19.7-20.2 GHz (space-to-Earth) 27.5-27.8285 GHz (Earth-to-space) 28.4445-28.8365 GHz (Earth-to-space) 28.8365-28.9485 GHz (Earth-to-space) 29.5-30.0 GHz (Earth-to-space) |  |
|  | 7 | Transmit power / Power density | Maximum e.i.r.p. for NGSO system specified by administrations with orbital characteristics specified in Annex 3 of ECC/DEC/(15)04 | The transmission characteristics of NGSO ESOMPs shall be limited to the conditions as specified in Annexes 1 to 3 of ECC/DEC/(15)04. |
|  | 8 | Channel access and occupation rules | Not applicable | Defined by the satellite network operator. |
|  | 9 | Authorisation regime | Exemption from individual licensing | No registration and/or notification is required (commonly known as “license exempt”). |
|  | 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
|  | 11 | Frequency planning assumptions |  | Additional technical and operational requirements are described in ECC Decision (15)04 for Land and Maritime NGSO ESOMPs for the protection of other authorised users in the operating frequency band. Note that national requirements related to the protection of aircraft from NGSO ESOMPs operating on the earth surface in the vicinity of an airfield should be provided to the ECO (see Decides 2e of ECC/DEC/(15)04). |
| Informative part | 12 | Planned changes | --- |  |
|  | 13 | Reference | ECC Decision (15)04 ETSI EN 303 979 |  |
|  | 14 | Notification number | --- |  |
|  | 15 | Remarks | Definitions |  |
|  |  |  | Abbreviations e.i.r.p. equivalent isotropically radiated power |  |