- RIS implementation of ECC/REC/(09)01

- Foreword

The ECC has decided that RIS implementations of ECC deliverables should be uploaded to the ECO website in order to help administrations fill out the EFIS database.

This RIS implementation is limited to the use of the band 57-64 GHz for point-to-point FWS.

| CEPT | Radio Interface Specification | Use of the 57-64 GHz band for point-to-point FWS | ECC/REC(09)01 | February 2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | FIXED SERVICE |  |
|  | 2 | Application | Point-to-point fixed wireless systems |  |
|  | 3 | Frequency band | 57 - 64 GHz | Country specific sub-allocation possible, combination with 64-66 GHz channel planning (ECC/REC(05)02) is also possible |
|  | 4 | Channelling | n * 50 MHz | Maximum aggregated bandwidth up to 2500 MHz; |
|  | 5 | Modulation / Occupied bandwidth | Modulation: not available Occupied bandwidth: see channelling |  |
|  | 6 | Direction / Separation | FDD – TDD Duplex gap: flexible |  |
|  | 7 | Transmit power / Power density | Maximum EIRP +55 dBm Minimum antenna gain +30 dBi Maximum transmitter output power +10 dBm | An additional limit on the transmit output power density (-10dBm/MHz) in the 59 - 64 GHz can be implemented to support the deployment of wideband systems (i.e. bandwidth higher than 100 MHz) by consequently limiting the maximum transmitter output power for narrow band systems (i.e. bandwidth lower than 100 MHz) below that of the maximum (+10dBm) allowed in the 59 - 64 GHz band. This limit will not apply if administrations wish to implement narrowband systems in the band |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime | Country specific – Individual or General Authorisation | ECC REC(09)01 is flexible to accommodate e.g. link by link (including light licensing), block licensing and license exempt authorisation regime. |
|  | 10 | Additional essential requirements | Not applicable |  |
|  | 11 | Frequency planning assumptions | Not applicable |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | ECC/REC/(09)01, ETSI EN 302 217 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |