RIS implementation of ECC REC(11)01

Foreword

The ECC has decided that RIS implementations of ECC deliverables should be uploaded to the ECO website in order to help administrations fill out the EFIS database.

The RIS implementation is limited to Guidelines for assignment of frequency blocks for Fixed Wireless Systems in the bands 24.5-26.5 GHz, 27.5-29.5 GHz and 31.8-33.4 GHz.

| Radio Interface Specification | ECC REC(11)01 | 2 February 2011 |
| --- | --- | --- |

| Nr | Parameter | Description | Comments |
| --- | --- | --- | --- |
| 1 | Radiocommunication Service | FIXED SERVICE |  |
| 2 | Application | Point-to- MULTI point fixed wireless systems |  |
| 3 | Frequency band | 24.5-26.5 GHz, 27.5-29.5 GHz and 31.8-33.4 GHz |  |
| 4 | Channelling | n * 28 MHz blocks | As aggregation of the channels recommended in ERC Recommendation T/R 13-02 and ERC/REC/(01)02 |
| 5 | Modulation / Occupied bandwidth | Modulation: not specified Occupied bandwidth: free inside the block |  |
| 6 | Direction / Separation | FDD – TDD Duplex gap: none for TDD and fixed for FDD as defined in ERC Recommendation T/R 13-02 and ERC/REC/(01)02 |  |
| 7 | Transmit power / Power density | Not specified |  |
| 8 | Channel access and occupation rules | Not specified |  |
| 9 | Authorisation regime | Country specific |  |
| 10 | Additional essential requirements according to Art. 3.3 of R&TTE Directive | None |  |
| 11 | Frequency planning assumptions | Point to Multi-point cell or Mesh structure |  |

Informative Part

| Nr | Parameter | Description | Comments |
| --- | --- | --- | --- |
| 12 | Planned changes | None |  |
| 13 | Reference | ERC Recommendation T/R 13-02 and ERC/REC/(01)02 ETSI EN 302 326-2 ETSI EN 302 326-3 (antenna pattern) |  |
| 14 | Notification number |  |  |
| 15 | Remarks |  |  |