European Radiocommunications Committee (ERC)

within the European Conference of Postal and Telecommunications Administrations (CEPT)

GENERAL METHODOLOGY FOR ASSESSING

COMPATIBILITY BETWEEN RADIO LOCAL AREA NETWORKS (RLANs)

AND THE FIXED SERVIE

Oslo, December 1991

Reports are being issued from time to time by the European Radiocommunications Committee (ERC) of CEPT to inform industry, operators, users and other interested parties of the work in hand, provisional conclusions and future activities in specific areas of radio frequency management. Such Reports give more details than is normally possible in a Recommendation and allow an opportunity for comment to be made on the work carried out so far. In most cases, it would be hoped that a formal CEPT Recommendation could be issued on the subject of the Report in due course, taking into consideration any comments received on the Report.

Reports are formally approved by, and issued in the name of, the Committee itself. In general the detailed preparation of Reports, and further work on the subject, will be done by Working Groups or Project Teams. Thus, any reference in the Reports to the ERC should be taken to include the whole framework of the ERC, including its Working Groups, Project Teams, etc.

GENERAL METHODOLOGY FOR ASSESSING

COMPATIBILITY BETWEEN RADIO LOCAL AREA NETWORKS (RLANs)

AND THE FIXED SERVICE

Based on ERC Report I on harmonisation of frequency bands for Radio Local Area Networks (RLANs) the ERC has adopted CEPT Recommendation T/R 10-01 E recommending frequency bands and power levels to be used i RLANs.

The present ERC Report outlines the general methodology for assessing compatibility between RLANs using TDMA/TDD technology and stations of the fixed service in Appendix I and methodology for calculating interference between direct sequence spread spectrum systems and narrow band systems in Appendix 2.

The methodology described is intended for use by Administrations and users when performing compatibility calculations.

Appendix 1

# GENERAL METHODOLOGY FOR ASSESSING

COMPATIBILITY BETWEEN RADIO LOCAL AREA NETWORKS

USING TDMA/TDD TECHNOLOGY AND FIXED SERVICES

- INTRODUCTION

A report from the European Radiocommunications Committee within CEPT on "Harmonisation of frequency bands to be designated for Radio Local Area Networks'' (ERC Report 1, Lisbon, February 1991) identifies three basic categories of Radio Local Area Networks (RLANs). The report notes that a RLAN has been developed which operates in the frequency band 18.8-19.2 GHz, uses TDMA/TDD technology and meets the requirements of Category (c) Systems, defined in the ERC Report 1 as giving ''reasonable in-building penetration and medium data rates (at least 10-15 Mbits.)". In many CEPT countries, this frequency range is used to accommodate a dense network of fixed links, consequently a methodology has been developed to assess the compatibility between this type of RLAN and the fixed service. The methodology is sufficiently flexible to be used for similar sharing situations in any frequency range.

- ASSESSMENT OF COMPATIBILITY

In order to assess the compatibility between systems, it is necessary to specify a system performance criterion which must not be degraded beyond a desired minimum level. This can usually be related to the maximum permissible level of received interference power in a reference bandwidth, or the minimum value for the carrier to interference (C/I) ratio.

By using either of these parameters, together with a suitable propagation model and the effective radiated power of the interfering source (in the direction of the receiver to be protected), the minimum separation distance between the different systems can be calculated.

As the separation distance decreases, fewer systems will be within the co-ordination range, so, at certain distance, the probability of interference between systems will be sufficiently low such that co-ordination may be considered unnecessary.

- THE COMPATIBILITY MODEL

The compatibility model is derived from a series of system loss/gain equations combined into a single equation which enables the separation distance to be calculated.

The definitions for all the terms used are listed at Annex 1.

- Power level received from the interfering source

The example RLAN considered (most RLANs will have a similar basic configuration) comprised a number of user modules (at each computer terminal) which communicate with, and via, a central control module. The control module and each user module must be considered as potential interfering sources; the received power into the fixed service receiver is given by the following equations:

Equ. 1	Pu – Tu + Gu– Luf – B + Gf (v)

Equ. 2	Pc – Tc + Gc– Lcf – B + Gf (v)

- Minimum attenuation required between systems

By substituting the maximum permissible interference power level (1) for the fixed service for the terms Pu and Pc in equations 1 and 2 above, and rearranging, equations can be obtained for the necessary attenuation between systems:

Equ. 3	Luf > Tu + Gu  – B + Gf (v) – I

and

Equ. 4	Lcf > Tc + Gc  – B + Gf (v) – I

Annex 1 (note 3) gives a detailed consideration of the value for (I). Table 1 lists values of (I) for various fixed systems.

- Geographical separation requirements

The necessary isolation can be provided by the propagation loss obtained by geographical separation of the systems.

A first approximation of the minimum separation distance can be determined by using the simple free space path loss equation:

Equ. 5	Lbf = 92.45 + 20 log(f) + 20 log(d)	(dB)

where f = frequency (GHz); d = distance (km).

Alternatively, more detailed propagation models call be used which take into account diffraction loss, urban clutter, etc.

Where derailed terrain height and ground cover information is available, deterministic models may be applied to accurately calculate attenuation along the interference path using computer based prediction tools.

EXAMPLE CALCULATION

Fixed Link Parameters

(reference: CCIR IWP9/6)

Imax	=	– 147 dBW/MHz (Long term)

f	=	18 GHz

Gf(0)	=	45 dBi

Elevation	=	0°

See Annex 1, Note 3 for more information on the value for Imax.

## RLAN Parameters

| PARAMETER | User Module |  | Control Module |  |
| --- | --- | --- | --- | --- |
|  | Best | Worst | Best | Worst |
| Peak transmit power (dBW/MHz) Antenna Gain (dBi) Building Loss (dB) Window Loss (dB) Apparent e.i.r.p. density (dBW/MHz) | – 28 0 { 25 { – 43 | – 28 10 0 – 18 | – 28 0 0 5.6 – 33.6 | – 28 10 0 0 – 18 |

## Required separation distance (using equation 5)

| Fixed link Antenna offset from boresight (v°) | Gain of fixed link antenna Gf(v°) dBi (from Annex 2) | Separation distance (d) km |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | User Module |  | Control Module |  |
|  |  | Best | Worst | Best | Worst |
| 0 5 10 20 40 80 | 45 22 10 5 – 1 – 12 | 12 0.8 0.2 0.1 0.06 0.02 | 664 47 11.8 6.6 3.3 0.9 | 110 7.8 1.9 1.1 0.6 0.2 | 664 47 11.8 6.6 3.3 0.9 |

Results of detailed sharing analysis using computer based sharing analysis tools were made available to the project team by one Administration. The results were presented in graphical format, showing the interference zone around the fixed link receiver site as a series of colour contours corresponding to various values of effective RLAN e.i.r.p. (after building attenuation). The results of the detailed analysis indicated separation distances comparable to those indicated in the above table.

### ANNEX 2

### Table 1: PTO 18 GHz fixed services

| FREQUENCY BAND (GHz) | 17.7-19.7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MODULATION CAPACITY CHANNEL PLAN CHANNEL SPACING (MHz) ANTENNA GAIN MAX (dB) ANTENNA GAIN TYP (dB) FEEDER/MUX LOSS MIN (dB) FEEDER/MUX LOSS TYP (dB) ANTENNA TYPE TYP ANTENNA R.P.E. MAX Tx OUTPUT POWER (dBW) TYPICAL Tx OUTPUT POWER (dBW) e.i.r.p. MAX (dB) e.i.r.p. TYP (dB) RECEIVER IF BANDWIDTH 3dB (MHz) RECEIVER NOISE FIGURE (dB) RECEIVER THERMAL NOISE (dBW) NOMINAL Rx. INPUT LEVEL (dBW) Rx I/P LEVEL FOR 10 E-3 BER (dBW) Rx I/P LEVEL FOR 10 E-6 BER (dBW) Rx I/P LEVEL FOR 10 E-10 BER (dBW) TYPICAL FADE MARGIN (dB) TYPICAL SPAN LENGTH (km) | QPSK 140 Mbit/s Rec. 595-1 110 48 43 7 10 0.3-1.8 m RC6691A - 10 - 10 31 23 68 7 - 119 -63 -103 -99 -95 40 1-12 | 4QAM 140 Mbit/3 Rec. 595-1 55 48 43 7 10 0.3-1.8 m RC6691A - 4 - 4 37 29 68 8 - 118 - 64 - 104 - 100 - 96 40 1-15 | 2FSK 8 Mbit/s Rep. 936-1 20 45 45 0 3 0.6-1.2 m RC6691A - 12 - 12 33 30 4 11 - 127 - 72 - 111 - 107 - 103 35 1-15 | 4FSK 8 Mbit/s Rep. 936-1 20 45 38 0 3 0.3-1.2 m RC6691A - 16 - 16 29 19 8 13 - 122 - 65 - 106 - 102 - 99 50 1-15 | QPSK 8 Mbit/s Rep. 936-1 20 45 38 0 3 0.3-1.2 m RC6691 A - 6 - 6 39 29 4 7 - 131 - 65 - 116 - 112 - 109 50 1-25 | BPSK 8 Mbit/s Rep. 936-1 20 45 38 0 3 0.3-1.2 m RC6691A - 9 - 9 27 23 8 7 - 128 - 65 - 116 - 112 - 108 40 1-25 | QPSK 34 Mbit/s Rec. 595-1 27.5 45 38 0 3 0.3-1.2 m RC6691A - 8 - 10 37 25 18 7 - 124 - 65 - 113 - 110 - 107 45 1-20 |
| MAX. LONG-TERM INTERFERENCE (dBW) Refer to notes | - 129 (B), (1) | - 131 (B), (1) | - 137 (B), (1) | - 132 (B), (1) | - 141 (B), (1) | - 138 (B), (1) | - 134 (B), (1) |
| EQUIVALENT P.S.D. (dBW/MHz) | - 147 | - 149 | - 143 | - 141 | - 147 | - 147 | - 147 |

NOTES.

- Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor).

- Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor).

- FS/FSS sharing on a co-primary basis (see method in CCIR Reps. 382/448 and Appendix 28 to the Radio Regulations).

- The specified interference level is total power within the receiver 3 dB bandwidth

- The specified interference level should be divided by the receiver 3 dB bandwidth to obtain an average spectral density

- The interference spectral density, averaged over any 4 kHz within the receiver 3 dB bandwidth, must not exceed this value

### APPENDIX 2

METHODOLOGY FOR CALCULATING INTERFERENCE BETWEEN DIRECT SEQUENCE SPREAD

SPECTRUM SYSTEMS AND NARROW BAND SYSTEMS

#### INTRODUCTION

This document comprises two sections, the first gives a methodology for calculating the interference caused to a narrow band transmission from spread spectrum interference. The second section considers the reverse case of narrow band interference to spread spectrum transmissions.

METHODOLOGY FOR CALCULATING SPREAD SPECTRUM INTERFERENCE

TO NARROW BAND TRANSMISSIONS

#### Note

All parameters and results should be expressed in dBs.

#### Assumptions

- A typical e.i.r.p. can be defined for the spread spectrum interferer.

- Free space propagation is assumed (as defined in CCIR Recommendation 525-1).

- All spread spectrum transmitters transmit on the same centre frequency and spread their signals over the same bandwidth.

#### Definitions

| eirpSS | Typical e.i.r.p. per spread spectrum transmitter in the direction of the narrow band receiver. |
| --- | --- |
| DI | Distance between spread spectrum interferer and the narrow band receiver. |
| N | Number of spread spectrum transmitters causing interference to narrow band system. |
| Ir | Interference power at narrow band receiver originating from a single spread spectrum interferer. |
| IR | Total interference power at the narrow band receiver from spread spectrum transmissions. |
| IRN | Total interference power contained within the receiver bandwidth of the narrow band receiver. |
| fcs | Centre frequency of the interfering spread spectrum transmission. |
| fcn | Centre frequency of the narrow band transmission. |
| G(,)nb | Gain of the narrow band receiving antenna in the direction of the spread spectrum transmitter. |
| eirpnt | Effective isotropically radiated power from the narrow band transmitter in the direction of the narrow band receiver |
| Dw | Distance separation between the narrow band transmitter and receiver. |
| PNR | Power received at the narrow band receiver. |
| NT | Total thermal noise within the narrow band receiver. |
| cs | Centre wavelength or the spread spectrum transmission. |
| cn | Centre wavelength of the narrow band transmission. |
| Tc | Chip rate. |
| k | Added margin to account for out of band interference, adjacent channel interference, etc. |
| M | Carrier to interference ratio required to ensure satisfactory operation. |

The bandwidth occupied by the wanted spread spectrum signal once it has been de-spread is taken to have an upper frequency limit fu and a lower frequency fl. These limits are symmetrical about the centre frequency of the spread spectrum transmission. The narrow band interferer which is now spread across the bandwidth BWT is centred on the centre frequency of the narrow band transmission. The level of interference experienced by the spread spectrum signal from the narrow band interiors its determined by the peak level of the ‘spread’ narrow band signal, the relative positions of the signals centre frequency’s find the information bandwidth BWI.