European Radiocommunications Committee (ERC) within the

European Conference of Postal and Telecommunications Administrations (CEPT)

FREQUENCY SHARING BETWEEN UMTS

AND EXISTING FIXED SERVICES

Menton, May 1999

FREQUENCY SHARING BETWEEN UMTS

AND EXISTING FIXED SERVICES

1	INTRODUCTION	1

2	FIXED LINK DEPLOYMENT AND PARAMETERS	1

3	UMTS SHARING PARAMETERS	1

4	MODEL FOR STUDY OF FREQUENCY SHARING	1

4.1	Propagation model	1

4.2	Interference calculation model	1

5	CO-CHANNEL FREQUENCY SHARING ANALYSES	2

5.1	Sharing combinations	2

5.2	Example results	2

6	DISCUSSION AND CONCLUSION	4

7	REFERENCE	4

ANNEX A : 	Existing Services in the Frequency Bands designated for UMTS in accordance with

the ERC Decision (97)07 (January 1998)………………………………………………………………………...7

FREQUENCY SHARING BETWEEN UMTS

AND EXISTING FIXED SERVICES

# INTRODUCTION

This report addresses co-frequency sharing and compatibility issues surrounding the implementation of UMTS, given the operation of existing fixed service systems in some European countries in the bands identified in ERC Decision DEC (97) 07 on UMTS.

The report examines the geographical separation distances that may be required between UMTS stations and fixed services.

# FIXED LINK DEPLOYMENT AND PARAMETERS

The ERO Study on UMTS [1] contains details of the current fixed link deployment within the UMTS bands in Europe. Most countries have some fixed links in operation at present in the bands identified for UMTS, many having several hundred transmitters.

Fixed links in the UMTS bands are deployed mainly in accordance with ITU-R Rec. F.382 and F.283 channel plans. The sharing parameters of typical 2GHz fixed links can be found in ITU-R Recommendation F.759.

The key fixed link parameters required for the co-frequency sharing studies are listed in Table 1.

Results of the ERO Survey on existing use of the 2 GHz band are shown in Annex A.

# UMTS SHARING PARAMETERS

The key UMTS radio system parameters required for the frequency sharing studies are shown in Table 2 below.

At the current time (July 1998) the UMTS radio parameters are not yet fully defined within ETSI. However, in liaison with ETSI SMG2 (see Doc ERC TG1 (97) 66), some example values have been derived for the key parameters which are relevant to co-channel frequency sharing and it is therefore possible to undertake some preliminary example calculations.

# MODEL FOR STUDY OF FREQUENCY SHARING

## Propagation model

The standard model agreed in the ITU and CEPT for terrestrial interference assessment at microwave frequencies is ITU-R Recommendation P.452-8. It is therefore proposed that this model is used in the assessment of interference potential between UMTS and fixed links using MCL methods.

For Monte Carlo simulations the “WG-SE” (Hata based) model developed in CEPT is recommended.

## Interference calculation model

There are two basic approaches to calculating the interference potential between systems that have been used in previous CEPT ERC WG-SE work, namely minimum coupling loss (MCL) and the Monte Carlo statistical method.

The former model is simpler to implement, but is conservative and can give more pessimistic results than are obtained in reality in cases where some of the input parameters in practice have a probability associated with them. CEPT ERC WG-SE is in the process of developing a software implementation of the Monte Carlo model (via a MoU between interested parties), however this may not be available until Summer 1999.

For the purposes of this ERC Report the MCL method is adopted in order to carry out a first assessment in short timescales. It is considered that the MCL method is sufficient for examining co-frequency sharing between fixed links and base stations (a relatively static situation), whereas this approach is slightly pessimistic in the case of fixed links vs mobiles.  However, it is not envisaged that a Monte Carlo analysis would give very different results from the MCL approach in the specific co-channel sharing situation covered by this Report.

# CO-CHANNEL FREQUENCY SHARING ANALYSES

## Sharing combinations

There are four interference modes that need to be considered :

UMTS Mobile transmitter ® Fixed Link Receiver

UMTS Base transmitter  ® Fixed Link Receiver

Fixed link transmitter ® UMTS Mobile Receiver

Fixed link transmitter ® UMTS Base Station Receiver

The various fixed link equipment types and UMTS environments need to be considered in order to get a view of the scale of the typical co-channel frequency sharing problem between UMTS and co-channel fixed links.

Although these studies apply to co-channel interference problems, the same method could be applied to adjacent band compatibility issues. In this case the receiver blocking performance and the transmitter out of band emissions need to be taken into account.

## Example results

Example 1 : UMTS Base station transmitter ® Fixed Link Receiver

Figure 1 shows the results of long-term (20% time) interference calculations around an example 2 GHz fixed link in the United Kingdom from a single multichannel UMTS wide-area coverage base station. Similar calculations done for propagation conditions at shorter time percentages (e.g. 0.01%) and the example fixed link sharing criterion for that time percentage are shown in Figure 2.

The sharing parameters used in this example study are listed below.

| UMTS Base Tx |  | Fixed link receiver |  |
| --- | --- | --- | --- |
| (W-CDMA) |  | (960cct FM FDM) |  |
| Carrier Frequency | 2 .15 GHz | Antenna gain and radiation pattern | 35 dBi (measured RPE) |
| Location | Trial located every 0.25 sq km | Feeder and multiplexer losses | 3 dB |
| Ground height | (Database) | Antenna height a.g.l. | 48m |
| Antenna height | 20m agl | 3dB IF bandwidth (and preferably Rx filter mask) | 29 MHz |
| Antenna pointing | Towards FS | Nominal Rx input level | N/A |
| 3dB channel bandwidth | 4.1 MHz |  |  |
| Power into antenna | 11 dBW | Maximum permissible interference power density | -141 dBW/MHz (20% of time) |
| Antenna gain | 14.5 dBi | (long and short-term time %) | -111 dBW/ MHz (0.01 % time) |
| Channeling and frequency re-use plan | Single multi- channel BS |  |  |

The shaded areas on the plot represent locations where, if a UMTS base station is located, the interference level at the fixed service receiver is predicted to exceed the sharing criterion. Additional contours are included to illustrate the improvement in the sharing situation if additional losses were available (e.g. Frequency Offset Rejection for adjacent channel sharing, additional losses due to clutter and site shielding etc.).

In this particular example it can be seen that the potential interference zones extend to many 10s of km, particularly along the fixed link receiver antenna boresight.

Example 2 :  Fixed link transmitter ® UMTS base station receiver

Figure 3 shows the results of long-term (20% time) interference calculations around an example 2 GHz fixed link transmitter in the United Kingdom into a UMTS wide-area coverage base station receiver.  The short term interference requirements of the UMTS receivers are unspecified (as is normally the case with such mobile systems) and so this has not been evaluated.

The sharing parameters used in this example study are listed below.

| UMTS BS receiver |  | Fixed link transmitter |  |
| --- | --- | --- | --- |
| (W-CDMA) |  | (960 cct FM FDM) |  |
| Carrier Frequency | 1.95 GHz | Antenna gain and radiation pattern | 35 dBi (measured RPE) |
| Location | Trial located every 0.25 sq km | Feeder and multiplexer losses | 3 dB |
| Ground height | (Database) | Antenna height a.g.l. | 48m |
| Antenna height | 20m agl | Ground height | (Database) |
| Antenna pointing | Towards FS | Transmitter power (in UMTS receiver bandwidth) | 0 dBW |
| 3dB channel bandwidth | 4.1 MHz |  |  |
| Maximum permissible interference power density | -133 dBW (20% of time) |  |  |
| Antenna gain | 14.5 dBi |  |  |

The shaded areas on the plot represent locations where, if a UMTS base station receiver is located, the interference level from the fixed service transmitter is predicted to exceed the sharing criterion. Additional contours are included to illustrate the improvement in the sharing situation if additional losses were available (e.g. Frequency Offset Rejection for adjacent channel sharing, additional losses due to clutter and site shielding etc.).

# DISCUSSION AND CONCLUSION

The sharing situation between UMTS and existing fixed services will depend on the exact operational parameters of the UMTS and fixed service systems as well as factors such as the terrain features at the particular geographical location under consideration. The UMTS sharing parameters are not yet fully defined, particularly those parameters needed to assess adjacent band compatibility.

It is therefore not possible to give definitive results on the separation/coordination distances required between these systems. However, the methods described above and the results of example studies can provide guidance to administrations planning deployment or coordination of UMTS with existing fixed services.

# REFERENCE

1.	ERO report on UMTS

European Radio Office, Copenhagen, September 1996

Table 1: Fixed link sharing parameters

| Transmitter | Receiver |
| --- | --- |
| Centre Frequency | Antenna gain and radiation pattern |
| Modulation & Capacity | Feeder and multiplexer losses |
| 3dB Bandwidth | Antenna height a.g.l. |
| Antenna gain and radiation pattern | 3dB IF bandwidth |
| Feeder and multiplexer losses | Nominal Rx input level |
| Antenna height a.g.l. |  |
| Tx output power | Maximum permissible interference power density |
| Polarization | (long and short-term time %) |
| Transmitter mask | Frequency offset attenuation |

Table 2: UMTS Parameters for frequency sharing

| Base / Mobile Tx | Base / Mobile Rx |
| --- | --- |
| Carrier Frequency | Centre Freq. |
| Location or density | Location or density |
| Ground height | Ground height |
| Antenna height | Antenna height |
| Antenna pointing and radiation pattern | Antenna pointing and radiation pattern |
| 3dB channel bandwidth | 3dB channel bandwidth |
| Power into antenna | Noise figure |
| Antenna gain | Sensitivity |
| Channeling and frequency re-use plan | Intrasystem C/Ic |

Figure 1 : Example Sharing Study (UMTS BS Tx into FS Rx) –Long term interference

Figure 2 : Example Sharing Study (UMTS BS Tx into FS Rx) –Short term interference

Figure 3 : Example Sharing Study (FS Tx into BS Rx) –Long term interference

ANNEX A : Existing Services in the Frequency Bands designated for UMTS in accordance with the ERC Decision (97)07 (January 1998)

| Country | Frequency range 1900 - 1980 MHz |  |  |  | Frequency range 2010-2025 MHz |  |  |  | Frequency range 2110-2170 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y2002 |  | Y2005 | > Y2005 | Y2002 |  | Y2005 | > Y2005 | Y2002 | Y2005 | > Y2005 |
| Austria | 1920-1980 40 links R- 382 |  | Free |  | 2012-2025 10 links R-382 |  | Free |  | 2120-2170 10 ENG-OB | Free |  |
| Belgium | Free |  |  |  | Free |  |  |  | Fixed Service |  |  |
| Cyprus | Free |  |  |  | Free |  |  |  | Fixed service - 2130 MHz |  |  |
| Denmark | Free |  |  |  | Free |  |  |  | Free |  |  |
| Estonia | Fixed Service 10 links (35 dBW) |  |  |  |  |  |  |  |  |  |  |
| Spain | Fixed Service 350 High Capacity links |  |  |  | Fixed Service 150 High Capacity links |  |  |  | Fixed Service 450 High Capacity links |  |  |
| Finland | Fixed Service 1919.5-1975.5 197 links 2*8 Mbit/s |  |  |  | Free |  |  |  | Free |  |  |
| France | No info |  |  |  | No info |  |  |  | No info |  |  |
| Germany | Free |  |  |  | 2020-2025 Tactical Radio Relay |  |  |  | Free |  |  |
| Iceland | Fixed Service 30 links (ITU-R F 283) |  |  |  | Fixed Service 4 links (ITU-R F 283) |  |  |  | Fixed Service 54 links (ITU-R F 283) |  |  |
| Italy | Fixed links (BC distribution) 600 links |  |  |  | Fixed links (BC distribution) 100 links |  |  |  | Fixed links (BC distribution) 400 links |  |  |
| Latvia | Fixed Service 1930-1980 ITU-R 382 | Free |  |  | Fixed Service 2010-2025 ITU-R 382 | Free |  |  | Fixed Service 2110-2170 ITU-R 382 | Free |  |
| Luxembourg | Military use |  |  |  | Military use |  |  |  | Military use |  |  |
| Poland | Fixed Service 1900-1980 Military / 1900-1960 Civil 22 analogue links |  |  |  | Fixed Service 2015-2025 Civil 1 analogue link 1985-2015 Military (Altimeter) |  |  |  | 2120-2170 Military Radiolocation |  |  |
| Portugal | Fixed Service 35 links 9 analogue links 16links 8Mbit/s - 19 links 34 Mbit/s |  |  |  | Fixed Service 5 links 1 analogue link 4 links 34 Mbit/s |  |  |  | Fixed Service 17 links 7 analogue links 10 links 34 Mbit/s |  |  |
| Slovak Republic | ENG 1900-2000 2 links (28 MHz 30dBm) Military Fixed Mobile |  |  |  | ENG 2016.5 1 link (28 MHz 30dBm) Military Fixed/Mobile |  |  |  | MVDS 2100-2300 MHz Military Fixed |  |  |
| Sweden | Military use 150 links 3.5/14 MHz channels |  |  |  | Military use 150 links 3.5/14 MHz channels |  |  |  | Military use 100 links 3.5/7 MHz channels |  |  |
| Turkey | Rural Telephone 50 links (34 Mbit/s) |  |  |  | Rural Telephone 7 links (34 Mbit/s) |  |  |  | Rural Telephone 1 link (34 Mbit/s) |  |  |
| United Kingdom | Fixed Service 13 Analogue TROPO FMTV,QPSK,FMFDM |  |  |  | Fixed Service 14 Analogue TROPO FMTV,QPSK,FMFDM |  |  |  | Fixed Service 24 Analogue TROPO FMTV,QPSK,FMFDM |  |  |