European Radiocommunications Committee (ERC)

within the European Conference of Postal and Telecommunications Administrations (CEPT)

MONTE-CARLO SIMULATION METHODOLOGY FOR THE USE IN SHARING AND COMPATIBILITY STUDIES BETWEEN DIFFERENT RADIO SERVICES OR SYSTEMS

Naples, February 2000

revised in Regensburg, May 2001 and Baden, June 2002

SUMMARY

In this report background information on a Monte-Carlo Radio Simulation methodology is given. Apart from giving general information this text also constitutes a specification for the first generation of SEAMCAT software which implements the Monte-Carlo methodology applied to radio communication scenarios. This Reports contains the upgrade of phase 1 specified finally in November 2000, is based on the previous version of this Report as it was adopted in Naples, February 2000 and the corresponding new ITU-R Report SM.2028.

GENERAL

The problem of unwanted emissions, as a serious factor affecting the efficiency of radio spectrum use, is being treated in depth in various fora, internal and external to the CEPT. As the need to reassess the limits for unwanted emissions within RR-Appendix S3 is observed, it is widely recognised that a generic method is preferable for this purpose.

One of numerous reasons why generic methods are favoured is their a priori potential to treat new communication systems and technologies as they emerge. Other reason is that only generic method can aspire to become a basis for a widely recognised analysis tool.

The Monte-Carlo Radio Simulation tool described in this Report was developed, based on the above considerations, within the ERC (European Radiocommunication Committee) process.

SEAMCAT®

SEAMCAT® (Spectrum Engineering Advanced Monte-Carlo Analysis Tool) is the implementation of a Monte-Carlo Radio simulation model developed by the group of CEPT Administrations, ETSI members and international scientific bodies. SEAMCAT is public object code software distributed by the CEPT European Radiocommunications Office (ERO), Copenhagen. The address is as follows:

http.//www.ero.dk

The software is also available in the ITU-R software library. Further details can be provided by the SEAMCAT Management Committee (SMC), which can be contacted via the ERO, email: ero@ero.dk. The source code is only available for members of SEAMCAT MoU.

INDEX TABLE

1	BACKGROUND	1

2	MONTE-CARLO SIMULATION METHODOLOGY: AN OVERVIEW	1

2.1	Illustrative example (only unwanted emissions, most influential interferer)	2

3	ARCHITECTURE REQUIREMENTS	3

3.1	Event Generation Engine	4

3.2	Distribution Evaluation Engine	5

3.3 	Interference Calculation Engine	5

3.4 	Limits Evaluation Engine	6

ANNEX 1: List of input parameters	8

ANNEX 2: Event Generation Engine	10

Appendix 1 to Annex 2: (Reference (3)) : Propagation model	20

Appendix 2 to Annex 2: Power control function	33

Appendix 3 to Annex 2: Distribution definitions	34

Appendix 4 to Annex 2: Pseudo-random number generaton (References (3, 4))	35

Appendix 5 to Annex 2: dRSS calculation flow chart	37

Appendix 6 to Annex 2: irss due to unwanted and blocking calculation	38

Appendix 7 to Annex 2: Receiver Blocking	39

Appendix 8 to Annex 2: iRSS due to intermodulation	41

Appendix 9 to Annex 2: Intermodulation in the Receiver	42

Appendix 10 to Annex 2: Influence of different bandwidths	44

Appendix 11 to Annex 2: Radio cell size in a noise limited network	47

Appendix 12 to Annex 2: Symmetric antenna pattern	48

ANNEX 3: Distribution Evaluation Engine	50

Appendix  1 to Annex 3: Chi-Squared Goodness-of-Fit Test	53

Appendix 2 to Annex 3: Kolmogorov-Smirnov test of stability	54

ANNEX 4: Interference calculation engine	55

# 1	BACKGROUND

In order to reassess the limits for unwanted emissions within Appendix S3 of the Radio Regulations, it is desirable to develop an analytical tool to enable us to evaluate the level of interference which would be experienced by representative receivers. It has been agreed in the ITU-R that level of interference should be expressed in terms of the probability that reception capability of the receiver under consideration is impaired by the presence of an interferer. To arrive at this probability of interference, statistical modelling of interference scenarios will be required and this Report describes the methodology and offers a proposal for the tool architecture.

The statistical methodology described here and used for the tool development is best known as Monte-Carlo technique. The term "Monte-Carlo" was adopted by von Neumann and Ulan during World War II, as a codename for the secret work on solving statistical problems related to atomic bomb design. Since that time, the Monte-Carlo method has been used for the simulation of random processes and is based upon the principle of taking samples of random variables from their defined probability density functions. The method may be described as the most powerful and commonly used technique for analysing complex statistical problems. Monte-Carlo approach is seen not to have an alternative in development of a methodology for analysing unwanted emission interference.

The approach is:

- generic - A diversity of possible interference scenarios can be handled by single model.

- flexible - The approach is very flexible, and may be easily devised in a such way to handle the composite interference scenarios.

# 2	MONTE-CARLO SIMULATION METHODOLOGY: AN OVERVIEW

This methodology is appropriate to be used to address the following items in spectrum engineering:

- Sharing and compatibility studies between different radio systems operating in the same or adjacent frequency bands, respectively;

- Evaluation of transmitter and receiver masks;

- Evaluation of limits for parameters such as unwanted (spurious and out-of-band), blocking or intermodulation levels.

The Monte-Carlo method can address virtually all radio-interference scenarios. This flexibility is achieved by the way the parameters of the system are defined. The input form of each variable parameter (antenna pattern, radiated power, propagation path,…) is its statistical distribution function. It is therefore possible to model even very complex situations by relatively simple elementary functions. Number of diverse systems can be treated, such as:

- broadcasting (terrestrial and satellite);

- mobile (terrestrial and satellite);

- point-to-point;

- point-to-multipoint, etc.

The principle is best explained on the following example, which considers only unwanted emissions as the interfering mechanism. In general the Monte-Carlo method addresses also other effects present in the radio environment such as receiver blocking and intermodulation.

Some examples of applications of this methodology are:

- Compatibility study between digital PMR (TETRA) and GSM at 915 MHz;

- sharing study between FS and FSS;

- sharing study between short range devices (Bluetooth) and RLANs in the ISM band at 2.4 GHz;

- compatibility study for IMT-2000 and PCS1900 around 1.9 GHz;

- Compatibility study for ultra wideband systems and other radio systems operating in this frequency bands.

## 2.1	Illustrative example (only unwanted emissions, most influential interferer)

For interference to occur, it has been assumed that the minimum carrier-to-interference ratio (C/I) is not satisfied at the receiver input. In order to calculate the C/I experienced by the receiver, it is necessary to establish statistics of both the wanted signal and unwanted signal levels. Unwanted emissions considered in this simulation are assumed to result from active transmitters. Moreover, only spurii falling into the receiving bandwidth have been considered to contribute towards interference. For the mobile to fixed interference scenario, an example is shown in Figure 2.1.

Many potential mobile transmitters are illustrated. Only some of the transmitters are actively transmitting and still fewer emit unwanted energy in the victim receiver bandwidth. It is assumed that interference occurs as a result of unwanted emissions from the most influent transmitter with the lowest path loss (median propagation loss + additional attenuation variation + variation in transmit power) to the receiver.

An example of Monte-Carlo simulation process as applied to calculating probability of interference due to unwanted emission is given in Figure 2.2. For each trial, a random draw of the wanted signal level is made from an appropriate distribution. For a given wanted signal level, the maximum tolerable unwanted level at the receiver input is derived from the receiver's C/I figure.

For the many interferers surrounding the victim, the isolation due to position, propagation loss (including any variations and additional losses) and antenna discrimination is computed. The lowest isolation determines the maximum unwanted level which may be radiated by any of the transmitters during this trial.

From many trials, it is then possible to derive a histogram of the unwanted levels and for a given probability of interference, then to determine the corresponding unwanted level.

By varying the values of the different input parameters to the model and given an appropriate density of interferers, it is possible to analyse a large spectra of interference scenarios.

Figure 2.1: An example of interference scenario involving TV  receiver and portable radios

- Figure 2.2: An example formulation of the Monte-Carlo evaluation process

# 3	ARCHITECTURE REQUIREMENTS

One of the main requirements is to select such an architectural structure for the simulation tool which would be flexible enough to accommodate analysis of composite interference scenarios in which a mixture of radio equipment sharing the same habitat and/or multiple sources of interference (e.g. out-of-band emission, spurious emission, intermodulation, ..) are involved and can be treated concurrently.

Other requirements would be that the proposed architecture consists of modular elements and is versatile enough to allow treatment of the composite interference scenarios.

The proposed Monte-Carlo architecture which meets these constraints is presented in Figure 3.1. The proposed architecture is basically of a sequential type and consists of four processing engines:

•	event generation engine;

•	distribution evaluation engine;

•	interference calculation engine;

•	limits evaluation engine.

The schematic view on the entire tool is in Figure 3.1.

Figure 3.1: Architecture of the Simulation Tool

The list of interference parameters and their relevance to one or more of the processing engines is shown in Annex 1.

## 3.1	Event Generation Engine

The Event Generation Engine (EGE) takes the relevant parameters from the submitted interference scenario and generates information on the received signal strength of the desired as well as on the strength for each of the interfering signals included in the composite interference scenario. This process is repeated N times, where N is a number of trials which should be large enough to produce statistically significant results. Generated samples of the desired and all interfering signals are stored in separate data arrays of the length N.

The trials on parameters being common for desired and interfering radio paths are done concurrently in order to capture possible correlation between desired and interfering signals. Such an implementation will not cover only those seldom cases of interference in which one interference mechanism is excited by another interference (e.g. a strong transmitter mixes with spurious emission of the second transmitter and produces an intermodulation type of interference).

The flow chart description and detailed algorithm description for the EGE are presented in Annex 2.

List of potential sources of interference to be found in a radio habitat includes:

- transmitter interference phenomena;

- unwanted (spurious and out-of-band) emissions;

- wideband noise;

- intermodulation;

- adjacent channel;

- co-channel.

Receiver interference phenomena:

- spurious radiation.

Background noise:

- antenna noise;

- man-made noise.

Other receiver interference susceptibility parameters:

- blocking;

- intermodulation rejection;

- adjacent and co-channel rejections;

- spurious response rejection.

All of the above sources can be classified into three generic interference mechanism categories: unwanted emission, intermodulation and receiver susceptibility. Each of the above three categories requires a different model for physical processes being characteristic for that interfering mechanism. The man-made noise and the antenna temperature noise can be considered as an increase of the thermal noise level, decreasing thus the sensitivity of a receiver, and can be entered in the simulation when the criteria of interference is I/N (interference-to-noise ratio) or C/(I+N) (wanted signal-to-interferer+noise).

## 3.2	Distribution Evaluation Engine

The Distribution Evaluation Engine (DEE) takes arrays of the data generated by the EGE and processes the data with the aim of:

- assessing whether or not the number of samples is sufficient to produce statistically stable results;

- calculating correlation between the desired signal and interfering signal data and between different types of the interfering signals (e.g. blocking vs. unwanted emissions);

- calculating a known "continuous" distribution function, e.g. Gaussian, as the best fit to the generated distributions of the desired and interfering signal data.

The first and third of the above points can be achieved using well known goodness-of-fit algorithms for general distributions such as the Kolmogorov-Smirnov test. Applicability of the «fit» to this specific task is to be further investigated in the planned phase 2 of the development of the methodology.

If DEE detects unacceptable variation in discrete distribution parameters estimated in two successive estimations using N and N+ΔN sample sizes, the EGE is instructed to generate another ΔN of additional samples. This test is repeated until a tolerable variation of the parameters is measured over the pre-defined number of successive tests.

Three different kinds of outputs are possible from the DEE engine:

- data arrays of the wanted and interfering signals. This is the output in the case that a high degree of correlation is detected between the wanted and any of the interfering signals.

- discrete distributions of the wanted and interfering signals are passed in the case of a weak correlation between the signals or in the case that there was no correlation between the signals but no "continuous" distribution approximation with satisfactory accuracy was possible.

- continuous distribution functions of the wanted and interfering signals are passed to ICE in the case that signals were de-correlated and discrete distributions were successfully approximated with continuous distribution functions.

The proposed flow chart and detailed algorithm specification are presented in Annex 3.

## 3.3 	Interference Calculation Engine

The Interference Calculation Engine (ICE) is the heart of the proposed architecture. Here, information gathered by the EGE and processed by DEE are used to calculate probability of interference. Depending on which kind of information was passed from DEE to ICE, three possible modes of calculating the probability of interference are identified, as shown in Annex 4.

Mode 1: data arrays for dRSS (wanted signal) and inRSS (interfering signal resulting from n different systems) passed by DEE to the ICE, and vector representing the composite interfering signal I is calculated as a sum of the inRSS data vectors.

Mode 2: distribution function for the composite interfering signal is calculated by taking random samples for inRSS distributions and linearly adding them up.

Mode 3: the inRSS is calculated using numerical or analytical integration of the supplied distribution functions for each of the interference sources.

Mode 4: all signals are assumed to be mutually independent and the overall probability for interference is identified as the probability to be disturbed by at least one kind of interference.

Different criteria for calculation of interference probability can be accommodated within the processing engine. A cumulative probability functions can be calculated for CI, C(N+I), IN or N(N+I) random variables.

The flow of information together with associated processes is shown in the form of a flow chart in Annex 4.

All interfering signal distributions are calculated with respect to reference levels or functions of unwanted (emission mask), blocking (receiver mask) or intermodulation attenuation. Interfering signal distributions for some other reference levels or functions can be derived by first order (unwanted or blocking) or third order (intermodulation) linear translation of the reference distributions (see Annex 4).

## 3.4 	Limits Evaluation Engine

The Limits Evaluation Engine (LEE) is envisaged to play a very important role in two aspects of the tool development:

- selection of optimal values for the limits;

- verification of the tool.

Output from the Interference Calculation Engine (ICE) is presented as a multi-dimensional surface characterising the dependence of the probability of interference versus the radio parameters. Two main features of the probability surface are:

- the same probability of interference is achieved by different sets of the limit values for the radio parameters under consideration;

- probability of interference parameter is not used in the radio system design and as such does not lend itself nicely for the validation through the system performance measurements. Instead, degradation in system coverage or traffic capacity seems to be more appropriate for understanding impact of a particular probability of interference to the radio system performance.

The radio variables are transformed from the probabilistic space into a system performance space enabling us to evaluate the system performance degradation due to presence of interference. When the inter-system compatibility is analysed (e.g. unwanted emission), radio coverage and/or traffic capacity can be used to evaluate the impact of the radio parameters limits. For the case of intra-system compatibility study (e.g. out-of-band emission), spectrum efficiency should be used to derive appropriate values for the radio parameters.

The limit values are derived by means of an optimisation algorithm. For optimisation to work a criteria needs to be set. The criteria is usually termed the «cost» function and the optimisation process has for a task to minimise this «cost» function. The «cost» function is a function of all radio parameters and their significance to the «cost» can be altered by means of the weight coefficients.

The weight coefficients can integrate any of the following aspects into optimisation process:

- system availability;

- traffic capacity;

- spectrum utilisation;

- technological limitations;

- economic constraints.

The set of radio parameters values for which the «cost» function is minimised represents the optimal solution for the limit values.

The role of LEE is very important within the tool. However, since its various elements are still under consideration, it will not be possible to include LEE into the first phase of the implementation.

### ANNEX 1

### List of input parameters

The following rules are applied:

- a capital letter is used for a distribution function, e.g. P;

- a small letter is a variable (result of a calculation or a trial), e.g. p;

- the index refers to a "player": wanted transmitter, victim receiver, wanted receiver and interfering transmitter.

Parameters for the wanted transmitter:

Pwtsupplied	=	power level distribution for various transmitters

pwtsupplied	=	sample power level taken from the above distribution (in dBm)

gwtmax	=	maximum antenna gain (in dBi)

patternwt=	antenna directivity within operating bandwidth (in dB) (supplied as a function or a look-up table)

Hwt	=	antenna height distribution (in 1/m)

Rwtmax	=	radius of the wanted transmitter coverage (in km), (not required for point-to-point)

Parameters for the victim receiver:

CI		=	protection ratio (in dB)

gvrmax		=	maximum antenna gain (in dBi)

patternvr	=	antenna directivity within operating bandwidth (in dB) (supplied as a function or a look-up table)

Hvr	=	antenna height distribution (in 1/m)

block		=	receiver frequency response (in dB)

avr		=	receiver susceptibility characteristic is expressed as a ratio between desired interfering signal

levels producing unacceptable receiver performance and is n as a function of frequency

separation between the two signals

intermod	=	receiver intermodulation response (in dB)

The intermodulation response is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of two unwanted signals with a specific frequency relationship to the wanted signal frequency.

fvr		=	frequency (in MHz)

sensvr	=	sensitivity of victim receiver (in dBm)

b		=	bandwidth of victim receiver (in kHz)

Parameters for the interfering transmitter:

Pitsupplied	=	power level distribution of various transmitter

pitsupplied	=	sample power level taken from the above distribution (in dBm)

pitthold	=	power control threshold (in dBm)

pitdycrg	=	power control dynamic range (in dB)

pitstrg	=	power control step range (in dB)

gitmax	=	maximum antenna gain (in dBi)

Ritmax	=	radius of the interfering transmitter coverage (in km)

Rsimu		=	radius of the area where interferers are spread (in km)

d0		=	minimum protection in distance (in km) between the victim receiver and interfering  				transmitter

patternit	=	antenna directivity (in dB)  (supplied as a function or a look-up table)

emissionrelit	=	relative emission mask (in dBc/(reference bandwidth)) only used for interferer and consists of the wanted signal level and all unwanted emissions including part of emission floor depending on the power control.

emissionfloorit = absolute emission floor (in dBm/(reference bandwidth)) only used for interferer (unwanted emissions which would be emitted with the lowest possible power of the transmitter)

Note that up to Version 1.1.5 of SEAMCAT the reference bandwidth of the floor is fixed to 1 MHz.

fit	=	frequency (in MHz)

densit	=	density (in 1/km²)

pittx	=	probability of transmission (in %), which is a statistical description of the transmitter activities

averaged over a large number of users and long period of time

tempit	=	normalised temporal activity variation function of time of the day (in 1/h)

Parameters for the wanted receiver belonging to the interfering transmitter:

gwrmax		=	maximum antenna gain (in dBi)

patternwr	=	antenna directivity (in dB)   (supplied as a function or a look-up table)

Hwr		=	antenna height distribution (in 1/m)

senswr		=	dynamic sensitivity of the wanted receiver, taking into account margin for the fast-fading and

intra-system interference (in dBm)

Environmental and Propagation Parameters

fpropag		= 	propagation law (median loss + variation) (given in Appendix 1 to Annex 2)

fmedian		= 	propagation law (median loss only) (given in Appendix 1 to Annex 2)

env		= 	environment type (indoor/outdoor, urban/suburban/open area)

### ANNEX 2

### Event Generation Engine

Introduction

N – array vectors

N

In this Annex it is described how to construct signals that are used in the interfering scenarios: the desired signal and the interfering signals due to unwanted emission, blocking and intermodulation. The calculated signals are the stored in an array which serves as input to the DEE as shown in Figure 1.

Figure 1: General flow chart of the EGE

Inputs

The input parameters are defined in Annex 1. The different players are shown in Figure 2.

Outputs

dRSS	=	desired Received Signal Strength (in dBm)

iRSSspur	=	interfering Received Signal Strength due to unwanted emissions (in dBm)

iRSSblocking	=	interfering Received Signal Strength due to blocking (in dBm)

iRSSintermod	=	interfering Received Signal Strength due to intermodulation (in dBm)

wr1

it1

wr2

it2

wrn

itn

vr

wt

Figure 2: Different players participating in the EGE

Calculation

In this section,

- T represents a trial from a given distribution (algorithm described in Appendix 4).

- Distributions U(0,1), G(σ) and R(σ) are defined in Appendix 3.

- Flow charts of dRSS and iRSS calculation are given respectively in Appendixes 5, 6, 8.

NOTE - distances d between transmitters and receivers are applied with the unit in km.

a)	dRSS calculation

There are 3 different choices to determine dRSS : depending on a variable distance, for a fixed distance or using a given signal distribution (see Appendix 5).

- Case of variable distance:

dRSS=f(pwtsupplied,gwt→vr,plwt↔vr,gvr→wt)=pwtsupplied+gwt→vr(fvr)−plwt↔vr(fvr)+gvr→wt(fvr)

If the received signal can not exceed a given value (i.e. if depending on the power control implemented in the victim system) then:

dRSS=min(dRSS, dRSSmax) 		using dRSS as calculated before

where:

- fvr=	frequency received in the victim receiver.

fvr=T(fvr)	This frequency can be set constant or determined by a certain distribution, e.g. the "discrete frequency distribution" (see Appendix 3). In general, the victim frequency should not be fixed but should computed and randomly chosen as the interferer frequency using a discrete distribution (see also section b)).

Note that this feature is available for later version of SEAMCAT than 1.1.5, only.

- Pwtsupplied	=	maximum power level distribution supplied to the wanted transmitter antenna

pwtsupplied=T(Pwtsupplied)

- plwt↔vr	=	path loss between the wanted transmitter and the victim receiver (propagation

loss, slow fading and clutter losses taken into account). Depending on whether the criteria of interference will apply to the instantaneous dRSS (Rayleigh fading excluded) or to the mean(dRSS):

plwt↔vr=fpropag(fvr,hvr, hwt, dwt↔vr, env)

or

plwt↔vr=fmedian(fvr,hvr, hwt, dwt↔vr, env)

where:

hvr	=	victim receiver antenna height

hvr=T(Hvr)

e.g. hvr=T(U(hvrmin ,hvrmax))=hvrmin+(hvrmax−hvrmin) T(U(0,1))

hwt	 =	wanted transmitter antenna height

hwt=T(Hwt)

e.g. hvr=T(U(hwtmin ,hwtmax))=hwtmin+(hwtmax−hwtmin) T(U(0,1))

dwt↔vr =		distance between the victim receiver and the wanted transmitter

dwt↔vr=T(Rmaxwt)	e.g. dwt↔vr=RmaxwtT(U(0,1))

Three different choices for Rmaxwt are considered:

1)	Given distance Rmaxwt

2)	Noise limited network

Rmaxwt is determined by the following equation:

fmedian(fvr,hvr, hwt, dwt↔vr, env)+fslowfading(X%)=Pwtsupplied+gwtmax+gvrmax−sensvr

fmedian	= 	propagation loss not including slow fading

fslowfading(X%) = 	fading margin to be used for 1-X% coverage loss.

In the case of lognormal fading and a 95% coverage loss at the edge of the coverage, for large distances, the value fslowfading is well known 1.64 times the standard deviation of the propagation loss. Further details of the determination of the radio cell size in a noise limited network are given in Appendix 11.

- Traffic limited network

Rmaxwt=nchannels nuserperchannelπ densmaxclusterfrequency

- gwt→vr	 = wanted transmitter antenna gain in the victim receiver direction

gwt→vr=f(gwtmax ,patternwt)=gwtmax patternwt(θwt→vr, ϕwt→vr,fvr)

where:

(θwt→vr, ϕwt→vr)= 	azimuth and elevation angles between the top of the wanted transmitter antenna and the

top of the victim receiver antenna
e.g.: θwt→vr=T(U(0, 2π))=2π T(U(0,1))

ϕwt→vr=TU−π2,π2=π T(U(0, 1))−π2

The computation of the gain for symmetric antenna patterns see Appendix 12.

- gvr→wt = victim receiver antenna gain in the wanted transmitter direction

gvr→wt=f(gvrmax ,patternvr)=gvrmax patternvr(θwt→vr+π, −ϕwt→vr,fvr)

- Case of fixed distances:

Pwtnominal	=	nominal power distribution

ffading,fixed link	= 	fading distribution

dRSS=f(Pwtnominal,ffading,fixed link)=T(Pwtnominal)−T(ffading,fixed link)

- Case of given dRSS: distribution to be given by the user

- iRSSblock calculation

iRSSblock=j=1ninterferersf(pitsupplied,gitPC,git→vr,plit↔vr,avr,gvr→it)j=10logj=1ninterferers10iblockj10

where the j-th interferer signal is given by

iblockj=(pitsupplied+gitPC+git→vr(fit)−plit↔vr−avr+gvr→it(fit))j

where for each interferer:

- fit		= 	interferer transmitting frequency

fit=T(fit)	For the "discrete frequency distribution see Appendix 3.

Note that it is clear that the trial of the dRSS frequency, fvr , occurs once and only once on each simulation round, i.e. fvr is tried once as the wanted victim positions, the wanted transmit power, and other distributions pertaining to the victim link. These values then tried from the dRSS-distributions apply to >N trials of iRSS (where N is the number of interferers).

If randomness of some parameters could be limited, than SEAMCAT could not be used also for simulation only, but also for more exact calculations. This feature would allow an easier check of the validity of the simulation results.

- Pitsupplied	= 	maximum power supplied to the interfering transmitter antenna (before power control)

pitsupplied=T(Pitsupplied)

- gitPC=fpc(pitsupplied,git→vr,plit↔vr,gvr→it,pcitt_hold,pcitdyc_rg,pcitst_rg)gicPC		= 	power control gain for the interfering transmitter

where:

fpc	 = 	power control function (given in Appendix 2)

plit↔wr	= 	path loss between the interfering transmitter and the wanted receiver (propagation loss, slow fading and clutter losses taken into account). Depending on the power control implementation, this can be either mean path loss or instantaneous path loss (Rayleigh fading excluded)

plit↔wr=fpropag(fit, hwr, hit, dit↔wr,env)+fclutter(env)

or

plit↔wr=fmean(fit, hwr, hit, dit↔wr,env)+fclutter(env)

where:

hwr = antenna height of wanted transmitter

hwr=T(Hwr)

e.g. hwr=T(U(hwrmin ,hwrmax))=hwrmin+(hwrmax−hwrmin) T(U(0,1))

hit = interfering transmitter antenna height

hit=T(Hit)

e.g. hit=T(U(hitmin ,hitmax))=hitmin+(hitmax−hitmin) T(U(0,1))

dit↔wr = distance between the interfering transmitter and the wanted receiver

dit↔wr=T(Rmaxit)	e.g. dit↔wr=RmaxitT(U(0,1))

Three different choices for hwr=T(U(hwrmin,hwrmax))=hwrmin+(hwrmax−hwrmin) x T(U(0,1)) are made:

1)	Given distancehwr=T(U(hwrmin,hwrmax))=hwrmin+(hwrmax−hwrmin) x T(U(0,1))

2) 	Noise limited network

- Traffic limited network.

For further details of the cell size determination see section a).

- git→wr	=	interfering transmitter antenna gain in the direction of the closest base station

git→wr=f(gitmax ,patternit)=gitmax patternit(θit→wr, ϕit→wr,fit)

where

(θit→wr, ϕit→wr)=	azimuth and elevation angles between the top of the interfering transmitter antenna and the top of the wanted receiver antenna

e.g.: 	θit→wr=T(U(0, 2π))=2π T(U(0,1))

ϕit↔wr=TU−π2,π2=π T(U(0, 1))−π2

The computation of the gain for symmetric antenna patterns see Appendix 12.

- gwr→it	 = base station antenna gain in the interfering transmitter direction

gwr→it=f(gwrmax ,patternwr)=gwrmax patternwr(θit→wr+π, −ϕit→wr,fit)

- plit↔vr	=	path loss between the interfering transmitter i and the victim receiver (propagation loss, slow

fading and clutter losses taken into account).

plit↔vr=fpropag(fit,hvr, hit, dit↔vr, env)

or

plwt↔vr=fmedian(fvr,hvr, hwt, dwt↔vr, env)

The choice between fmedian and fpropag would depend on the criteria of interference, and is closely related to the choice made for assessment of dRSS, e.g. whether ICE will evaluate:

dRSSmeaniRSSmean; dRSSpropagiRSSpropag; dRSSmeaniRSSpropag;

where:

hvr	 = 	victim receiver antenna height (defined in the dRSS calculation)

hit	 =	interfering transmitter antenna height (defined previously)

dit↔vr 	= 	distance between the victim receiver and the interfering transmitter

Three different ways to choose dit↔vr:

1)	The most common case is when there is no spatial correlation between the elements of the victim system and the elements of the interfering system.

Then dit↔vr is a result of a trial:

dit↔vr=RsimuT(U(0,1))

Rsimu	= radius of the area where interferers are spread

Rsimu=nactiveπ densitactive

where:

nactive = 	number of active interferers considered in the simulation

nactive	should be sufficiently large so that the n+1 interferer would bring a negligible additional interfering power.

densitactive=densit pittx tempit(time)

If a minimum protection, dit→vr≥d0 between the victim receiver and interfering transmitter is introduced then Rsimu results in

Rsimu=nactiveπ densitactive+d02.

Note that each trial of dit→vr<d0 has to be rejected and repeated for another trial dit→vr≥d0.

Note that if the protection distance d0>0 then a uniform distribution of the interfering transmitter has to be chosen.

- This case deals with the situation where the victim system and the interfering system are geographically correlated (e.g. co-located base stations).

This correlation is assumed to be only between one element (victim or wanted transmitter) of the victim system and one element (interferer or wanted receiver) of the interfering system.

A trial (if the distance is not fixed) of the distances and angles between the two correlated elements is made

(e.g. dwr↔vr, θwr↔vr). The knowledge of dit↔wr, dvr↔wt, θit↔wr, θvr↔wt enables to derive the missing co-ordinates (e.g. dit↔vr, θit↔vr).

it

vr

wt

wrr

rr

Figure 3: Interfering scenario with a geographical correlation between the victim and the interfering systems

- Closest interferer

The influence of the closest interferer can be estimated by having a distance dit-vr following a Rayleigh distribution R(σ) defined in Appendix 3 and where the parameter σ is related to the density of transmitters. This is an alternative method for calculating the relative location of the interfering transmitter respect to the victim receiver in non correlated mode which should avoid to perform multiple trials on the number of interferers.

In this case the distribution for the distance between it and vr in the simulation area is always a Rayleigh distribution:

dit→vr=Rsimu×Rσ

where standard deviation  is related to the density of active transmitters:

σ=12πdensitactive

Note that the simulation radius is useless but associated parameters (density, activity and probability) are still required for calculation of the density of active transmitters.

densitactive=densit.pit.activity

- git→vr(fit)	 = interfering transmitter antenna gain in the victim receiver direction

git→vr=f(gitmax ,patternit)=gitmax patternit(θit→vv, ϕit→vr,fit)

where:

(θit→vr, ϕit→vr)	=	azimuth and elevation angles between the top of the 
 		closest interfering transmitter antenna and the top of the victim receiver antenna

e.g.: 		θit→vr=T(U(0, 2π))=2π T(U(0,1))

ϕit↔vr=TU−π2,π2=π T(U(0, 1))−π2

- avr(fit,fvr) = attenuation of the victim receiver

Three possible ways are considered for calculating this attenuation:

1)	avr is given by the user

2)	Blocking is given in terms of blocking attenuation or protection ratio. For a wanted signal 3 dB above the sensitivity, the attenuation avr can be derived from the following equation (see Appendix 7):

avr=fCN+I,blockatt=3+CN+I+blockatt(fit,fvr)

3)	blocking is given in terms of absolute level of blocking:

avr=fCN+I,blockabs=CN+I+blockabs(fit,fvr)−sensvr

Two cases are envisaged:

1)	block is a mask which is a function of Δf=(fit−fvr). It is introduced to enable calculations of interference between systems in adjacent band.

2)	block is a fixed value (e.g. 80 dBm). It is used to derive generic limits.

- gvr→it(fit)	= victim receiver antenna gain in the interfering transmitter direction

gvr→it=f(gvrmax ,patternvr)=gvrmax patternvr(θit→vr, ϕit→vr,fit)

c)	iRSSspur calculation

iRSSspur=f(emissionit,git→vr,plit→vr,gvr→it)==10logj=1ninterferers10ispurj10

where the j-th interferer signal is defined as

ispurj=(emissionit(fit,fvr)+git→vr(fvr)−plit→vr(fvr)+gvr→it(fvr))

Most of the parameter are already defined either in section a) or b).

- emissionit(fit, fvr)= emission mask by the interfering transmitter which generally

depends on the relative emission mask, the interfering power, the gain power control and the bandwidth of emission majored by the absolute emission floor. For further details and the influence of different bandwidths of the wanted and interfering radio systems see Appendix 10.

emissionit(fit,fvr)=maxpitsupplied+emissionrelit(fit,fvr)+gitPC, emissionfloorit(fit,fvr)

- emissionrelit is a relative emission mask which is a function of Δf=(fit−fvr). It is introduced to enable calculations of interference between systems in the same or adjacent bands. The real emission is always greater or equal than the absolute emission floor emissionfloorit(fit,fvr).

- gitpc 	=	power control gain for the interfering transmitter (defined in section b))

- plit↔vr	=	path loss between the interfering transmitter and the victim receiver  (propagation loss, slow fading and

clutter losses taken into account)

plit↔vr=fpropag(fvr, hvr, hit, dit↔vr, env)+fclutter(env)

where:

hvr	 = 	victim receiver antenna height (defined in dRSScalculation)

hit	 = 	interfering transmitter antenna height

dit↔vr	 = 	distance between the victim receiver and the interfering transmitter

- git→vr(fvr) = interfering transmitter antenna gain in the victim receiver direction.

git→vr(fvr)=(gitmax, patternit)=gitmax patternit(θit→vr , ϕit→vr ,fvr)

where:

(θit→vr , ϕit→vr) = 	azimuth and elevation angles between the top of the closest interfering transmitter antenna and the top of the victim receiver antenna (defined in section b)

- gvr→it(fvr) = victim receiver antenna gain in the interfering transmitter direction.

gvr→it(fvr)=(gvrmax, patternvr)=gvrmax patternvr(θvr→it+π , −ϕvr→it ,fvr)

d)	iRSSintermod calculation

iRSSintermod=f(pit,ksupplied,git,kpc,git,k→vr,plit,k→vr,gvr→it,k,sensvr,intermod) with k=i,j

=10logi=1nj=1,j≠in10ii,jRSSintermod10

where:

- ii,jRSSintermod= Intermodulation product of third order at the frequency f0

ii,jRSSintermod=2iRSSint+ijRSSint−3intermod−3sensvr−9 dB.

The interferer i transmits at the frequency fit,i=fit and the interferer j at the frequency fit,j (see section b), which defines Δf=fit,j−fit, and yields f0=fit−Δf=2fit−fit,j. Assuming an ideal filter (roll off factor 0) the intermodulation product has to be considered only for the bandwidth b

fvr−b2 ≤ f0≤fvr+b2

For all other cases the intermodulation product can be neglected.

- ikRSSint 	= 	received power in the victim receiver due to interferer k=i at fit or

interferer k=j at fit,j.

ikRSSint=pit,ksupplied+git,kpc+git,k→vr−plit,k↔vr+gvr→it,k

The various parameters are defined in the previous sections a) - c). For the computation of iiRSSint the same algorithms as given in Appendix 6 can be used because iiRSSint corresponds to iiRSSblock+avr(fit, fvr).

- intermod 	= 	receiver intermodulation response for a wanted signal 3 dB above the sensitivity.

Two cases are envisaged:

1) 	intermod is given by the user, e.g. typical values are 70 dB for base station equipment and 65 dB for mobile and handportable equipment. It is used to derive generic limits.

2) 	intermod(Δf) is measured as a function of Δf referred to fvr (see Appendix 9)

- sensvr	 = 	sensitivity of victim receiver

#### Appendix 1 to Annex 2: (Reference (3)) : Propagation model

A number of propagation models are provided in the tool. They are depending on the environment chosen for the scenarios:

- general environment: open area, suburban or urban area;

- environment for the interferers: indoor or outdoor;

- environment for the victim receiver: indoor or outdoor.

The domain of validity for the models is described in the table below:

| Below 30 MHz | No model available. Curves of Recommendation ITU-R P.368 is suited for high power transmitters and large distances and is therefore not adapted to interference calculations. |
| --- | --- |
| Greater 30 MHz | Free space model: L[dB]=32.5+20log(f[MHz])+20log(d[km]) Mandatory condition for the use is free line-of-sight, i.e. the first Fresnel zone has to be clear! |
| Between 30 MHz and 3 GHz | Modified Hata model available for outdoor-outdoor path loss calculations. Care should be taken when propagation distances are expected to be above 20 km. Indoor-indoor and indoor-outdoor models also suitable. For broadcasting the propagation model provided by Recommendation ITU-R P.1546 is implemented. |
| Above 3 GHz | Modified Hata model not advised. Spherical diffraction model is suitable for open area environment and point-to-point. No model available for suburban and urban environment. Indoor-indoor and indoor-outdoor models also suitable. |

To improve the flexibility of the tool, a "generic" model, e.g. L=A+Blog(d)+C d, both for the wanted signal path and the interfering path d can also be entered by the user. The user of the tool is then to enter the parameters A, B, C of the median attenuation formula and the distribution of the variation in path loss Dv. As a default distribution, a lognormal distribution is to be proposed with a standard deviation to be entered by the user. Then we have for the path loss pl:

pl=fpropag(d)=L+T(Dv)

a) User-defined propagation model, e.g. free space model

The free space path loss is implemented in SEAMCAT:

L[dB]=32.5+20log(f[MHz])+20log(d[km])

Also, more elaborate models can be implemented by the user using a simple script. As an example for the notation in the User defined propagation model, the free space path loss considering the difference in antenna height is denoted:

L1 = 32.5;

L2 = 20 * log10(freq());

L3 = 10 * log10(dist()*dist()+(hrx()*hrx()+htx()*htx())/1000000);

L = L1 + L2 + L3;

eval L.

b) Modified Hata model

pl=fpropag(f, h1, h2, d, env)=L+T(G(σ))

where:

L	= 	median propagation loss (in dB)

	= 	standard deviation of the slow fading distribution (in dB)

f	= 	frequency (in MHz)

Hm=minh1, h2

Hb=maxh1, h2

d	=	distance (in km), preferably less than 100 km.

env	=	(outdoor/outdoor), (rural, urban or suburban), (propagation above or below roof).

If Hm and/or Hb are below 1 m, a value of 1 m should be used instead. Antenna heights above 200 m might also lead to significant errors. Propagation below roof means that both Hm and Hb are above the height of roofs. Propagation is above roof in other cases (Hb above the height of roofs).

Calculation of the median path loss L

Case 1:  d  0.04 km
	L=32.4+20log(f)+10log(d2+(Hb−Hm)2106)

Case 2:  d  0.1 km
	a(Hm)=(1.1 log(f)−0.7) min10,Hm−(1.56 log(f)−0.8)+max0, 20 log(Hm10)
	b(Hb)=min0, 20 log(Hb30)

Note that in the "SE24-model" for short range devices in the case of low base station antenna height,

Hb, b(Hb)=min0, 20 log(Hb30) is replaced b(Hb)=(1.1 log(f)−0.7) min10,Hb−(1.56 log(f)−0.8)+max0, 20 log(Hb10)
α= α=1                                                                                                   d≤20 km            α=1+0.14+0.000187 f+0.00107 Hblogd200.8              20 km<d≤100 km

Sub-case 1: Urban

•	30 MHz < f  150 MHz
L=69.6+26.2log(150)−20log(150f)−13.82log(max30, Hb)+ 44.9−6.55log(max30, Hb)(log(d))α−a(Hm)−b(Hb)

•	150 MHz < f  1500 MHz
L=69.6+26.2log(f)−13.82log(max30, Hb)+ 44.9−6.55log(max30, Hb)(log(d))α−a(Hm)−b(Hb)

•	1500 MHz < f  2000 MHz
L=46.3+33.9log(f)−13.82log(max30, Hb)+ 44.9−6.55log(max30, Hb)(log(d))α−a(Hm)−b(Hb)

•	2000 MHz < f  3000 MHz
L=46.3+33.9log(2000)+10log(f2000)−13.82log(max30, Hb)+ 44.9−6.55log(max30, Hb)(log(d))α−a(Hm)−b(Hb)

Sub-case 2: Suburban
	L=L(urban)−2logminmax150, f, 2000282−5.4

Sub-case 3: Open area
L=L(urban)−4.78 logminmax150, f, 20002+18.33 logminmax150, f, 2000−40.94

Case 3:  0.040 km < d < 0.1 km
L=L(0.04)+log(d)−log(0.04)log(0.1)−log(0.04) L(0.1)−L(0.04)

When L is below the free space attenuation for the same distance, the free space attenuation should be used instead.

Assessment of the standard deviation for the lognormal distribution

Case 1:  d  0.04 km		σ=3.5 dB

Case 2:  0.040 km < d  0.1 km
σ=3.5+(12−3.5)(0.1−0.04) (d−0.04)   dB	for propagation above the roofs

σ=3.5+(17−3.5)(0.1−0.04) (d−0.04)  dB	for propagation below the roofs

Case 3:  0.1 km < d  0.2 km 
	σ=12  dB	for propagation above the roofs
	σ=17  dB	for propagation below the roofs

Case 4:  0.2 km < d  0.6 km
σ=12+(9−12)(0.6−0.2) (d−0.2)  dB	for propagation above the roofs

σ=17+(9−17)(0.6−0.2) (d−0.2)	  dB	for propagation below the roofs

Case 5:  0.6 km < d			σ=9 dB

c) Spherical diffraction model

The spherical propagation model is based on various ITU-R Recommendations P.452, P.676 and P.526.

According to ITU-R Recommendation P.452 the median loss between transmitter and receiver is given by the following equation:

Lbd(p)=92.5+20logf+20logd+Ld(p)+Ag

where:

Lbd(p)	is the basic loss in dB as function of the time percentage p in %

f	is the frequency in GHz

d	is the distance in km

Ld(p)	is the diffraction loss in dB as function of the time percentage p in %

Ag	is the attenuation due to atmospheric gas and water in dB

The Attenuation due to atmosphere is given by

Ag=γO(f)+γw(ρ,f) d

where:

γO(f)		linear attenuation due to dry air (oxygen) in dB/km

γw(ρ,f)	linear attenuation in dB/km due to water as function 
of the water concentration ρ in g/m³, default value: 3 g/m³

Both terms can be approximated by the following equations according to Recommendation ITU-R P.676:

- Attenuation due to water:
γw(ρ,f)=0.050+0.0021ρ+3.6(f−22.2)2+8.5+10.6(f−183.3)2+9+8.9(f−325.4)2+26.3 f2ρ 10−4
for f < 350 GHz

- Attenuation due to oxygen:
γO(f)=7.19 ⋅10−3+6.09f2+0.227+4.81(f−57)2+1.50 f2 10−3	f≤ 57 GHz
γO(f)=10.5+1.5 (f−57)	57 <f≤ 60 GHz
γO(f)=15−1.2 (f−60)	60 <f≤ 63 GHz
γO(f)=3.79 ⋅10−7 f+0.265(f−63)2+1.59+0.028(f−118)2+1.47 (f+198)2 10−3

f> 63 GHz

Note that for simplification a linear interpolation between 57 and 63 GHz is used. The maximum is 15 dB/km for 60 GHz.

According to ITU-R Recommendation P.526, the diffraction loss Ld(p) can be derived by the received field strength E referred to the free space E0:

−Ld(p)=20logEE0=F(X)+G(Y1)+G(Y2)

where:

X	is the normalized radio path between transmitter and receiver

X	Y1 is the normalized antenna height of the transmitter

Y2	is the normalized antenna height of the receiver

X=2.2β f 13 ae−23 d

Y=9.6 ⋅10−3β f23 ae−13 h

where:

β	is a parameter derived from the earth admittance factor K: β = 1 for f> 20 MHz.

f	is the frequency in MHz

ae 	is the equivalent earth radius in km (definition see below)

d	is the distance in km

hi	is the antenna height above ground in m with i=1 or 2 for the transmitter or receiver,
respectively.

The distance-dependent term F(X) is given by the semi-empirical formula:

F(X)=11+10log(X)−17.6X

The antenna height gain G(Y) is given by the formula set:

G(Y)=17.6(Y−1.1)12−5log(Y−1.1)−8		for Y>2

G(Y)=20log(Y+0.1Y3)				for 10K<Y<2

G(Y)=2+20logK+9log(YK)log(YK)+1	for K10<Y<10K

G(Y)=2+20logK					for Y<K10

where:

K	is the normalised Earth surface admittance factor (see Recommendation ITU-R P.526), default value: 10−5.

Note that the different units for the frequency used.

This variation in path loss is provided through the variability of the equivalent earth radius ae (unit: km) which is considered to be dependent on the time percentage p:

- ae(p)=6375 k(p)

with the Earth radius factor k(p) expressed as:

- k(p)=k50+(5−k50)(1.7−logp)(1.7−logβ0)	for p<50%

- k(p)=k50				for p>50%

- and

- k50=157157−ΔN

- where:

ΔN	is the mean gradient of the radio refraction profile over a 1 km layer of the atmosphere from the surface. The default value is 40 units/km for Europe (standard atmosphere). This value yields to k50≈43 and ae= 8500 km.
NOTE - The mean gradient is positive!

β0	is the existence probability (in %) of the super-refractive layer (ΔN> 100 units/km) in the low atmosphere. Default value: 1 % for Europe.

Note that the probabilities p and β0 are denoted in %, i.e. a range of variety: 0...100%.

Note that the default value p=50% is normally chosen constant. Small time percentages allow the simulation of anomalous propagation conditions. The following restrictions of application of this model are to be considered:

- The frequency range should be larger than 3 GHz, with caution lower frequencies may be used but not below 300 MHz due to the surface admittance and polarisation effects.

- The model was developed for open (rural) area. Therefore, the additional attenuation due to obstacles like

buildings found in suburban or urban environment is not included.

- The loss due to rain is not covered.

- This model is applicable only for terrestrial radio paths.

d) Combined indoor-outdoor propagation models

Most of the propagation models published in the open literature are derived either for outdoor or indoor application. But in the "real world" a combination of both types is required.

In SEAMCAT, the classical outdoor models, Hata (modified Hata, see section b) and spherical diffraction model (ITU-R Recs. P.452, P.526 and P.676) are combined with an indoor model. An illustrative description is given in the following.

The path loss pl consists of median path loss L and the Gaussian variation T(G(σ)) where σ is the standard deviation:

pl=f(f,h1,h2,d,env)=L+T(G(σ))

where:

f	is the frequency in MHz,

h1	is the antenna height of the transmitter antenna in m,

h2 	is the antenna height of the receiver antenna in m,

d 	is the distance in km,

env	is a parameter for the environments of the transmitter and receiver.

For outdoor-outdoor holds:

- Scenario: transmitter and receiver are both outdoor;

- Modified Hata model:

Median:	L(outdoor−outdoor)=LHata(outdoor−outdoor)

Variation:	intrinsic variation, σ(outdoor−outdoor)=σHata

- Spherical diffraction model:

Median:	L(outdoor−outdoor)=Lspherical

Variation:	no variation possible, σ(outdoor−outdoor)=0

Case 1: Indoor-outdoor or outdoor-indoor

- Scenario: transmitter is indoor and receiver is outdoor, or vice versa;

- Modified Hata model:

Median:	L(indoor−outdoor)=LHata(outdoor−outdoor)+Lwe

where 			Lwe is the attenuation due to external walls (default value = 10 dB)

Variation:	σ(indoor−outdoor)=σHata2+σadd2

where σadd is the additional standard deviation of the signal (default value: 5 dB).

The standard deviation of the lognormal distribution is increased, compared to the outdoor-outdoor scenario due to additional uncertainty on materials and relative location in the building.

- Spherical diffraction model:
Median:	L(indoor−outdoor)=Lspherical+Lwe
Variation:	σ(indoor−outdoor)=σadd

The lognormal distribution is determined by the additional variation is due to the variation in building materials, only, because for the spherical diffraction model no variation is considered.

Case 2: Indoor-indoor

There are two different scenarios possible: The transmitter and receiver are in the same or in different buildings. Which scenario is used by SEAMCAT is randomly selected.

i)	Selection of the scenario

The first step is to determine whether the indoor-indoor scenario corresponds to transmitter and receiver in the same building or not. This is done by the calculation of the random variable SB (Same Building).

Trial of the condition SB (Same Building):

- d < 0.020 km (20 m):	SB = Yes 	=> P(Yes)=1

- 0.020 km < d < 0.050 km (50 m):

SB = Yes 	P(Yes) = (0.050-d)/0.030

SB = No	P(No) = 1- P(Yes) = (d-0.020)/0.030

- d > 0.050 km (50 m):	SB = Yes 	=> P(Yes)=0

ii)	Indoor-indoor, different buildings

- Scenario: transmitter and receiver in different buildings: P(Yes)=0 or P(No)=1

- Modified Hata model:

Median:	L(indoor−indoor)=LHata(outdoor−outdoor)+2Lwe

It is to remark that the loss due to 2 external walls is to add.

Variation:	σ(indoor−indoor)=σHata2+2σadd2

- Spherical diffraction model

Median:	L(indoor−indoor)=Lspherical+2Lwe
Variation:	 σ(indoor−indoor)=2σadd

The lognormal distribution is determined by the additional variation is due to the variation in building materials, only, because for the spherical diffraction model no variation is considered. The variation is increased to the second external wall.

iii)	Indoor-indoor, same building

•	Scenario: transmitter and receiver in the same building: P(Yes)=1 or P(No)=0

•	Indoor propagation model:

Median:

L(indoor−indoor)=−27.6+20log(1000d)+20log(f)+fix1000ddroomLwi+kfkf+2kf+1 − bLf

with				kf=fixh2−h1hfloor

Lwi	= 	loss of internal wall (in dB)	(default value = 5 dB)

Lf	= 	loss between adjacent floor (in dB)	(default value = 18.3 dB)

b	= 	empirical parameter 		(default value = 0.46)

droom	= 	size of the room (in m)		(default value = 4 m)

hfloor	= 	height of each floor (in m)		(default value = 3 m).

Note that the path length d uses the unit km and the frequency the unit MHz.

Variation:	 σ(indoor−indoor)=σin

The lognormal distribution trial is made using a standard deviation entered by the user and covering the variation, internal in the building, due to building design, in furniture of the rooms, etc. The default value is σin=10 dB.

e) VHF/UHF Propagation model (ITU-R Rec. P.1546)

The propagation curves derived for broadcasting are given in the ITU-R Rec. P.1546 is based on the former ITU-R Rec P.370: A set of received field strength E (in dB(μV/m)) normalised to a transmitting power of 1 kW e.r.p.. Using the conversion given in ITU-R Rec. P.525-2, this field strength level can be converted into the median basic radio path loss L (in dB) between two isotropic antennas by the following equation:

L(pl,pt)=139.4+20logf[MHz]−E(f,d,h1,env)

where

pl	= 50 % of the locations,

pt	= 50, 10, 5 or 1 % of the time,

f	= within the ranges 30-250 MHz and 450-1000 MHz,

d	= distances between 10 and 1000 km,

h1	= antenna height of the transmitter varying between 37.5 and 1200 m

env	= different types of environments: land (used in SEAMCAT), cold or warm sea.

Note that the path loss should be not less than the free space path loss.

The path loss, pl, including the variation of the locations can be denoted as the sum of the median path loss and a Gaussian distribution:

pl=L(pt,pl=50%)+T(G(σ))

ITU-R Rec. P.1546 proposes a propagation model for point-to-area prediction of field strength mainly for the broadcasting, but also for land mobile, maritime mobile and certain fixed services (e.g. those employing point-to-multipoint systems) in the frequency range 30 to 3 000 MHz and for the distance range 1 km to 1 000 km. For the use of analysing compatibility scenarios, the following simplification are assumed:

- Flat terrain.

- Restriction to propagation over land only, i.e. exclusion of mixed and sea paths.

- Positive antenna heights only.

Parameters of this propagation model are listed below:

i)	Path dependant parameters (constant during a simulation for a given path) are:

- Time percentage: pt (%)

- Transmitter system: analogue/digital

- Transmitter bandwidth: Bt

- Global environment: rural, suburban, urban.

ii)	Variable parameters (which vary for each event of a simulation):

- Transmitter antenna height: ht (m)

- Receiver antenna height: hr (m)

- Frequency f (MHz)

- Distance d (km).

For calculation of the path loss according to Rec. 370 the following procedure is to be followed:

- Check range of application of the propagation model regarding time percentage, frequency, distance, and antenna height:

- Time percentage: 1% < pt < 50 %, for pt  50 % pt is set to = 50 %

- Frequency: 30 MHz < f < 3 000 MHz

- Distance: 0.001 km  < d < 1 000 Km

- Transmitter antenna height: 0 m < ht < 3 000 m

- Receiver antenna height: 1 m < hr < 3 000 m.

- Determination of lower and higher nominal percentages ptinf and ptsup:

If pt < 10 then ptinf = 1 % and ptsup= 10 % else ptinf = 10 % and ptsup= 50 %

- Determination of the lower and higher nominal frequencies:

If f < 600 MHz then finf = 100 MHz and fsup = 600 MHz

else finf = 600 MHz and fsup = 2 000 MHz

- If ht≥10m: calculate field strength E=(f=f,d,ht,hr,pt) according to steps 4.1-4.4

  - Calculation of the four following fields strengths:

- E(f= finf , d, ht, hr, ptinf)

- E(f= fsup , d, ht, hr, ptinf)

- E(f= finf , d, ht, hr, ptsup)

- E(f= fsup , d, ht, hr, ptsup)

according to the procedure described in steps 4.1.1. to 4.1.4.

    - Calculate the dimensionless parameter k, function of the required transmitter height, ht, as follows:

    - Determine from following table the set of parameters a0 to a3, b0 to b7, c0 to c6 and d0 to d1 to be used according to nominal values of frequencies and time percentages.

| Frequency | 100 MHz |  |  | 600 MHz |  |  | 2 000 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pt % | 50 | 10 | 1 | 50 | 10 | 1 | 50 | 10 | 1 |
| a0 | 0.0814 | 0.0814 | 0.0776 | 0.0946 | 0.0913 | 0.0870 | 0.0946 | 0.0941 | 0.0918 |
| a1 | 0.761 | 0.761 | 0.726 | 0.8849 | 0.8539 | 0.8141 | 0.8849 | 0.8805 | 0.8584 |
| a2 | 30.444 | 30.444 | 29.028 | 35.399 | 34.160 | 32.567 | 35.399 | 35.222 | 34.337 |
| a3 | 90.226 | 90.226 | 90.226 | 92.778 | 92.778 | 92.778 | 94.493 | 94.493 | 94.493 |
| b0 | 33.6238 | 40.4554 | 45.577 | 51.6386 | 35.3453 | 36.8836 | 30.0051 | 25.0641 | 31.3878 |
| b1 | 10.8917 | 12.8206 | 14.6752 | 10.9877 | 15.7595 | 13.8843 | 15.4202 | 22.1011 | 15.6683 |
| b2 | 2.3311 | 2.2048 | 2.2333 | 2.2113 | 2.2252 | 2.3469 | 2.2978 | 2.3183 | 2.3941 |
| b3 | 0.4427 | 0.4761 | 0.5439 | 0.5384 | 0.5285 | 0.5246 | 0.4971 | 0.5636 | 0.5633 |
| b4 | 1.256E-7 | 7.788E-7 | 1.050E-6 | 4.323E-6 | 1.704E-7 | 5.169E-7 | 1.677E-7 | 3.126E-8 | 1.439E-7 |
| b5 | 1.775 | 1.68 | 1.65 | 1.52 | 1.76 | 1.69 | 1.762 | 1.86 | 1.77 |
| b6 | 49.39 | 41.78 | 38.02 | 49.52 | 49.06 | 46.5 | 55.21 | 54.39 | 49.18 |
| b7 | 103.01 | 94.3 | 91.77 | 97.28 | 98.93 | 101.59 | 101.89 | 101.39 | 100.39 |
| c0 | 5.4419 | 5.4877 | 4.7697 | 6.4701 | 5.8636 | 4.7453 | 6.9657 | 6.5809 | 6.0398 |
| c1 | 3.7364 | 2.4673 | 2.7487 | 2.9820 | 3.0122 | 2.9581 | 3.6532 | 3.547 | 2.5951 |
| c2 | 1.9457 | 1.7566 | 1.6797 | 1.7604 | 1.7335 | 1.9286 | 1.7658 | 1.7750 | 1.9153 |
| c3 | 1.845 | 1.9104 | 1.8793 | 1.7508 | 1.7452 | 1.7378 | 1.6268 | 1.7321 | 1.6542 |
| c4 | 415.91 | 510.08 | 343.24 | 198.33 | 216.91 | 247.68 | 114.39 | 219.54 | 186.67 |
| c5 | 0.1128 | 0.1622 | 0.2642 | 0.1432 | 0.1690 | 0.1842 | 0.1309 | 0.1704 | 0.1019 |
| c6 | 2.3538 | 2.1963 | 1.9549 | 2.2690 | 2.1985 | 2.0873 | 2.3286 | 2.1977 | 2.3954 |
| d0 | 10 | 5.5 | 3 | 5 | 5 | 8 | 8 | 8 | 8 |
| d1 | 1 | 1 | 2 | 1.2 | 1.2 | 0 | 0 | 0 | 0 |

    - Calculate the unblended to the maximum value field strength, Eu, at the distance, d, and transmitting height, ht, as follows:

where:

pb=d0+d1⋅k

and:

and:

where:

where:

ξ=log(d)b5

and:

    - Calculate the blended to the free space value of field strength, Eb, at the distance, d, and transmitting height, ht, as follows:

where:

Efs 	is the free-space field strength

Efs = 106.9  20 log (d) 	dB(V/m)

pbb 	is a blend coefficient set to value 8.

  - Calculation of the field strength E(f, d, ht, hr, ptinf) using log-linear interpolation in frequency range:

E = Einf + (Esup  Einf) log(f/finf)/log(fsup/finf)	dB(V/m)

where:

Einf,:	E(f= finf , d, ht, hr, ptinf)

Esup:	E(f= fsup , d, ht, hr, ptinf)

4.3)	Dual calculation for the field strength E(f, d, ht, hr, ptsup) using log-linear interpolation in frequency range:

E = Einf + (Esup  Einf) log(f/finf)/log(fsup/finf)	dB(V/m)

where:

Einf:	E(f= finf , d, ht, hr, ptsup)

Esup:	E(f= fsup , d, ht, hr, ptsup)

4.4)	Calculation of the field strength E(f, d, ht, hr, pt) using log-linear interpolation formula in time percentage range:

E = Esup (Qinf  Qt)/(Qinf  Qsup) + Einf (Qt  Qsup)/(Qinf  Qsup)     dB(μV/m)

where

(Qi (x) being the inverse complementary cumulative normal distribution function):

Qt	= Qi (pt/100)

Qinf	= Qi (ptinf/100)

Qsup	= Qi (ptsup /100)

Einf 	= E(f, d, ht, hr, ptinf)

Esup	= E(f, d, ht, hr, ptsup)

- For a transmitting/base antenna height ht less than 10 m determine the field strength for the required height and

distance using following method.

The procedure for extrapolating field strength at a required distance d km for values of ht in the range 0 m to 10 m is based on smooth-Earth horizon distances in km written as dH(h) = 4.1h, where h is the required value of transmitting/base antenna height ht in metres.

For d < dH(ht) the field strength is given by the 10 m height curve at its horizon distance, plus E, where E is the difference in field strengths on the 10 m height curve at distances d and the ht horizon distance.

For d  dH(ht) the field strength is given by the 10 m height curve at distance d beyond its horizon distance, where d is the difference between d and the ht horizon distance.

This may be expressed in the following formulae where E10 (d) is the field strength in dB(V/m) calculated for transmitter antenna 10 m and for a distance d (km) according to the procedure described in step 4:

E	=		E10(dH(10)) + E10(d)  E10(dH(ht))	dB(μV/m)	d < dH(ht)

=	E10(dH(10) + d  dH(ht))	dB(μV/m)	d  dH(ht)

If in the latter equation dH(10) + d  dH(ht) exceeds 1 000 km, even though d  1 000 km, E10 may be found from linear extrapolation for log(distance) of the curve, given by:

E10 	=	Einf + (Esup  Einf) log (d/Dinf)/log (Dsup/Dinf)	dB(V/m)

where:

Dinf:	penultimate tabulation distance (km)

Dsup:	final tabulation distance (km)

Einf:	field strength at penultimate tabulation distance (dB(μV/m))

Esup:	field strength at final tabulation distance (dB(μV/m)).

Note: This recommendation is not valid for distances greater than 1 000 km. This method should be used only for extrapolating for ht < 10 m.

- If the receiving/mobile antenna height hr is not equal to the height of representative clutter at its location (denoted R),

correct the field strength as follows:

The field-strength values given by the land curves and associated tabulations in this Recommendation are for a reference receiving/mobile antenna at a height, R (m), representative of the height of the ground cover surrounding the receiving/mobile antenna, subject to a minimum height value of 10 m. Examples of reference heights are 20 m for an urban area, 30 m for a dense urban area and 10 m for a suburban area.

If the receiving/mobile antenna height, hr (m), is different from R, a correction should be added to the field strength taken from the curve.

Where the receiving/mobile antenna is adjacent to land account should first be taken of the elevation angle of the arriving ray by calculating a modified representative clutter height R' (m), given by:

R'  =  R m                                                          for ht  6.5d + R 		            = (1 000 d R – 15 ht)/(1 000 d  15) m        for ht > 6.5d + R

where ht is in metres and distance d is in km.

The value of R' must be limited if necessary such that it is not less than 1 m.

When the receiving/mobile antenna is in an urban environment the correction is then given by:

Correction 	= (6.03 hr /R')  J()	dB	for hr < R_______________(1)

= K hr log (hr /R') 	dB	for hr  R_______________(2)

where J() is given by:

J(v)=6.9+20log(v−0.1)2+1+v−0.1

and where:

 	=	Knu  (hdif clut)

hdif	 =	R'  hr m

 clut 	=	arctan (hdif /15) degrees

K hr 	=	3.2 + 6.2 log (f)

Knu 	=	0.0108  f

F	 =	frequency (MHz).

Where the receiving/mobile antenna is adjacent to land in a rural environment the correction is given by equation (2) for all values of hr.

If the required distance is equal to or greater than d10, then again the correction for the required value of h2 should be calculated using equation (2) with R' set to 10 m.

If the required distance is less than d10, then the correction to be added to the field strength E should be calculated using:

Correction 	= 0.0 			dB	d  d (hr)

= (C10) log(d/dhr)/log(d10/dhr)	dB	dhr < d < d10

where:

C10:	correction for the required value of hr at distance d10 using equation (9.2) with R' set to 10 m

d10:	distance at which the path just has 0.6 Fresnel clearance for hr = 10 m calculated as D06(f, ht, 10) as given in Note 2

dhr:	distance at which the path just has 0.6 Fresnel clearance for the required value of hr calculated as D06(f, ht, hr) as given in Note 2.

This Recommendation is not valid for receiving/mobile antenna heights, hr, less than 1 m.

- Add a log-normal term G(L) corresponding to the variability in the percentage of locations:

Values of standard deviation for digital systems having a bandwidth less than 1 MHz and for analogue systems are given as a function of frequency by:

L = K + 1.6 log(f)                           dB

where:

K =	2.1 for mobile systems in urban locations

3.8 for mobile systems in suburban locations or amongst rolling hills

5.1 for analogue broadcasting systems.

For digital systems having a bandwidth of 1 MHz or greater, a standard deviation of 5.5 dB should be used at all frequencies.

- If necessary limit the resulting field strength to the maximum value calculated as follows.

The field strength must not exceed a maximum value Emax given by:

EMax = Efs     dB(V/m) for land paths

where Efs is the free space field strength for 1 kW e.r.p. given by:

Efs = 106.9  20 log (d)   dB(V/m)

- Convert field strength to path loss using following formula.

Lb = 77.2 – E + 20 log f                    dB

where:

Lb:	basic transmission loss (dB)

E:	field strength in dB(V/m) measured with a transmitting power of 1 W e.i.r.p.

f:	frequency (MHz).

Note 1: The following approximation to the inverse complementary cumulative normal distribution function, Qi(x),

is valid for 0.01  x  0.99:

Qi(x) = T(x) –(x) if x  0.5

Qi(x) = -{T(1-x) – (1-x)}	if x > 0.5

where:

T(x) = [−2 ln(x)]

ξ(x)=[(C2⋅T(x)+C1)⋅T(x)]+C0[(D3⋅T(x)+D2)⋅T(x)+D1]⋅T(x)+1

C0 = 2.515517

C1 = 0.802853

C2 = 0.010328

D1 = 1.432788

D2 = 0.189269

D3 = 0.001308

Note 2: The path length which just achieves a clearance of 0.6 of the first Fresnel zone over a smooth curved Earth, for a

given frequency and antenna heights ht and hr, is given approximately by:

D06 = Df DhDf + Dh km

where:

Df:	frequency-dependent term

= 	0.0000389 f h1 h2	km

Dh:	asymptotic term defined by horizon distances

= 	4.1(ht+hr)	km

f:	frequency (MHz)

ht, hr:	antenna heights above smooth Earth (m).

In the above equations, the value of ht must be limited, if necessary, such that it is not less than zero. Moreover, the resulting values of D06 must be limited, if necessary, such that it is not less than 0.001 km.

Note 3: The case ht is less than zero described in the recommendation is not handled.

Note 4: No correction due to terrain clearance angle is implemented.

#### Appendix 2 to Annex 2: Power control function

gitPC=fpc(pitsupplied,git→wr,plit↔wr,gwr→it,pcitthold,pcitdycrg,pcitstrg)

P=f(pitsupplied,git→wr,plit↔wr,gwr→it)=pitsupplied+git→wr−plit↔wr+gwr→it

P  =   power received by the wanted receiver, e.g. closed base station, of the interfering system

where pitsupplied,git→wr,gwr→it and plit↔wr are defined in the iRSS calculation sections.

pitthold is the lowest threshold (minimum) of the receiver

Case 1:		P≤pcitthold

pitsupplied_PC=pitsupplied

gitPC=0

Case (i+1):		pcitthold+(i−1)⋅pcitstrg≤P<pcitthold+i⋅pcitstrg

pitsupplied_PC=pitsupplied−(i−1)⋅pcitstrg

gitPC=−(i−1)⋅pcitstrg

where i is an integer ranging from 1 to n_steps = pcitdycrgpcitstrg

Case (n_steps+2):		P>pcitthold+pcitdycrg

pitsupplied_PC=pitsupplied−pcitdycrg

gitPC=−pcitdycrg

#### Appendix 3 to Annex 2: Distribution definitions

- Uniform distribution:	U(0,1)=1    if 0≤x≤10    otherwise

- Gaussian distribution:	G(σ)=12πσexp−x22σ2

- Rayleigh distribution:	R(σ)=rσ2exp−r22σ2

- User defined distribution: The option to include an User-defined distribution in the tool should be considered.

- Discrete distribution:

This is a special distribution bounded by a lower boundary Xmin , an upper boundary Xmax and the step Sbetween the samples xi . A common example of such a distribution is the discrete frequency distribution having a constant channel spacing.

The corresponding distribution for xi is then defined by the following equation:

xi=Xmin+S2+(i−1)S

where

i=1...N

N=(Xmax−Xmin)S

In the case of a uniform distribution, each value is assigned to the same probability

P(xi)=1N. In the case of non-uniform distribution , each value is assigned to a specific weight Pi with the constraint that the sum of these weights is equal one.

#### Appendix 4 to Annex 2: Pseudo-random number generaton (References (3, 4))

- From a uniform distribution U(0,1)

ui+1=T(U(0,1))=xi+1m

where:

xi+1=(a⋅xi)(modm)

a 	= 	multiplier e.g. a = 16,807 or 396,204,094 or 950,706,376

m 	= 	modulus e.g. m=231 – 1 = 2,147,483,647

x0 	= 	seed, integer variable taking a value between 1 and (m-1)

- From a Gaussian distribution G(σ)

T(G(σ))=ν1−2ln(s)s

where:

while s  1, do ν1=2⋅Tseed1(U(0,1))−1ν2=2⋅Tseed2(U(0,1))−1s=ν12+ν22

v1 and v2 are two independent random variables (using two different seeds) uniformly distributed between -1 and +1.

•	From a Rayleigh distribution R(σ)

T(R(σ))=(ν12+ν22)⋅−2ln(s)s

where:

while s  1, do ν1=2⋅Tseed1(U(0,1))−1ν2=2⋅Tseed2(U(0,1))−1s=ν12+ν22

v1 and v2 are two independent random variables (using two different seeds) uniformly distributed between -1 and +1.

- From any type of distribution with a given cumulative distribution function, cdf

Some trials may be performed according to a user-defined distribution F.

Trial is based on the use of the reciprocal cumulative distribution function cdf−1 relative to the user-defined distribution F applied to the result of a uniform sample between 0 and 1.

T(F)=cdf−1(p)	where	p=T(U(0,1)) (uniform trial between 0 and 1)

Figure 1: Direct cumulative distribution function

Figure 2: Inverse cumulative distribution function

#### Appendix 5 to Annex 2: dRSS calculation flow chart

Calculation of

Trial of relevant victim parameters:

Calculation of

Calculation of

Calculation of

Variable distance

yes

no

dRSS

fixed distance

given distribution: yes

fixed value?

Start

given wanted signal

Go to ICE

no

#### Appendix 6 to Annex 2: irss due to unwanted and blocking calculation

Calculation of intermodulation products

(see Sub-Annex B.h)

Calculation of :

Calculation of :

no

Power

Control?

Trial of :

Calculation of :

Calculation of :

yes

Trial of relevant interferer parameters :

Calculation of

Interferer i = 1 : n

Calculation of

#### Appendix 7 to Annex 2: Receiver Blocking

- Basic concept

The receiver is capturing some unwanted signal because its filter is not ideal.

Figure 1: Basic concept

Definition: Blocking is a measure of the capability of the receiver to receive a modulated wanted input signal in the presence of an unwanted input signal on frequencies other than those of the spurious responses or the adjacent channels, without these unwanted input signals causing a degradation of the performance of the receiver beyond a specified limit(Document I-ETS 300 113:1992).

2	Blocking level measurements

- Adjust the desired signal at the BER limit level.

- Increase this desired signal by 3 dB and add the interfering signal which is increased until the same BER is obtained.

- The ratio (interfering signal/desired signal) is the value of the Receiver blocking.

Attenuater

Desired signal

Interfering signal

Rx

BER

Figure 2: Measurement procedure

3	Attenuation of the receiver

During the measurement procedure, the three following equations are valid:

- Noise Floor + Protection ratio + 3 dB 	= Desired Signal Level,

- Desired Signal Level + Blocking 	= Interfering Signal Level,

- Interfering Signal Level – Attenuation	= Noise Floor.

Hence,

Attenuation = 3 dB + Protection ratio + Blocking

Figure 3

Figure 4: Receiver mask

#### Appendix 8 to Annex 2: iRSS due to intermodulation

This flow chart is part of the flow chart given in Appendix 6.

Frequencies 
,

Interferer i = 1 : n

Interferer j = 1 : n

no

Stored values for relevant interferer parameters

and calculated ones

with k=i,j (see also Sub-annex B.f)

Calculation of

with k=i,j

Calculation of

yes

#### Appendix 9 to Annex 2: Intermodulation in the Receiver

The main contribution to Intermodulation Interference originates from interfering signals in neighbouring channels due to the frequency selectivity of the antennas and the receiver equipment. We consider a service with a desired signal at frequency f0, a channel separation f and interfering signals Ei1 and Ei2 at frequencies f0+nΔf and f0+2nΔf, respectively. The receiver non-linearities produce an intermodulation product Eif of third order at the frequency (see Figure 1).

f0=2(f0+nΔf)−(f0+2nΔf)           n=±1,±2,...	(1)

Figure 1

The signal strength Eif of the intermodulation product is given by

Eif=kEi12Ei2	(2)

with some constant k to be determined. For signal levels (measured in dB) the Eq. (2) reads

Lif=2Li1+Li2+20logk.	(3)

The constant 20log k in Eq. (3) can be found from the measurement procedure which is described in the ETSI standard ETS 300-113, clause 8.8. The method is similar to the contribution in Appendix 7 for blocking interference.

ETS 300-113 defines via the intermodulation response Limr the interfering signal levels Li1=Li2 at which bit errors due to intermodulation just start to be recorded (see Figure 2).

Figure 2

This means, for Li1 and Li2 as in Figure 2, we have an intermodulation product Lif just at the noise floor (0 dB). Introducing Li1 and Li2 from Figure 2 into Eq. (3) we obtain

0=2(Limr+3dB+Lsens)+(Limr+3dB+Lsens)+20logk	(4)

with the value of k from Eq. (4), the Eq. (3) becomes

Lif=2Li1+Li2−3Limr−3Lsens−9dB.	(5)

#### Appendix 10 to Annex 2: Influence of different bandwidths

a)	Wanted Path

The wanted transmitter transmits its power pwt (dBm) at the frequency fvr within a given bandwidth bvr . This bandwidth is also used for the determination of the intermodulation products (see Appendix 8).

b)	Interfering Transmitter

For the interfering transmitter, an emission mask emissionit as function of Δf=f−fit can be  defined as maximum power levels emissionit(Δf) in reference bandwidth bs(Δf) as specified by the user. This mask can also be expressed as the maximum of

- the sum of the supplied interfering power pitsupplied, a relative emission mask (containing the wanted transmission and all unwanted emissions including the emission floor depending on the power control) and the gain power control

- or the absolute emission floor.

The relative emission mask is described by a triplet (frequency offset (MHz), relative emission level (dBc) and reference bandwidth (MHz)). The emission floor is defined in section e) of this Appendix.

The interfering transmitter power pitsupplied (dBm) at fit is used for evaluating the link budget with the wanted receiver (i.e. power control).

c)	Determination of interfering power, principle

Figure 1: Principle of determination of interfering power

The Figure shows the principle of the determination of the interfering power. If fit=fvr then the interfering frequencies fall exactly in the receiving band of the victim receiver (co-channel interference).

For simplification within the algorithms the mask function pmit is normalised to 1 Hz reference bandwidth:

pnit=pmit(Δf)−10logb1Hz

- The bandwidth b is the bandwidth used for the emission mask. The total received interfering power emissionit can easily calculated by integration over the receiver bandwidth from a=fvr−fit−bvr2 to b=fvr−fit+bvr2

powerit=10logab10(pnit(Δf)10) dΔf

- with pnit denoting the normalised mask in dBm/Hz. Using 1 Hz reference bandwidth the integral can be replaced by a summation where powerit is given in dBm:

powerit=10logi=ab10(pnit(Δfi)10)

NOTE - The interfering power of a radio system having a different bandwidth can be estimated by the aforementioned algorithms. This calculation is only required for the interference due to unwanted emissions or co-channel but not for blocking and intermodulation.

Note that it is recommended to apply a user defined mask even the mask is flat.

d)	Implementation in SEAMCAT

In section c) the principle is explained. However, this algorithms is very slow in terms of computation time. Therefore the following approach is implemented:

The total interfering power relative to carrier emissionrelit can be calculated by integration over the receiver bandwidth from a=fvr−fit−bvr2 to b=fvr−fit+bvr2

emissionrelit=10logPrellinearΔfdΔf=10log10PreldBcΔf10dΔf

with PreldBc denoting the normalised user-defined mask in dBc/Hz.

This mask is expressed as an array of N+1 points (Δfi,Pi)and assumed linear between these points.

PreldBcΔf=Pi+Δf−ΔfiΔfi+1−ΔfiPi+1−Pi

This leads to:

emissionrelit=10logi=0N−110PreldBcΔf10dΔf

where:

Df0=a=fvr−fit−Bvr2

DfN=b=fvr−fit+Bvr2

Intermediate calculation

emissionrelidBc=10PreldBcΔf10dΔfemissionrelidBc=10Pi1010Pi+1−Pi10Δfi+1−ΔfiΔf−ΔfidΔfemissionrelidBc=10Pi10KΔfiKΔf−ΔfidΔf, K=10Pi+1−Pi10Δfi+1−Δfi

emissionrelidBc=10Pi10KΔfielnKΔfiΔfi+1=10Pi10lnKKΔfi+1−Δfi−1, lnK=ln1010.Pi+1−PiΔfi+1−Δfi

emissionrelidBc=10ln1010Pi+1−10PiPi+1−PiΔfi+1−Δfi

Eventually:

emissionrelit=10log10ln10i=0N−1Pi+1linear−PilinearΔfi+1−ΔfiPi+1dBc−PidBc

e)	Emission floor

The aforementioned formulas are also applicable to absolute emission floor emissionfloorit (dBm). This emission floor mask can be described by a triplet (frequency offset (MHz), reference bandwidth (MHz), emission floor (dBm)).

The real emission is bounded by the emission floor by the following equation

emissionit=max(emissionrelit+pitsupplied+gitPC,emissionfloorit)

which is also illustrated in Figure 3.

Figure 3: Resulting power level for the calculation of iRSS

Note that the comparison involves the power control gain if power control is selected.

Note that the unwanted emission floor is referred to 1 MHz in SEAMCAT up to version 1.1.5.

#### Appendix 11 to Annex 2: Radio cell size in a noise limited network

Assuming that the received power is equal to the sensitivity of the victim receiver, then the radius Rmax can be determined for the wanted radio path by the following equation according to ERC Report 68

fmedian(fvr,hvr,hwt,Rmax,env)+fslowfading(X%)=Pwt+gwt+gvr−sensvr

where the path loss is defined by a median loss plus an additional term representing the distribution

ploss=fmedian+fslowfading(X%)

The distribution of the of the path loss ploss can be expressed in a general way by the following equation

Q(μ+a,Rmax)=y

where Q is the cumulative distribution for Rmaxand the resulting mean path loss μ and an additional path loss a due to availability or coverage y . The coverage loss x corresponds to y by 1−y. Assuming that slow fading can be approximated by log-normal distribution, i.e. median  mean, the relation a=bσ can be introduced where b stands for a multiple of the well known standard deviation σ. A few examples for illustration: At a 95 % coverage, b results in 1.96, for 99 % in 2.58, for 99.9 % in 3.29, or b=1 68 % coverage, for b=2 for 95.5 %. The exact values can be easily determined by using the inverse Gaussian function.

Then the transcental equation

g(Rmax)=Pwt+gwt+gvr−sensvr−fmedian(fvr,hvr,hwt,Rmax,env)−bσ

can be solved by using a linear iteration like regula falsi.

Rmax=Rmax0−Rmax0−Rmax1g(Rmax0)−g(Rmax1)g(Rmax0)

Note that faster convergence can be obtained by applying the distance in logarithmic scale, i.e. the variableR has to be replaced by log(R).

Note that in this case, formulas given for fmedianRmaxwt+... have to be inverted.

Note that in SEAMCAT the following simplifications are introduced:

- All antenna gains are set to 0 dBi and

- up to version 1.1.5 the modified Hata model is applied as propagation model.

#### Appendix 12 to Annex 2: Symmetric antenna pattern

There are three different ways to describe the antenna pattern used by SEAMCAT:

- omnidirectional antenna

- directional antenna pattern (in dBi); the gain is referred to the main lobe and depending on the angle in azimuth and elevation

- symmetric antenna pattern (not up to version 1.1.5 of SEAMCAT)

Such pattern are often used by Fixed and Space Services. According to ITU-R Rec. IS.847 or F.699, the antenna gain g (in dBi) can by expressed by the following equation:

g=g0−10log(Dλ )−25logψ		for (100λ D)°≤ψ≤ψ0

where:

g0	maximum of gain of the main lobe (in dBi), e.g. 52 dBi

D	diameter of the antenna dish (in m)

λ=300f[MHz]	wavelength (in m)

ψ	spherical angle (in degree) between the direction considered and the main beam, defined by ψ=0

ψ0	boundary for the main lobe (in degree), e.g. 48º.

SEAMCAT does not allow to use the antenna pattern for spherical angles between the direction considered and the axis of the main beam (An elevation antenna pattern and an azimuth antenna pattern must be defined). The spherical angle ψ, symmetric around its axis, is a combination of the azimuth and elevation angles according to the following equation:

cosψ=cos(θ) cos(ϕ )

where:

θ	Azimuth angle (in degree)

ϕ	Elevation angle (in degree)

The gain outside the main lobe has to be defined as a fixed value covering the whole range of angles.

References for Annex 2

(1)	Annex B of document SE21(94)/68. Subject: An Objective Derivation of Isolation Distance. Source: Motorola.

(2)	France, ITU Radio communication Study Group, Document 1-3/31(Rev.1)-E.
Subject: Proposal for a Propagation Model to be used in Models for Calculating Spurious Emission Interference, May 1995

(3)	Knuth, D. E., The Art of Computer Programming, Vol. 2, Seminumerical Algorithms, 
Addison-Wesley, Reading, Massachusetts, 1969.

(4)	Reuven Y. Rubinstein, Simulation and the Monte-Carlo Method, Haifa, Israel, 1981.

### ANNEX 3

### Distribution Evaluation Engine

Flow chart for DEE is shown in Figure 1. Fit-of-goodness test can be performed either by the chi-squared test or by the Kolmogorov-Smirnov algorithm (used in SEAMCAT).

The algorithm basically tests if a random sample of observations conform to a pre-specified cumulative distribution. The pre-defined distribution can be continuous, discrete or hybrid. Thus, chi-squared method is very versatile and single algorithm is proposed for use within DEE for testing all possible types of probability distribution functions.

An array of samples on RSS random variable is passed to DEE. Firstly DEE test if the array length, N (number of samples), is long enough to produce a stable distribution. This is accomplished by using N-dN samples to establish an initial discrete distribution function and calculate the corresponding cumulative distribution function (cdf). This cdf is then used as a reference in the chi-squared test performed now on the complete population of N samples. Should the test show that two discrete distribution differs more than an acceptable and prespecified value, a message is send back to EGE to generate some extra samples. On contrary, if the chi-squared criteria is satisfied DEE proceed with testing whether or not a continuous probability density function can be used.

The flow-chart in Figure 1, as an example, shows Gaussian distribution test. The chi-squared algorithm is equally applicable to any other continuous distribution that might be representative of RSS random variable. A continuous distribution function enables a closed form expression for probability calculation in ICE what in return warrants a numerically efficient calculation. If no continuous pdf fits the sample population with the adequate accuracy, discrete pdf representation and numerical probability calculation is the only way forward.

The flow chart in Figure- 2 presents one of many different possibilities to form the discrete pdf for a random variable.

Notation used:

RSS>	= 	random variable population

N		= 	sample population size

I		= 	counter internal to result stability testing

dN		= 	portion of population size (e.g. dB=0.1N)

Y		= 	chi-squared test criteria (see Appendix 1 to Annex 3)

χ1−α		= 	quantile - reference level for chi-squared test

n		= 	total sample counter

C>		= 	discrete cdf coefficient array .

Figure 1: DEE flow chart

no

If for one of the couple iiRSS/ijRSS :

yes

no

dRSS and iRSS not vectors

Part 2 for dRSS and iRSS

dRSS not a vector

all iRSS are vectors

Part 2 for dRSS only

If for one of the couple dRSS/iRSS:

yes

Go to ICE : dRSS and all iRSS are vectors

Calculation of the correlation factor for each couple of iRSS and each couple of iRSS/dRSS :

I=2 ?

array vectors :

N - array vectors dRSS/iRSS

- array under test

Take N-dN samples

Sorting

Take N  samples

I=I+1

yes

I=1

chi-square test

Go to EGE

Do additional trials

yes

Part 2 of DEE :

dRSS and/or iRSS

Calculation of distribution parameters:

E(X) and E((X-E(X))²)

chi-square test

Go to ICE with Gaussian distribution

Go to ICE with discrete distribution

Yes

Figure 2

#### Appendix  1 to Annex 3: Chi-Squared Goodness-of-Fit Test

The chi-squared goodness-of-fit test is one the oldest and best known statistical tests.

Lets assume X1, X2, . . . XN be a sample set drawn from a population with unknown cumulative distribution function (cdf) Fx(x). Chi-squared test is based on testing the null hypothesis

H0 : Fx(x) = F0 (x) for all x	against the alternative	H1 : Fx(x)  F0 (x) for some x

Assume that N observations are grouped into K mutually exclusive categories. Lets denote by Nj the observed number of trials in j th category ( j = 1,2, ..., K). In addition, denote Nj0 the number of trials expected to fall into j-th category according to the known cdf F0(x).

The actual test employs the following criteria:

Y=j=1K(Nj−Nj0)2Nj0,j=1KNj=N

which tends to be small when H0 is true and large when H0 is false. The Y is also random variable which obeys chi-square distribution for large N.

In practice, for the hypothesis H0 to prevail we expect

P(Y>χ1−α2)=α

where  is the significant level, say 0.05 or 0.1; the quantile χ1−α2 corresponds to probability of 1- is given in the tables for chi-squared distribution.( see Table 1).

The chi-squared goodness-of-fit test is equally applicable to discrete and continuous probability density functions.

| 1- K | 0.975 | 0.95 | 0.90 | 0.75 |
| --- | --- | --- | --- | --- |
| 10 | 3.25 | 3.94 | 4.86 | 6.74 |
| 20 | 9.59 | 10.85 | 12.44 | 15.45 |
| 30 | 16.79 | 18.49 | 20.60 | 24.48 |
| 40 | 24.43 | 68.51 | 29.05 | 33.66 |
| 50 | 32.36 | 34.76 | 37.69 | 42.94 |
| 60 | 40.48 | 43.19 | 46.46 | 52.29 |
| 70 | 48.76 | 51.74 | 55.33 | 61.70 |
| 80 | 57.15 | 60.39 | 64.28 | 71.14 |
| 90 | 65.65 | 69.13 | 73.29 | 80.62 |
| 100 | 74.22 | 77.93 | 82.36 | 90.13 |

Table 1: Quantile χ1−α2 for chi-squared distribution

#### Appendix 2 to Annex 3: Kolmogorov-Smirnov test of stability

Purpose of this evaluation stage is to estimate whether the number of generated events is enough to consider the results as stable from a statistical point of view. The stability evaluation is performed by a fit-of-goodness test with the Kolmogorov-Smirnov (K-S) test in order to check if the distribution obtained with N−dN samples and the one obtained with N samples do not differ more than a specified value:

First of all two cumulative distribution function have to be derived from the input array vector:

- distribution derived from the first N−dN samples of the array vector.

- distribution derived from the complete array vector (N samples).

This is done by means of a simple array sort. The test then simply consists in performing the chi-squared test with following input:

- specified stability threshold (between 0 and 1)

- reference distribution: distribution derived from the N-array

- tested distribution: distribution derived from the N−dN array.

According to the result of the K-S test, if the result is greater than the stability threshold, stability evaluation is considered successful.

### ANNEX 4

### Interference calculation engine

The ICE has two different functions:

- Process different interfering signals in order to calculate the probability for interference. Three type of interfering signals are considered: spurious emission, out-of-band emission, blocking and intermodulation.

- Derive generic limits. The output of the ICE is then a multi-dimensional surface giving the probability of interference versus the radio parameters. The general ICE flow chart is shown in Figure 1.

The interfering signal distributions are calculated with respect to reference levels or functions of unwanted (emission mask), blocking (receiver mask) or intermodulation attenuation. The translation law for the cdf from reference refi−init to reference refi is given by the following formula:

P(iRSSi(refi)<X)=P(iRSSi(refi−init)<X−t(refi−refi−init))     ;   t=   1       ;     i=spur−1       ;     i=block−3      ;      i=intermod	      (1)

The complete and quick (approximate) flow charts for the ICE are shown in Figs. 2 and 3 respectively. For sake of simplicity, the case of t=1 (Eq. (1), spurious case) appears in flow charts 2 and 3.

Quick calculation algorithm

In the ICE quick calculation algorithm we make the following two assumptions:

1)	The iiRSS are independent variables, where the index i corresponds to the i-th type of interfering scenario.

2)	One of the iiRSS is dominant with respect to all the other interfering signals.

The overall probability PD for not being interfered by the composite interfering signal reads

PD=PdRSSiRSScomposite>CI| dRSS>sensvr	(2)

Using the second assumption, we can approximate Eq. (2) by the following equation :

PD=Pintersectni=1dRSSiiRSS>CI| dRSS>sens  	(3)

and since the iiRSS are independent variables, we can write (3) as

PD≈i=1nPdRSSiiRSS>CI|dRSS>sens≡i=1nPi(CI)	(4)

For each interfering scenario corresponds a set of references, refi, e.g. spur, avr, etc. The user can choose the set of references that will be used in the calculation of PD. We incorporate refi in (4) and get the following approximation :

PD≈i=1nPi(CI, refi)	(5)

which is used in the quick calculation algorithm. It can be easily shown that 1-PD gives the probability of being disturbed by at least one of the n interferers.

Complete ICE flow chart

Three cases are considered:

1)	The desired and/or the interfering signals are correlated. In this case the probability PD is calculated by processing directly the data vectors. For each interfering scenario, the interfering signals of all interferers are summed up to get iRSScomposite. Then, from the two vectors iRSS and iRSScomposite we calculate the probability PD:

PD=PdRSSiRSScomposite>CI| dRSS>sens	(6)

by summing up all the terms satisfying dRSS>sens. Similarly to the quick calculation case, when we sum up elements form the data vectors to calculate (5), we should update the data so that it corresponds to a desired set of references.

- All signals are uncorrelated and their distributions (calculated by the DEE) are given in closed form. First, the cumulative distribution function of the composite interfering signal is calculated by integrating the iiRSS distribution functions. Note that the refi cause linear shifts of the iiRSS distributions with respect to one another. In the calculation of iiRSScomposite composite the iiRSS distributions should be shifted so that they all refer to the same set of references. Finally, (5) is calculated by using the conditional probability formula which integrates the distributions dRSS and iRSScomposite.

3)	The third case is similar to the second one, with the exception that the iRSScomposite distribution function is determined by the MC technique.

Probability

calculation

(Figure 2)

Derive generic   limits ?

End of ICE

no

Shift the reference

distributions

(spur, block, intermod)

(note-1)

yes

Note-1: This loop is repeated for each

value of spur, block, and intermod

in order to get an N-dimensional curve

Figure 1: General ICE flow chart

The flow chart in Figure 1 is describing the logical process of ICE, which is well suited in the case of the full integration for the calculation of iRSScomposite (see flow chart 2). However, in the case of input vector data or Monte-Carlo sampling process, the calculation of the summation of vectors for determining iRSScomposite and the trials of iiRSS, respectively, which are time and resource consuming, can be made only once as shown in Figure 2.

no

dRSS

iiRSS(refinit)

i2RSS(refinit)

...

inRSS(refinit)

Quick Calculation ?

yes

Go to

Quick Calculation Flow Chart

integration or sampling ?

(note 1)

j=1 to number of samples

yes

iRSS vectors ?

(note 4)

dRSS vector ?

no

Sorting of  iRSScomposite(ref1,..,refn)

(note 3)

yes

sorting of iRSScomposite(ref1,..,refn)

(note 3,4)

Trial of

i1RSS

i2RSS

...

inRSS

ref1=ref1-min ... ref1-max

ref2=ref2-min ... ref2-max .......

refn=refn-min ... refn-max

Calculation and sorting of:    (iRSScomposite+Nth)/iRSScomposite or   dRSS/iRSScomposite

or dRSS/(Nth+IRSScomposite)

Calculation of :

P((iRSScomposite+Nth)/Nth>

(iRSScomposite+Nth)/Nth)

P(dRSS/iRSScomposite>C/I)

P(dRSS/(Nth+iRSScomposite)>C/N+I)

Calculation of :

(note 2)

Probability distributions

of iRSScomposite(ref1 , .., refn)

no

Figure 2: ICD flow chart

NOTE 1 - Computing time is the criteria to choose between sampling or integrating.

no

NOTE 2 - This formula is detailed in the document SE21(96)/20 add 1. (dRSS/I) is the criteria used in this example. Other criteria may be used.

NOTE 3 - ref1,..., refn are the values of the relevant parameters (spur, aav, ...) for which the calculation of the probability of interference is needed.

NOTE 4 -The meaning of this sum is symbolic since the addition is to be made on the linear values and that iiRSS are expressed in dB.

ref1=ref1-min...ref1-max

refn=refn-min...refn-max

Quick Calculation Flow Chart

ref2=ref2-min...ref2-max

Figure 3: ICE quick calculation flow chart

The used documentation is based on documents published in 1990-1994. In the meantime newer Recommendations are available. Unfortunately some of the useful information were shifted to Reports or other Recommendations.